3GPP TSG-RAN WG4 Meeting # 114bis	R4-2504973

# Wuhan, Hubei, CN, 7 April - 11 April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft bigCR on Rx BSF optimization for NR RRM ph5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-04-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Introduce the reduced UE Rx beam sweeping factor to L3 RRM measurements, which helps reduce the measurement period. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To define L3 RRM measurements based on reduced UE Rx beam sweeping factor for the following scenarios: Scenario 1: SSB based Intra-frequency measurement without MG. Scenario 2: SSB based Intra-frequency measurement with MG. Scenario 3: SSB based Inter-frequency measurement without MG. Scenario 4: SSB based Inter-frequency measurement with MG. Scenario 5: Handover. Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection. This draft bigCR includes the following endorsed draft CRs: Draft CRs endorsed in RAN4#114bis: T-doc; Company; Title. R4-2503357; OPPO; DraftCR on Multi-Rx based RRC Connection Release with Redirection requirements. R4-2504957; CATT; DraftCR on SSB-based intra-frequency measurement without gap delay reduction by optimizing Rx BSF. R4-2504958; CMCC; DraftCR for optimizing Rx beam sweeping factor on SSB based Inter-frequency measurement with MG. R4-2504959; Nokia; Draft CR HO requirements and FBS requirement applicability. R4-2504960; ZTE Corporation, Sanechips; Draft CR on FBS based RRC Re-establishment for R19 RRM enh. R4-2504970; Apple; Draft CR on SSB based Intra-frequency measurement with MG. R4-2504971; MediaTek Germany GmbH; draft CR for Rx beam sweeping optimization for SSB based Intra-frequency measurement with NCSG. |
| Consequences if not approved: |  | L3 RRM measurement requirements based on reduced UE Rx beam sweeping factor are not defined in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.6.x (new), 6.1.1.4.2, 6.2.1.2.1, 6.2.3.2.1, 9.2.5.1, 9.2.6.2, 9.2.7.1, 9.3.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | NA |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 3.6.x	Applicability of requirements for UE supporting L3 fast beam sweeping operation in FR2-1

The requirements related to the support of [Rel-19 reduced L3 beam sweeping capability] are applicable when

-	the carrier to be measured is the only carrier in the FR2-1 band configured for L3 SSB measurement, and

-	UE is not configured with CA or DC.

<End of Change 1>

<Start of Change 2>

##### 6.1.1.4.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

Where:

Tsearch is the time required to search the target cell when the handover command is received by the UE. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	If the target cell is a known intra-frequency cell, then Tsearch = 0 ms.

-	If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms.

-	If the target cell is a known inter-frequency cell, then

-	For a UE supporting ncd-SSB-BWP-Wor-r18:

-	if the measured SSB is the target SSB for handover of the target cell, Tsearch = 0 ms;

-	if the measured SSB of the target cell and the target SSB for handover belong to the same NR target cell, Tsearch = Trs ms provided one of the following conditions is satisfied:

-	The measured SSB is the CD-SSB in the target SSB of the HO target SSB is the NCD-SSB in the first active DL BWP, or

-	The measured SSB is the NCD-SSB in the target celland the HO target SSB is the CD-SSB in the first active DL BWP, or

-	The measured SSB is the NCD-SSB in the target cell and the HO target SSB is the NCD-SSB and both are within different DL BWPs

-	If a UE not supporting ncd-SSB-BWP-Wor-r18, the target cell is a known inter-frequency cell, then Tsearch = 0 ms.

-	If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, Tsearch = N*3* Trs ms.

Where

N = [reduced beam sweeping factor] when the UE supports [Rel-19 reduced L3 beam sweeping capability] and the conditions in section [3.6.x] are fulfilled, otherwise N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2.

Tprocessing is time for UE processing. Tprocessing can be up to 20 ms.

Tmargin is time for SSB post-processing. Tmargin can be up to 2 ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ =  Trs for both known and unknown target cell.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If such measObjectNRs configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

In FR2, the target cell is known if it has been meeting the following conditions:

-	During the last 5 seconds before the reception of the handover command:

-	the UE has sent a valid measurement report for the target cell and

-	One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

-	One of the SSBs measured from the target cell also remains detectable during the handover delay according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell.

otherwise, it is unknown.

<End of Change 2>

<Start of Change 3>

##### 6.2.1.2.1	UE Re-establishment delay requirement

The UE re-establishment delay (TUE_re-establish_delay) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 38.331 [2] is detected by the UE and when the UE sends PRACH to the target PCell. The UE re-establishment delay (TUE_re-establish_delay) requirement shall be less than:

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

The intra-frequency target NR cell shall be considered detectable if each relevant SSB can satisfy that:

-	SS-RSRP related side conditions given in clauses 10.1.2 and 10.1.3 are fulfilled for a corresponding NR Band for FR1 and FR2, respectively, and

-	the conditions of SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding NR Band are fulfilled.

The inter-frequency target NR cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in clauses 10.1.4 and 10.1.5 are fulfilled for a corresponding NR Band for FR1 and FR2, respectively, and

-	the conditions of SSB_RP and SSB Ês/Iot according to Annex B.2.3 for a corresponding NR Band are fulfilled.

Tidentify_intra_NR: It is the time to identify the target intra-frequency NR cell and it depends on whether the target NR cell is known cell or unknown cell and on the FR of the target NR cell. If the UE is not configured with intra-frequency NR carrier for RRC re-establishment then Tidentify_intra_NR=0; otherwise Tidentify_intra_NR shall not exceed the values defined in table 6.2.1.2.1-1 when highSpeedMeasFlagFR2-r17 is not configured or UE is not capable of FR2 power class 6 and table 6.2.1.2.1-3 when highSpeedMeasFlagFR2-r17 is configured and UE is capable of FR2 power class 6.

Tidentify_inter_NR,i: It is the time to identify the target inter-frequency NR cell on inter-frequency carrier i configured for RRC re-establishment and it depends on whether the target NR cell is known cell or unknown cell and on the FR of the target NR cell. Tidentify_inter_NR,i shall not exceed the values defined in table 6.2.1.2.1-2.

TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

TSI-NR: It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for the target NR cell.

TPRACH: It is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell. TPRACH can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in table 8.1-1 of TS 38.213 [3].

Nfreq: It is the total number of NR frequencies to be monitored for RRC re-establishment; Nfreq = 1 if the target intra-frequency NR cell is known, else Nfreq = 2 and Tidentify_intra_NR = 0 if the target inter-frequency NR cell is known.

There is no requirement if the target cell does not contain the UE context.

In the requirement defined in the below tables, the target FR1 cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown.

Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2-1 | N/A | MAX (1000 ms, (M1Note2 x TSMTC + Tproc)) |
| ≥ -8 | FR2-2 | N/A | MAX (1000 ms, 120 x TSMTC)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2-1 | N/A | M2Note1,Note3 + Tproc |
| < -8 | FR2-2 | N/A | 5280Note1 |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: M1 = 80 and Tproc = 0 ms for the UE not capable of [FBS-R19], otherwise, M1 = N1*10, N1 = [reduced N] and Tproc = 2 ms. NOTE 3: M2 = 3520 and Tproc = 0 ms for the UE not capable of [FBS-R19], otherwise, M2 = N1*440, N1 = [reduced N] and Tproc = 2 ms. |  |  |  |

Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | MAX (800 ms, 13 x TSMTC, i) |
| ≥ -8 | FR2-1 | N/A | MAX (1000 ms, (M1Note2 x TSMTC, i + Tproc)) |
| ≥ -8 | FR2-2 | N/A | MAX (1000 ms, 156 x TSMTC, i)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2-1 | N/A | M2Note1,Note3 + Tproc |
| < -8 | FR2-2 | N/A | 6000 Note1 |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: M1 = 104 and Tproc = 0 ms for the UE not capable of [FBS-R19], otherwise, M1 = N1*13, N1 = [reduced N] and Tproc = 2 ms. NOTE 3: M2 = 4000 and Tproc = 0 ms for the UE not capable of [FBS-R19], otherwise, M2 = N1*500, N1 = [reduced N] and Tproc = 2 ms. |  |  |  |

Table 6.2.1.2.1-3: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell when highSpeedMeasFlagFR2-r17 is configured (Frequency range FR2)

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 10 xN2 x TSMTC)) |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: When SMTC <= 40 ms, N2=2 when highSpeedMeasFlagFR2-r17 = set1; N2=6 when highSpeedMeasFlagFR2-r17 = set2. |  |  |  |

<End of Change 3>

<Start of Change 4>

6.2.3.2.1	RRC connection release with redirection to NR

The UE shall be capable of performing the RRC connection release with redirection to the target NR cell within Tconnection_release_redirect_NR.

The time delay (Tconnection_release_redirect_NR) is the time between the end of the last slot containing the RRC command, “RRCRelease” (TS 38.331 [2]) on the NR PDSCH and the time the UE starts to send random access to the target NR cell. The time delay (Tconnection_release_redirect_NR) shall be less than:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH

The target NR cell shall be considered detectable when for each relevant SSB, the following side conditions are met:

-	the conditions of SSB_RP and SSB Ês/Iot according to Annex B.2.5 for a corresponding NR Band are fulfilled.

TRRC_procedure_delay: It is the RRC procedure delay for processing the received message “RRCRelease” as defined in clause 6.2.2 of TS 38.331 [2].

Tidentify-NR: It is the time to identify the target NR cell and depends on the FR of the target NR cell. It is defined in table 6.2.3.2.1-1. Note that Tidentify-NR = TPSS/SSS-sync + Tmeas, in which TPSS/SSS-sync is the cell search time and Tmeas is the measurement time due to cell selection criteria evaluation.

TSI-NR: It is the time required for acquiring all the relevant system information of the target NR cell. This time depends upon whether the UE is provided with the relevant system information of the target NR cell or not by the old NR cell before the RRC connection is released. TRACH: It is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell. TRACH can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then:

-	the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise,

-	there is no requirement if the SSB transmission periodicity is larger than 20 ms.

Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR

| FR of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (680 ms, 11 x Trs) |
| FR2-1 | MAX (880 ms, NFBSx11 x Trs) + Tproc |
| FR2-2 | MAX (880 ms, 12x11 x Trs) |
| NOTE: If the UE has been provided with higher layer signalling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. NOTE2: If the UE supports [Rel-19 reduced L3 beam sweeping capability], NFBS = [reduced beam sweeping factor] indicated via UE capability and Tproc = 2 ms, otherwise NFBS = 8 and Tproc = 0. |  |

<End of Change 4>

<Start of Change 5>

9.2.5.1	Intrafrequency cell identification

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise the UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 with SCS smaller or equal to 480 kHz.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TSSB_processing) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra + TSSB_processing) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TPSS/SSS_sync_intra is given in table 9.2.5.1-11; otherwise, TPSS/SSS_sync_intra is given in table 9.2.5.1-2.

-	For UE indicating no-gap-no-interruption via NeedForInterruptionInfoNR-r18, TPSS/SSS_sync_intra is given in table 9.2.5.1-1 for FR1 and  table 9.2.5.1-2 for FR2. For UE indicating no-gap-with-interruption via NeedForInterruptionInfoNR-r18, TPSS/SSS_sync_intra is given in table 9.2.5.1-17 for FR1 and table 9.2.5.1-18 for FR2.

-	Otherwise, TPSS/SSS_sync_intra is given in tables 9.2.5.1-1, 9.2.5.1-2, 9.2.5.1-4 (deactivated SCell) or 9.2.5.1-5 (deactivated SCell) or 9.2.5.1-9 (deactivated SCell) or 9.2.5.1-11 or 9.2.5.1-12 (deactivated PSCell) or 9.2.5.1-13 (deactivated PSCell).

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured

-	For UE indicatting no-gap-no-interruption via NeedForInterruptionInfoNR-r18, TSSB_time_index_intra is given in table 9.2.5.1-3 for FR1 and table 9.2.5.1-15 for FR2-2. For UE indicating no-gap-with-interruption via NeedForInterruptionInfoNR-r18, TSSB_time_index_intra is given in table 9.2.5.1-19 for FR1.

-	Otherwise, TSSB_time_index_intra is given in tables 9.2.5.1-3, 9.2.5.1-15 (FR2-2), 9.2.5.1-6 (deactivated SCell), 9.2.5.1-10(deactivated SCell) or 9.2.5.1-14 (deactivated PSCell).

-	TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TSSB_measurement_period_intra is given in table 9.2.5.2-7; otherwise, TSSB_measurement_period_intra is given in table 9.2.5.2-2.

-	For UE indicating no-gap-no-interruption via NeedForInterruptionInfoNR-r18, TSSB_measurement_period_intra is given in table 9.2.5.2-1 for FR1 and  table 9.2.5.2-2 for FR2. For UE indicating no-gap-with-interruption via NeedForInterruptionInfoNR-r18, TSSB_measurement_period_intra is given in table 9.2.5.2-10 for FR1 and  table table 9.2.5.2-11 for FR2.

-	For power class 6 UE supporting measEnhCAInterFreqFR2-r18 when highSpeedMeasFlagFR2 is configured, the TSSB_measurement_period_intra given in table 9.2.5.2-7 (if SMTC ≤ 40ms) and table 9.2.5.2-2 (if SMTC > 40ms) shall apply for SCC.

-	Otherwise, TSSB_measurement_period_intra is given in table 9.2.5.2-1, table 9.2.5.2-2, table 9.2.5.2-3 (deactivated SCell), 9.2.5.2-4 (deactivated SCell), 9.2.5.2-5 or 9.2.5.2-6 (deactivated SCell), 9.2.5.2-8 (deactivated PSCell) or 9.2.5.2-9 (deactivated PSCell).

TSSB_processing: it is the time period used to process multiple beams received in one SMTC. TSSB_processing = 0 ms for UE not supporting [reduced Rx BSF capability], and TSSB_processing = 2 ms for UE supporting [reduced Rx BSF capability] and it is activated.

-	CSSFintra: it is a carrier specific scaling factor and is determined

-	according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e.

-	 when intra-frequency SMTC is fully non overlapping or partially overlapping with GAP, or

-	 when intra-frequency SMTC is fully non overlapping with GAP for UE indicating no-gap-with-interruption, or

-	 when intra-frequency SMTC is fully non overlapping or partially overlapping with GAP for UE indicating no-gap-no-interruption, or

For a UE that supports Pre-MG, an SMTC occasion is only considered to be overlapped by Pre-MG if the Pre-MG is activated.

if the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24.  For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 6, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps = 36.

Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting power class 6, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36.

MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72 samples. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48 samples. For a UE supporting FR2 power class 3, MSSB_index_intra = 48 samples.

For a UE that supports [reduced Rx BSF capability], when highSpeedMeasFlagFR2-r17 is not configured and [reduced Rx BSF capability is activated], the following values shall apply for Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps:

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =3* Nreduced_Rx_BSF.

Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =3* Nreduced_Rx_BSF.

Where,

Nreduced_Rx_BSF is the reduced UE Rx beam sweeping factor reported by UE via [UE capability signalling including reduced Rx BSF value].

When UE supports concurrent GAPs, i.e., supports the following capability or capabilities’ combination:

-	concurrentMeasGap-r17, or

-	concurrentMeasGapsPreMG-r18, or

-	concurrentMeasGapsNCSG-r18,

Or when UE supports musim-GapPreference-r17 or both concurrent measurement GAPs and musim-GapPreference-r17 and UE concurrent GAPs or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured

Kp is the scaling factor for an SSB frequency layer to be measured without GAP. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

Kp = 1 when Navailable = 0.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W.

When UE supports [MUSIM-GapConfig-17] and the SMTC occasion of the target frequency layer is overlapping with the configured aperiodic MUSIM gap, longer cell identification period for the target frequency layer is expected.

-	Otherwise, when the UE is not configured with or UE does not support concurrent GAPs and the UE is not configured with periodic MUSIM gaps or UE does not support MUSIM gaps:

When intra-frequency SMTC is fully non overlapping with measurement gaps or NCSG, or intra-frequency SMTC is fully overlapping with MGs or NCSG, Kp=1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When intra-frequency SMTC is partially overlapping with NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP. For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.	If the higher layer signaling in TS 38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

If MCG DRX is in use, cell identification requirements for intra-frequency measurement in MCG specified in table 9.2.5.1-1, table 9.2.5.1-2, table 9.2.5.1-3, table 9.2.5.1-4, table 9.2.5.1-5 and table 9.2.5.1-6 shall depend on the MCG DRX cycle. If SCG DRX is in use, cell identification requirements for intra-frequency measurement in SCG specified in table 9.2.5.1-1, table 9.2.5.1-2, table 9.2.5.1-3, table 9.2.5.1-4, table 9.2.5.1-5, table 9.2.5.1-6, table 9.2.5.1-12, table 9.2.5.1-13 and table 9.2.5.1-14 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

-	When the target SSB is completely contained in active BWP of UE or the active downlink BWP is initial BWP, the intra-frequency measurement shall be conducted without gap and without interruption regardless of the NeedForGaps’ status reporting.

Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, ceil(Mpss/sss_sync_w/o_gaps x KFR x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x KFR x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(Mpss/sss_sync_w/o_gaps x KFR x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Table 9.2.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.2.5.1-4: Time period for PSS/SSS detection, deactivated SCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 9.2.5.1-5: Time period for PSS/SSS detection, deactivated SCell (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 9.2.5.1-6: Time period for time index detection, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(3 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(3 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 9.2.5.1-7: Void

Table 9.2.5.1-8: Void

Table 9.2.5.1-9: Time period for PSS/SSS detection, deactivated SCell (FR1), when highSpeedMeasCA-Scell-r17 is configured

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp) x max(measCycleSCell, M2 Note 1xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.2.5.1-10: Time period for time index detection, deactivated SCell (FR1)，when highSpeedMeasCA-Scell-r17 is configured

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(3 x Kp) x max(measCycleSCell, M2 Note 1xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(3 x Kp)x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.2.5.1-11: Time period for PSS/SSS detection when highSpeedMeasFlagFR2-r17 is configured, (FR2) when SMTC period ≤ 40ms

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 80 ms | max(600 ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80 ms< DRX cycle≤ 320 ms | ceil(1.5 x Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle) x CSSFintra |
| DRX cycle>320 ms | ceil(Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 NOTE 3: Void |  |

Table 9.2.5.1-12: Time period for PSS/SSS detection, deactivated PSCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Table 9.2.5.1-13: Time period for PSS/SSS detection, deactivated PSCell (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Table 9.2.5.1-14: Time period for time index detection, deactivated PSCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(3 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(3 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Table 9.2.5.1-15: Time period for time index detection (Frequency range FR2-2)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(200 ms, ceil(MSSB_index_intra x Kp x SMTC period) x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5 x MSSB_index_intra x Kp) x max(SMTC period, DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | Ceil(MSSB_index_intra x Kp )x DRX cycle x CSSFintra |

Table 9.2.5.1-16: Void

Table 9.2.5.1-17: Time period for PSS/SSS detection for UE indicating no-gap-with-interruption, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, 5 x max (80 ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320 ms] | max( 600 ms, ceil(M2 Note 2x 5) x [max(80 ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320 ms] | 5 x [DRX cycle x] CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 5: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates no-gap-with-interruption. |  |

Table 9.2.5.1-18: Time period for PSS/SSS detection for UE indicating no-gap-with-interruption, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, ceil(Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x max (80 ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320 ms] | max(600 ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x [max(80 ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320 ms] | [ceil(Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x DRX cycle] x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. NOTE 3: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates no-gap-with-interruption. |  |

Table 9.2.5.1-19: Time period for time index detection for UE indicating no-gap-with-interruption (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, 3 x max (80 ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320 ms] | max(120 ms, ceil (M2 Note 2 x 3) x [max(80 ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320 ms] | 3 x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 NOTE 5: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates no-gap-with-interruption. |  |

Editor’s note: RAN4 has to decide the UE behaviour when DRX is condifured whether interruptions are allowed.

Table 9.2.5.1-20: Void

Table 9.2.5.1-21: Void

Table 9.2.5.1-22: Void

Table 9.2.5.1-23: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120 ms, ceil(7 x Kp) x SMTC period) |
| DRX cycle≤ 320 ms | max(120ms, ceil(M2 x 7 x Kp) x max(SMTC period,DRX cycle)) |
| DRX cycle>320 ms | ceil(7 x Kp) x DRX cycle |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |  |

<End of Change 5>

<Start of Change 6>

9.2.6.2	Intra-frequency cell identification

When a measurement gap is provided or an activated Pre-MG is provided without any pre-MG status changed during the measurement period, the UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra-frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 with SCS smaller or equal to 480 kHz.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TSSB_processing ms

Tidentify_intra_with_index = TPSS/SSS_sync_ntra + TSSB_measurement_period_intra + TSSB_time_index_intra + TSSB_processing ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in tables 9.2.6.2-1, 9.2.6.2-2 or 9.2.6.2-9.

-	For UEs supporting [Rel-19 L3 fast beam sweeping (FBS) capability] when the FBS operation is activated, TSSB_processing = 2 ms; otherwise, TSSB_processing = 0.

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TPSS/SSS_sync_intra is given in table 9.2.6.2-9; otherwise, TPSS/SSS_sync_intra is given in table 9.2.6.2-2.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2.6.2-3 or 9.2.6.2-10 (for FR2-2).

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2.6.3-1 or 9.2.6.3-2.

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TSSB_measurement_period_intra is given in table 9.2.6.3-4; otherwise, T SSB_measurement_period_intra is given in table 9.2.6.3-2.

-	For power class 6 UE supporting measEnhCAInterFreqFR2-r18 when highSpeedMeasFlagFR2-r17 is configured, the TSSB_measurement_period_intra given in table 9.2.6.3-4 (if SMTC ≤ 40ms) and table 9.2.6.3-2 (if SMTC > 40ms) shall apply for SCC.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with or the UE does not support concurrent GAPs or MUSIM gaps. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period, xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps and per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other GAP and MUSIM gap occasions within the window, and

Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W after accounting for GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

When concurrent GAPs are configured, requirements in this clause do not apply if Navailable =0.

When UE supports [MUSIM-GapConfig-17] and the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in clause 9.1.10.4 is applied, longer cell identification period for the target intra-frequency is expected.

Mpss/sss_sync_with_gaps : For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync with_gaps=40. For a UE supporting FR2-1 power class 2, Mpss/sss_sync with_gaps =24.  For a UE supporting FR2-1 power class 3, Mpss/sss_sync with_gaps = 24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-1 power class 6, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync with_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync with_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync with_gaps = 36.

Mmeas_period_ with_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_ with_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_ with_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_ with_gaps = 24. For a UE supporting FR2-1 power class 4, Mmeas_period with_gaps =24. For a UE supporting FR2-1 power class 6, Mmeas_period with_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_ with_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_ with_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_ with_gaps = 36.

-	MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48. For a UE supporting FR2 power class 3, MSSB_index_intra = 48.

For a UE that supports [reduced Rx BSF capability], when highSpeedMeasFlagFR2-r17 is not configured and [reduced Rx BSF capability is activated], the following values shall apply for Mpss/sss_sync_with_gaps and Mmeas_period_with_gaps:

Mpss/sss_sync_with_gaps: For a UE supporting FR2-1 power class 3, Mpss/sss_sync_with_gaps =3* Nreduced_Rx_BSF.

Mmeas_period_with_gaps: For a UE supporting FR2-1 power class 3, Mmeas_period_with_gaps =3* Nreduced_Rx_BSF.

Where,

Nreduced_Rx_BSF is the reduced UE Rx beam sweeping factor reported by UE via [UE capability signalling including reduced Rx BSF value].

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index.

If MCG DRX is in use, cell identification requirements for intra-frequency measurement in MCG specified in table 9.2.6.2-1, table 9.2.6.2-2, and table 9.2.6.2-3 shall depend on the MCG DRX cycle. If SCG DRX is in use, cell identification requirements for intra-frequency measurement in SCG specified in table 9.2.6.2-1, table 9.2.6.2-2, and table 9.2.6.2-3 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

Table 9.2.6.2-1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, 5 x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(M2Note 1x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil( 5 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

Table 9.2.6.2-2: Time period for PSS/SSS detection (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, Mpss/sss_sync_with_gaps x KFR x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(1.5x Mpss/sss_sync_with_gaps x KFR x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(Mpss/sss_sync_with_gaps x KFR x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 2: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Table 9.2.6.2-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, ceil(3 x Kgap ) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil(M2Note 1x 3 x Kgap) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | Ceil(3 x Kgap )x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

Table 9.2.6.2-7: Void

Table 9.2.6.2-8: Void

Table 9.2.6.2-8: Void

Table 9.2.6.2-9: Time period for PSS/SSS detection when highSpeedMeasFlagFR2-r17 is configured, (FR2) when SMTC period ≤ 40ms

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, M1Note 2 x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 80 ms | max(600 ms, ceil(M1Note2 x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| 80 ms< DRX cycle≤ 320 ms | max(600 ms, ceil(Mpss/sss_sync_with_gaps x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil( Mpss/sss_sync_with_gaps x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 NOTE 3: Void |  |

Table 9.2.6.2-10: Time period for time index detection (FR2-2)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(200 ms, ceil(MSSB_index_intra x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5 x MSSB_index_intra x Kgap) x max(MGRP, SMTC period, DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | Ceil(MSSB_index_intra x Kgap)x DRX cycle x CSSFintra |

Table 9.2.6.2-11: Void

Table 9.2.6.1-12: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (FR1) (FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120 ms, 7 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(120ms, ceil(M2 x 7) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | 7 x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: Void NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |  |

<End of Change 6>

<Start of Change 7>

9.2.7	Intra-frequency measurements with NCSG

9.2.7.1	Intra-frequency cell identification

For the UE supporting NCSG, if NCSG is provided, the UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TSSB_processing) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_ntra + TSSB_measurement_period_intra + TSSB_time_index_intra + TSSB_processing) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2.7.1-1, 9.2.7.1-2, 9.2.7.1-4 (deactivated Scell) or 9.2.7.1-5 (deactivated SCell).

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2.7.1-3 or 9.2.7.1-6 (deactivated SCell).

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2.7.2-1, 9.2.7.2-2, 9.2.7.2-3, 9.2.7.2-4 (deactivated SCell) or 9.2.7.2-5 (deactivated SCell).

TSSB_processing: it is the time period used to process multiple beams received in one SMTC. TSSB_processing = 0 ms for UE not supporting [reduced Rx BSF capability] and TSSB_processing = 2 ms for UE supporting [reduced Rx BSF capability] and it is activated.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_ncsg,i in clause 9.1.5.3 for measurement conducted within NCSG.

KNCSG is the scaling factor for a SSB frequency layer to be measured within an associated NCSG pattern. KNCSG = 1 when the UE is not configured with concurrent GAPs. Otherwise, KNCSG = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period, xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAP and per-FR GAP within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated NCSG within the window W, including those overlapped with other GAP occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated NCSG within the window W after accounting for GAP collisions by applying the GAP collision rule in clauses 9.1.8.3, 9.1.12.3, and 9.1.13.3.

-	xRP = MGRP when configured GAP is MG, and xRP = VIRP when configured GAP is NCSG.

When concurrent GAPs are configured, requirements in this clause do not apply if Navailable =0.

Mpss/sss_sync_with_ncsg: For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync with_ncsg=40. For a UE supporting FR2 power class 2, Mpss/sss_sync with_ncsg =24. For a UE supporting FR2 power class 3, Mpss/sss_sync with_ncsg =24. For a UE supporting power class 4, Mpss/sss_sync with_ncsg =24

Mmeas_period_ with_ncsg: For a UE supporting power class 1 or 5, Mmeas_period_ with_ncsg =40. For a UE supporting power class 2, Mmeas_period_ with_ncsg =24. For a UE supporting power class 3, Mmeas_period_ with_ncsg =24. For a UE supporting power class 4, Mmeas_period with_ncsg =24.

For a UE that supports [reduced Rx BSF capability], when highSpeedMeasFlagFR2-r17 is not configured and [reduced Rx BSF capability is activated], the following values shall apply for Mpss/sss_sync_with_ncsg and Mmeas_period_with_ncsg:

Mpss/sss_sync_with_ncsg: For a UE supporting FR2-1 power class 3, Mpss/sss_sync_with_ncsg =3* Nreduced_Rx_BSF.

Mmeas_period_with_ncsg: For a UE supporting FR2-1 power class 3, Mmeas_period_with_ncsg =3* Nreduced_Rx_BSF.

Where,

Nreduced_Rx_BSF is the reduced UE Rx beam sweeping factor reported by UE via [UE capability signalling including reduced Rx BSF value].

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with NCSG and smtc2 is partially overlapping with NCSG, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index.

Table 9.2.7.1-1: Time period for PSS/SSS detection with NCSG (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, 5 x KNCSG x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(M2Note 1x 5 x KNCSG) x max(VIRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | 5 x KNCSG x max(VIRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

Table 9.2.7.1-2: Time period for PSS/SSS detection with NCSG (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, Mpss/sss_sync_with_ncsg x KNCSG x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600ms, ceil(1.5x Mpss/sss_sync_with_ncsg x KNCSG) x max(VIRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Mpss/sss_sync_with_ncsg x KNCSG x max(VIRP, DRX cycle) x CSSFintra |

Table 9.2.7.1-3: Time period for time index detection with NCSG (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, 3 x KNCSG x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil(M2Note 1x 3 x KNCSG) x max(VIRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | 3 x KNCSG x max(VIRP, DRX cycle) x CSSFintra |
| NOTE 1: highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

Table 9.2.7.1-4: Time period for PSS/SSS detection with NCSG (deactivated SCell) (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | 5 x KNCSG x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320 ms | 5 x KNCSG x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | 5 x KNCSG x max(measCycleSCell, VIRP, DRX cycle) x CSSFintra |

Table 9.2.7.1-5: Time period for PSS/SSS detection with NCSG (deactivated SCell) (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Mpss/sss_with_ncsg x KNCSG x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320 ms | Mpss/sss_with_ncsg x KNCSG x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Mpss/sss_with_ncsg x KNCSG x max(measCycleSCell, VIRP, DRX cycle) x CSSFintra |

Table 9.2.7.1-6: Time period for time index detection with NCSG (deactivated SCell) (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | 3 x KNCSG x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320 ms | 3 x KNCSG x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | 3 x KNCSG x max(measCycleSCell, VIRP,DRX cycle) x CSSFintra |

<End of Change 7>

<Start of Change 8>

9.3.4	Inter-frequency measurement with measurement gaps

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured) or deriveSSB-IndexFromCellInter-r17 is configured for the FR1 and FR2-1 target frequency layers and UE supporting deriveSSB-IndexFromCellInterNon-NCSG-r17. Otherwise UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter-frequency SS block of an already detected cell within Tidentify_inter_without_index.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_processing) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ TSSB_processing) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3.4-1, table 9.3.4-2, table 9.3.4-5 when highSpeedMeasInterFreq-r17 is configured and UE supports measurementEnhancementInterFreq-r17 and table 9.3.4-9 when highSpeedMeasFlagFR2-r17 is configured and UE supports measEnhCAInterFreqFR2-r18. When the SCG is deactivated, table 9.3.4-7 applies for an inter-frequency carrier configured by SCG and not configured by MCG and table 9.3.4-2 applies for an inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.4-2 applies for an inter-frequency carrier configured only by MCG.

-	For UE supporting power class 6 and measEnhCAInterFreqFR2-r18 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TPSS/SSS_sync_inter is given in table 9.3.4-9; otherwise, TPSS/SSS_sync_inter is given in table 9.3.4-2.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 9.3.4-3, table 9.3.4-6 when highSpeedMeasInterFreq is configured and UE supports measurementEnhancementInterFreq-r17, and table 9.3.4-10 when highSpeedMeasFlagFR2-r17 is configured and UE supports measEnhCAInterFreqFR2-r18. When the SCG is deactivated, table 9.3.4-8 applies for an inter-frequency carrier configured by SCG and not configured by MCG and table 9.3.4-4 applies for an inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.4-4 applies for an inter-frequency carrier configured only by MCG.

-	For UE supporting power class 6 and measEnhCAInterFreqFR2-r18 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TSSB_measurement_period_inter is given in table 9.3.5-5; otherwise, TSSB_measurement_period_inter is given in table 9.3.5-2.

TSSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3.5-1, table 9.3.5-2, table 9.3.5-3 when highSpeedMeasInterFreq is configured and UE supports measurementEnhancementInterFreq-r17, and in table 9.3.5-5 when highSpeedMeasFlagFR2-r17 is configured and UE supports measEnhCAInterFreqFR2-r18. When the SCG is deactivated, table 9.3.5-4 applies for an inter-frequency carrier configured by SCG and not configured by MCG and table 9.3.5-2 applies for an inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.5-2 applies for an inter-frequency carrier configured only by MCG.

TSSB_processing: the time period used to process multiple beams received in one SMTC. TSSB_processing = 0 ms for UE not supporting [fast beam sweeping factor for L3 measurement] and TSSB_processing = 2 ms for UE supporting [fast beam sweeping factor for L3 measurement] and it is activated.

-	For UE supporting power class 6 and measEnhCAInterFreqFR2-r18 with highSpeedMeasFlagFR2-r17 configured, TSSB_measurement_period_inter is given in table 9.3.5-5; otherwise, TSSB_measurement_period_inter is given in table 9.3.5-2.

Mpss/sss_sync_inter: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_inter = 64 samples. For a UE supporting FR2-1 power class 2, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_inter = 96. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_inter = 60. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_inter = 60.

MSSB_index_inter: For a UE supporting FR2-1 power class 1 or 5, MSSB_index_inter = 40 samples. For a UE supporting FR2 power class 2, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 3, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 4, MSSB_index_inter = 24 samples. For a UE supporting FR2-2 power class 2 or 3, MSSB_index_inter = 48 samples. For a UE supporting FR2 power class 1, MSSB_index_inter = 72 samples.

Mmeas_period_inter: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_inter =64. For a UE supporting FR2-1 power class 2, Mmeas_period_inter=40. For a UE supporting FR2-1 power class 3, Mmeas_period_inter =40. For a UE supporting FR2-1 power class 4, Mmeas_period_inter = 40. For a UE supporting FR2-2 power class 1, Mmeas_period_inter = 96. For a UE supporting FR2-2 power class 2, Mmeas_period_inter = 60. For a UE supporting FR2-2 power class 3, Mmeas_period_inter = 60.

For a UE that supports [reduced Rx BSF capability], when highSpeedMeasFlagFR2-r17 is not configured and [reduced Rx BSF capability is activated], the following values shall apply for Mpss/sss_sync_inter, MSSB_index_inter and Mmeas_period_inter:

Mpss/sss_sync_inter: For a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter =5* Nreduced_Rx_BSF.

MSSB_index_inter: For a UE supporting FR2-1 power class 3, MSSB_index_inter = 3* Nreduced_Rx_BSF.

Mmeas_period_inter: For a UE supporting FR2-1 power class 3, Mmeas_period_inter =5* Nreduced_Rx_BSF.

Where,

Nreduced_Rx_BSF is the reduced UE Rx beam sweeping factor reported by UE via [UE capability signalling including reduced Rx BSF value].

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

Kgap is a scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent GAPs or MUSIM gaps. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE measurement GAPs, periodic MUSIM gaps, and/or per-FR measurement GAPs within the same FR, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other GAP occasions and MUSIM gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for the GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Kgap is only applicable for UE supporting concurrent GAPs or MUSIM gaps. When concurrent GAPs or MUSIM gaps are configured, requirements in this clause do not apply if Navailable =0.

When UE supports [musim-GapPreference-r17] and if the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in clause 9.1.10.4 is applied, longer cell identification period for the target inter-frequency is expected.

Table 9.3.4-1: Time period for PSS/SSS detection (FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(8 * Kgap) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(8*1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(8 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-2: Time period for PSS/SSS detection, (FR2)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(Kgap Mpss/sss_sync_inter x KFR) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(1.5 * Kgap Mpss/sss_sync_inter x KFR) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap Mpss/sss_sync_inter x KFR) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. NOTE 4: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Table 9.3.4-3: Time period for time index detection (FR1)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120 ms, Ceil(3 * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil(3 1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(3 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-4: Time period for time index detection (FR2)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200 ms, Ceil(Kgap MSSB_index_inter) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(1.5 * Kgap MSSB_index_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap MSSB_index_inter) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-5: Time period for PSS/SSS detection when highSpeedMeasInterFreq-r17 is configured (FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600 ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7 |
| DRX cycle ≤ 160 ms | max(600 ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2 |
| 160 ms < DRX cycle ≤ 320 ms | ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2 |
| DRX cycle>320 ms | N4 x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: N4=6 if SMTC periodicity > 40 ms, otherwise N4=5 |  |

Table 9.3.4-6: Time period for time index detection when highSpeedMeasInterFreq-r17 is configured (FR1)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120 ms, 3 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil(3 M2 NOTE3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | 3 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |  |

Table 9.3.4-7: Time period for PSS/SSS detection when the inter-frequency carrier is configured only by SCG and the SCG is deactivated (FR2)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(Kgap Mpss/sss_sync_inter) Max(MGRP , measCyclePSCell)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(1.5 * Kgap Mpss/sss_sync_inter) Max(MGRP, measCyclePSCell, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap Mpss/sss_sync_inter) Max(measCyclePSCell, DRX cycle) CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-8: Time period for time index detection when inter-frequency carrier is configured only by SCG and the SCG is deactivated (FR2)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200 ms, Ceil(Kgap MSSB_index_inter) Max(MGRP, measCyclePSCell)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(1.5 * Kgap MSSB_index_inter) Max(MGRP, measCyclePSCell, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap MSSB_index_inter) Max(measCyclePSCell, DRX cycle) CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-9: Time period for PSS/SSS detection when highSpeedMeasFlagFR2-r17 is configured, (FR2-1) when SMTC period ≤40ms

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600 ms, M1Note 3 x Kgap x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle≤ 80 ms | max(600 ms, ceil(M1Note 3 x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| 80 ms< DRX cycle≤ 320 ms | max(600 ms, ceil(Mpss/sss_sync_with_gaps x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle>320 ms | Ceil( Mpss/sss_sync_with_gaps x Kgap ) x max(MGRP, DRX cycle) x CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. NOTE 3: For UE supporting power class 6 and measEnhCAInterFreqFR2-r18, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 |  |

Table 9.3.4-10: Time period for time index detection when when highSpeedMeasFlagFR2-r17 is configured (Frequency range FR2-1) when SMTC period <= 40 ms

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200 ms, Ceil(Kgap , M1Note 3) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle≤ 80 ms | Max(200 ms, Ceil(1.5 * Kgap M1Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| 80 ms< DRX cycle≤ 320 ms | Max(200 ms, Ceil(1.5 * Kgap MSSB_index_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap MSSB_index_inter) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. NOTE 3: For UE supporting power class 6 and measEnhCAInterFreqFR2-r18, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 |  |

Table 9.3.4-11: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| Condition NOTE1 | TSSB_time_index_inter_less_than_5Mhz |
| --- | --- |
| No DRX | Max(120 ms, Ceil(6 * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120ms, Ceil(6 M2 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(6* Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. |  |

9.3.4.1	Void

9.3.4.2	Void

<End of Change 8>