3GPP TSG-RAN4 Meeting #53 	R4-094424

# Jeju, Korea, November 9 – 13, 2009

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 398 | rev | - | Current version: | 9.1.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Band 1- PHS coexistence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Europe |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-RF |  |  |  |  |  |  | Date: |  | 10/30/2009 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change: |  | There is no A-MPR defined for 16QAM. The frequency ranges of applicability in Table 6.6.3.2-1 and in Table 6.6.3.3.1-1 are not fully consistent with each other due to a 300kHz PHS channel offset. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the “for QPSK” qualifier for A-MPR. Editorial changes in the notes in Table 6.6.3.2-1 Change the guardband from BW+4MHz to BW+4.3MHz in Table 6.6.3.3.1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Overly tight requirements for 16QAM |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4, 6.6.3.2. 6.6.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-1 |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

### 6.2.4	UE Maximum Output Power with additional requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2.2-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 the specific requirements and identified sub-clauses are specified in Table 6.2.4-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4.-1 and 6.2.4-2 are in addition to the allowed MPR requirements specified in clause 6.2.3.

Table 6.2.4-1: Additional Maximum Power Reduction (A-MPR) / Spectrum Emission requirements

| Network Signalling value | Requirements (sub-clause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 | NA | NA | NA | NA | NA |
| NS_03 | 6.6.2.2.1 | 2, 4,10, 35, 36 | 3 | >5 | ≤ 1 |
|  | 6.6.2.2.1 | 2, 4,10, 35,36 | 5 | >6 | ≤ 1 |
|  | 6.6.2.2.1 | 2, 4,10, 35,36 | 10 | >6 | ≤ 1 |
|  | 6.6.2.2.1 | 2, 4,10,35,36 | 15 | >8 | ≤ 1 |
|  | 6.6.2.2.1 | 2, 4,10,35, 36 | 20 | >10 | ≤ 1 |
| NS_04 | 6.6.2.2.2 | TBD | TBD | TBD |  |
| NS_05 | 6.6.3.3.1 | 1 | 10,15,20 | ≥ 50 | ≤ 1 |
| NS_06 | 6.6.2.2.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | n/a | n/a |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 | 10 | Table 6.2.4-2 | Table 6.2.4-2 |
| NS_08 | 6.6.3.3.3 | 19 | 10, 15 | > 29 | ≤ 1 |
|  |  |  |  | > 39 | ≤ 2 |
|  |  |  |  | > 44 | ≤ 3 |
| .. |  |  |  |  |  |
| NS_32 | - | - | - | - | - |

## << Unchanged sections omitted >>

#### 6.6.3.2	Spurious emission band UE co-existence

This clause specifies the requirements for the specified E-UTRA band

# Table 6.6.3.2-1: Requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Level (dBm) | Bandwidth (MHz) | Comment |
| 1 | E-UTRA Band 1, 3, 7, 8, 9, 11, 34, 38, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | Note6,Note7 |
|  |  | 1884.5 | - | 1915.7 |  |  | Note 6, Note8 |
|  | E-UTRA band 33 | 1900 | - | 1920 | -50 | 1 | Note 3 |
|  | E-UTRA band 39 | 1880 | - | 1920 | -50 | 1 | Note 3 |
| 2 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17 | FDL_low | - | FDL_high | -50 | 1 |  |
| 3 | E-UTRA Band 1, 3, 7, 8, 33, 34, 38 | FDL_low | - | FDL_high | -50 | 1 |  |
| 4 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17 | FDL_low | - | FDL_high | -50 | 1 |  |
| 5 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17 | FDL_low | - | FDL_high | -50 | 1 |  |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | Note7 |
|  |  | 1884.5 | - | 1915.7 |  |  | Note8 |
| 7 | E-UTRA Band 1, 3, 7, 8, 33, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 38 | 2570 | - | 2620 | -50 | 1 | Note 3 |
| 8 | E-UTRA Band 1, 8, 7, 33, 34, 38, 39, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | 1805 | - | 1830 | -50 | 1 | Note 4 |
|  | E-UTRA band 3 | 1805 | - | 1880 | -36 | 0.1 | Note 2,4 |
|  | E-UTRA band 3 | 1830 | - | 1880 | -50 | 1 | Note 4 |
|  | E-UTRA band 7 | 2640 | - | 2690 | -50 | 1 | Note 4 |
|  | E-UTRA band 7 | 2640 | - | 2690 | -36 | 0.1 | Note 2,4 |
| 9 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | Note7 |
|  |  | 1884.5 | - | 1915.7 |  |  | Note8 |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17 | FDL_low | - | FDL_high | -50 | 1 |  |
| 11 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1919.6 | -41 | 0.3 | Note7 |
|  |  | 1884.5 | - | 1915.7 |  |  | Note8 |
| 12 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17 | FDL_low | - | FDL_high | -50 | 1 |  |
| 13 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 763 | - | 775 | -35 | 0.00625 |  |
| 14 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 763 | - | 775 | -35 | 0.00625 |  |
| 17 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17 | FDL_low | - | FDL_high | -50 | 1 |  |
| 18 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 895 | -40 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | Note7 |
|  |  | 1884.5 | - | 1915.7 |  |  | Note8 |
| 19 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 895 | -40 | 1 | Note9 |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | Note7 |
|  |  | 1884.5 | - | 1915.7 |  |  | Note8 |
| … |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 3, 8, 34, 38, 39, 40 | FDL_low | - | FDL_high | -50 | 1 | Note 5 |
| 34 | E-UTRA Band 1, 3, 7, 8, 9, 11, 33, 38,39, 40 | FDL_low | - | FDL_high | -50 | 1 | Note 5 |
|  | Frequency range | 860 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | Note7 |
|  |  | 1884.5 | - | 1915.7 |  |  | Note8 |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1,3, 33, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
| 39 | E-UTRA Band 34, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
| 40 | E-UTRA Band 1, 3, 33, 34, 39 | FDL_low | - | FDL_high | -50 | 1 |  |
| Note |  |  |  |  |  |  |  |
| 1 | FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 |  |  |  |  |  |  |
| 2 | As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd or 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RE within the transmission bandwidth (see Figure 5.6-1) for which the 2nd or 3rd harmonic, i.e. the frequency equal to two or three times the frequency of that RE, is within the measurement bandwidth. |  |  |  |  |  |  |
| 3 | To meet these requirements some restriction will be needed for either the operating band or protected band |  |  |  |  |  |  |
| 4 | Requirements are specified in terms of E-UTRA sub-bands |  |  |  |  |  |  |
| 5 | For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band |  |  |  |  |  |  |
| 6 | Applicable when NS_05 in section 6.6.3.3.1 is signalled by the network. |  |  |  |  |  |  |
| 7 | Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. |  |  |  |  |  |  |
| 8 | Applicable when co-existence with PHS system operating in 1884.5-1915.7MHz. |  |  |  |  |  |  |
| 9 | Applicable when NS_08 in section 6.6.3.3.3 is signalled by the network |  |  |  |  |  |  |

## << Unchanged sections omitted >>

##### 6.6.3.3.1 	Minimum requirement (network signalled value "NS_05")

When "NS_05" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.1-1. This requirement also applies for the frequency ranges that are less than ΔfOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.1-1: Additional requirements (PHS)

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| 1884.5 f 1919.6*1 | -41 | -41 | -41 | -41 | -41 | -41 | 300 KHz |
| 1884.5 f 1915.7*2 | -41 | -41 | -41 | -41 | -41 | -41 | 300 KHz |
| Note - Applicable when the lower edge of the assigned E-UTRA UL channel bandwidth is larger than or equal to the upper edge of PHS band (1919.6 MHz) + 4.4 MHz + the Channel BW assigned, where Channel BW is as defined in Subclause 5.6. Operations below this point are for further study. - Applicable when the lower edge of the assigned E-UTRA UL channel bandwidth is larger than or equal to the upper edge of PHS band (1915.7 MHz) + 4.3 MHz + the Channel BW assigned, where Channel BW is as defined in Subclause 5.6. Operations below this point are for further study. |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>