TSG RAN WG4 meeting #58                                                R4-110882

Taipei, 21st-25th February 2011

Source: 	ZTE

Title: 	Comments on framework for eDL-MIMO PDSCH demodulation performance requirements

Agenda Item:	6.6.2

Document for:	Approval

- Introduction

A framework [1] about eDL-MIMO PDSCH performance requirements was drafted by NTT DOCOMO in RAN4 #57 AH. Then the framework document was updated to version 3 through email discussion before Taipei meeting. We found some mistakes on some parameters of table A.2 (Fixed reference channel) in the up-to-date framework. In this contribution, these mistakes were modified and the new table was provided. In addition, an editorial error in figure A.2 was also modified.

2. Reference measurement channels for 1-2 layer transmission

According to TS36.213 [2], for frame structure type 1, the UE is not expected to receive PDSCH resource blocks where DMRS overlap with a transmission of either PBCH or primary or secondary synchronisation signals in the same subframe. In R10, the DMRS in PDSCH resource blocks transmitted in some RBs overlap with primary and secondary synchronisation signals in subframe 0. Therefore, the number of available RBs is 41 i.e. RB0- RB20 and RB30- RB49 and the 9 middle RBs don’t transmit user data in subframe 0 for FDD. So the information bit payload for subframe 0 shall be calculated according to 41 RBs. We have modified the associated parameters and gave the new note 3 in the table below. For the previous note 3, the annotation is the uplink-downlink configuration table for TDD and it shouldn’t be present in FRC for FDD.

Table A.2 Reference measurement channel for FDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R1. FDD | R2.1. FDD | R2.2. FDD | R3. FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 | 50 | 50 |
| Allocated subframes per Radio Frame |  | 10 | 10 | 10 | 10 |
| CSI-RS duty cycle | msec | 5 | 5 | 5 | 5 |
| CSI-RS subframe number |  | 2, 7 | 2, 7 | 2, 7 | 2, 7 |
| CSI-RS SubframeConfig |  | 2 | 2 | 2 | 2 |
| CSI-RS reference signal configuration |  | 1 | 1 | 1 | 1 |
| ZeroPowerCSI-RS bitmap |  |  |  | 0x01000 |  |
| Number of CSI-RS ports |  | 8 | 4 | 4 | 2 |
| Number of CRS ports |  | 2 | 2 | 2 | 2 |
| Modulation |  | QPSK | 16QAM | 16QAM | 16QAM |
| Target coding information rate |  | 1/3 | 1/2 | 1/2 | 1/2 |
| Information Bit Payload |  |  |  |  |  |
| - For Sub-Frames (non CSI-RS subframe) | Bits | 3624 | 11448 | 11448 | 11448 |
| - For Sub-Frames (CSI-RS subframe) | Bits | 3624 | 11448 | 11448 | 11448 |
| - For Sub-Frame 5 | Bits | n/a | n/a | n/a | n/a |
| - For Sub-Frame 0 | Bits | 2984 | 9528 | 9528 | 9528 |
| Number of Code Blocks per Sub-Frame (note 4) |  |  |  |  |  |
| - For Sub-Frames (non CSI-RS subframe) |  | 1 | 2 | 2 | 2 |
| - For Sub-Frames (CSI-RS subframe) |  | 1 | 2 | 2 | 2 |
| - For Sub-Frame 5 |  | n/a | n/a | n/a | n/a |
| - For Sub-Frame 0 |  | 1 | 2 | 2 | 2 |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |  |
| - For Sub-Frames (non CSI-RS subframe) | Bits | 12000 | 24000 | 24000 | 24000 |
| - For Sub-Frames (CSI-RS subframe) | Bits | 11200 | 23200 | 22400 | 23600 |
| - For Sub-Frame 5 | Bits | n/a | n/a | n/a | n/a |
| - For Sub-Frame 0 | Bits | 9840 | 19680 | 19680 | 19680 |
| Max. Throughput averaged over 1 frame | Mbps | 3.1976 | 10.1112 | 10.1112 | 10.1112 |
| Number of layers |  | 1 | 1 | 1 | 2 |
| Note 1: 2 symbols allocated to PDCCH for 10 MHz channel BW Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 Note 3: 50 resource blocks are allocated in sub-frames 1,2,3,4,6,7,8,9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0 Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |

In addition, there was an editorial error in figure A.2 (Reference signal configurations without PDSCH muting pattern (R.1, R.2.1 and R.3)) in the framework version 3. The order of CSI-RS pattern for 8 ports should be 0, 1, 4, 5, 2, 3, 6, 7, and not be 0, 1, 2, 3, 4, 5, 6, 7. We have modified them as follows.

0

1

2

3

4

5

6

7

8

9

10

11

12

13

0

1

2

3

4

5

6

7

8

9

10

11

12

13

0

1

2

3

4

5

6

7

8

9

10

11

12

13

11

0

1

0

1

0

1

10

4

5

9

8

7

6

5

2

3

2

3

4

6

7

3

2

1

0

CSI-RS reference configuration: 1

CSI-RS reference configuration: 1

CSI-RS reference configuration: 1

CRS port#1,2

DMRS(Rel-9/10)

CSI-RS

2 CSI-RS ports w/o PDSCH Muting

4 CSI-RS ports w/o PDSCH Muting

8 CSI-RS ports w/o PDSCH Muting

3. Conclusions

In this contribution, we revised some mistakes of parameters in the reference measurement channel in the up-to-date framework and provided the update table. In addition, an editorial error in figure A.2 was also modified.

4. References

[1] R4-110xxx, Framework for PDSCH demodulation performance requirements on eDL MIMO, NTT DOCOMO.

[2] TS 36.213 v 930.

[3] TS 36.101 v a11.