3GPP TSG-RAN WG4 #58AH	R4- 111788

Shanghai, China, April 11 – April 15, 2011

Agenda item:	5.1.3

Source: 	Qualcomm Incorporated

Title: 	MPR for multi-cluster and non-contiguous multi-carrier UL allocation

Document for:	Discussion

# Introduction

In [1] discussed at RAN4 #58 in Taipei, simulation of various multi-cluster waveforms was described. In this document we present some simulation results to compare against one of the cases shown in [1].

The purpose of these simulations is to see if we get similar results for MPR.

# Simulation Details

Simulation conditions are as follows:

- Two contiguous carriers – 20MHz+20MHz

- Two clusters per carrier (4 clusters total)

- 16 QAM Modulation

- Modulator IQ Image -25 dBc

- Modulator Carrier Leakage -25 dBc

- Approximately 3600 waveforms are simulated

- PA Model developed from measured data on a commercial device

The allocation ratio metric is used and the waveforms simulated have allocation ratios less than 5%.

# Results

For waveforms with allocation ratio <5% the simulations show that the 7.2 dB MPR proposed in [1] is reasonable. The additional margin above the simulation results is needed for device, temperature and battery voltage variation.

# Conclusion

We find the 7.2 dB MPR value for allocation ratio < 5% to be reasonable and acceptable.

# References

[1] R4-110955, “MPR for LTE multi cluster transmission”