3GPP TSG-RAN WG4 Meeting #62 	R4-120830

# Dresden, Germany, February 6-10, 2012

| CR-Form-v9.9 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 1076 | rev | - | Current version: | 10.5.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Band 41 CA CR for TS36.101, section 7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Clearwire |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_B41 |  |  |  |  |  |  | Date: |  | 26/01/2012 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | Band 41 CA UE recevier characteristics are not captured in TS 36.101. Study of receiver characteristics is complete and this CR adds the necessary changes to TS 36.101 to introduce support for intra-band CA in Band 41. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Band 41 intra-band CA support is introduced into the receiver characteristics section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 41 CA UE receiver characteristics will not be complete in specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1A, 7.6.1.1A, 7.6.2.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-1 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

7.3.1A 	Minimum requirements (QPSK) for CA

For CA bandwidth class A the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1-1 and table 7.3.1-2.

For the UE that supports inter band CA the reference sensitivity is defined to be met with both downlink component carriers active and either of the uplink component carriers active. The UE shall meet the requirements specified in clause 7.3.1.

For CA bandwidth class C the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1-1 and table 7.3.1A-1. For UE(s) supporting one uplink, the uplink configuration of the Primary CC shall be in accordance with Table 7.3.1-2.

Table 7.3.1A-1 specifies the maximum number of allocated uplink resource blocks for which the intra-band contiguous CA reference receive sensitivity requirement must be met. The PCC allocation follows table 7.3.1-2. SCC and PCC transmission forms a contiguous allocation.

PCC and SCC TX–RX frequency separations are as defined in Table 5.7.4-1.

Table 7.3.1A-1: Intra-band CA uplink configuration for reference sensitivity

| CA Band / Aggregated channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Band | 100RB+50RB |  | 75RB+75RB |  | 100RB+75RB |  | 100RB+100RB |  | Duplex Mode |
| CA_1C | n/a | n/a | PCC | SCC | n/a | n/a | PCC | SCC | FDD |
|  | n/a | n/a | 75 | 55 | n/a | n/a | 100 | 30 |  |
| CA_40C | PCC | SCC | PCC | SCC | n/a | n/a | PCC | SCC | TDD |
|  | 100 | 50 | 75 | 75 | n/a | n/a | 100 | 100 |  |
| CA_41C | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC | TDD |
|  | 100 | 50 | 75 | 75 | 100 | 75 | 100 | 100 |  |
| NOTE 1. The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 2. The transmitted power over both PCC and SCC shall be set to PUMAX as defined in clause 6.2.5. NOTE 3. The UL resource blocks in both PCC and SCC shall be confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |  |  |  |  |

## <Next changed section>

#### 7.6.1.1A	Minimum requirements for CA

For intra-band contiguous CA (bandwidth Class C) the downlink Secondary CC shall be configured at nominal channel spacing to the Primary CC with the Primary CC configured closest to the uplink band. Downlink Primary CC and Secondary CC are both activated. The uplink output power shall be set as specified in Table 7.6.1.1A-1 with the uplink configuration set according to Table 7.3.1A-1 for the applicable CA Band. For UE(s) supporting one uplink, the uplink configuration of the Primary CC shall be in accordance with Table 7.3.1-2.

The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Tables 7.6.1.1A-1 and Tables 7.6.1.1A-2 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.1.1A-1 and 7.6.1.1A-2.

Table 7.6.1.1A-1: In band blocking parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Power per CC in Aggregated Transmission Bandwidth Configuration | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  |  | 12 |  |  |  |
| BWInterferer | MHz |  | 5 |  |  |  |
| FIoffset, case 1 | MHz |  | 7.5 |  |  |  |
| FIoffset, case 2 | MHz |  | 12.5 |  |  |  |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L as defined in clause 6.2.5A Note 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |  |

Table 7.6.1.1A-2: In-band blocking

| CA operating band | Parameter | Unit | Case 1 | Case 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |  |
|  | FInterferer | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |  |
| CA_1C, CA_40C, CA_41C | FInterferer (Range) | MHz | (Note 2) | FDL_low – 15 to FDL_high + 15 |  |
| Note 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band Note 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 Note 3: Foffset is the frequency offset from the center frequency of the adjacent CC being tested to the edge of aggregated channel bandwidth. Note 4: The Finterferer (offset) is relative to the center frequency of the adjacent CC being tested and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

## <Next changed section>

#### 7.6.2.1A	Minimum requirements for CA

For intra-band contiguous CA (bandwidth Class C) the downlink Secondary CC shall be configured at nominal channel spacing to the Primary CC with the Primary CC configured closest to the uplink band. Downlink Primary CC and Secondary CC are both activated. The uplink output power shall be set as specified in Table 7.6.2.1A-1 with the uplink configuration set according to Table 7.3.1A-1 for the applicable CA Band. For UE(s) supporting one uplink, the uplink configuration of the Primary CC shall be in accordance with Table 7.3.1-2.

The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Tables 7.6.2.1A-1 and Tables 7.6.2.1A-2 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.1.1A-1 and 7.6.1.1A-2.

For Table 7.6.2.1A-2 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of aggregated resource blocks in the downlink transmission bandwidth configuration. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

For Table 7.6.2.1A-2 in frequency range 4, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of aggregated resource blocks in the downlink transmission bandwidth configurations  and  is the number of resource blocks allocated in the uplink. For these exceptions the requirements of clause 7.7 spurious response are applicable.

Table 7.6.2.1A-1: Out-of-band blocking parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Power per CC in Aggregated Transmission Bandwidth Configuration | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  |  | 9 |  |  |  |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in TS 36.101 Table 7.3.1A-1, with PCMAX_L as defined in clause 6.2.5A. Note 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2. |  |  |  |  |  |  |

Table 7.6.2.1A-2: Out of band blocking

| CA operating band | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | range 1 | range 2 | range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_1C, CA_40C, CA_41C | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |

## << End of changes >>