3GPP TSG- RAN4 Meeting #62bis 	 R4-121365

# Jeju, South Korea, 26 - 30 March 2012

| CR-Form-v9.9 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 1216 | rev | - | Current version: | 10.5.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UL Transmit Timing Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility, Intel, Renesas, Fujitsu, Research in Motion, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10 |  |  |  |  |  |  | Date: |  | 19/02/2012 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | Current requirements for maximum adjustment step size in Table 7.1.2-2 does include all of the contributors to quantization error. There are 3 primary contributors to timing quantization for implementations using DigRF interface between baseband and RFIC: - Effect of baseband quantization (e.g., 2*Ts for 10 MHz, 16*Ts for 1.4 MHz, etc.) - DigRF protocol allowance (24 ns as allowed by the MIPI standard applied at both ends of the DigRF interface leading up to a 48 ns = 1.5*Ts error) - RF and analog uncertainties (typically in the range 32-48 ns). The current requirements only allow for (1) above (i.e., effect of baseband quantization). The remaining two components are not included. Therefore an additional 2.5*Ts is added to the current Tq requirements for all bandwidths. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Tq in Table 7.1.2-2 is changed as below. Table 7.1.2-2: Tq Maximum Autonomous Time Adjustment Step Downlink Bandwidth (MHz); Current value for Tq_; Proposed value for Tq_. 1.4; 16*TS; 18.5*TS. 3; 8*TS; 10.5*TS. 5; 4*TS; 6.5*TS. 10; 2*TS; 4.5*TS. Note: TS is the basic timing unit defined in TS 36.211. |
| Consequences if not approved: |  | Unnecessarily tight requirements for UEs. Excludes DigRF implementations and can potentially result in delayed market entry if the test is enforced by operators. Isolated impact analysis: No change to UE requirements. Changes Test conditions only. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-3 CR... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

# Timing and signalling characteristics

## 7.1	UE transmit timing

### 7.1.1	Introduction

The UE shall have capability to follow the frame timing change of the connected eNode B. The uplink frame transmission takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are defined in the following requirements.

### 7.1.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS or it is the PRACH transmission. The reference point for the UE initial transmit timing control requirement shall be the downlink timing minus . The downlink timing is defined as the time when [the first detected path (in time)] of the corresponding downlink frame is received from the reference cell. NTA_Ref for PRACH is defined as 0.  (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in section 7.3 was applied. NTA_Ref for other channels is not changed until next timing advance is received.

Table 7.1.2-1: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 1.4 | 24*TS |
| ≥3 | 12*TS |
| Note: TS is the basic timing unit defined in TS 36.211 |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame except when the timing advance in section 7.3 is applied. When the transmission timing error between the UE and the reference timing exceeds Te  the UE is required to adjust its timing to within Te. The reference timing shall be  before the downlink timing. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq seconds.

2)	The minimum aggregate adjustment rate shall be 7*TS per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200ms.

where the maximum autonomous time adjustment step Tq is specified in Table 7.1.2-2.

Table 7.1.2-2: Tq Maximum Autonomous Time Adjustment Step

| Downlink Bandwidth (MHz) | Tq_ |
| --- | --- |
| 1.4 | 18.5*TS |
| 3 | 10.5*TS |
| 5 | 6.5*TS |
| 10 | 4.5*TS |
| Note: TS is the basic timing unit defined in TS 36.211 |  |

## << End of changes >>