TSG-RAN Working Group 4 (Radio) meeting #64                  R4-123853

Qingdao, China, 13 – 17 Aug, 2012

Source:	ZTE

Title:	On PBCH IC capability for FeICIC

Agenda item:	6.23.2

Document for:	Discussion

- Introduction

During RAN1#69, RAN1 discussed detection of system information in presence of dominant interferers with 9 dB handover bias. In that meeting, RAN1 has been discussing PBCH cancellation for Release 11 FeICIC and has sent an LS[1] to RAN2/RAN3/RAN4 and asking RAN4 whether it can be assumed that FeICIC capable UEs will always have PBCH interference cancelation capability.

In the RAN4#64 AH meeting, some contributions [2][3][4] provide preliminary analysis/results regarding the feasibility and performance of PBCH demodulation under interference cancellation (IC) but with different simulation assumptions, eg. number of interfering cell(s) and power level(s). No consensuses were reached on how to answer RAN1’s LS that Rel-11 baseline FeICIC receiver is assumed to be capable of PBCH interference cancellation from dominant interferers or not.

To alignment analysis/results, the group draft WF on feasibility criteria and simulation assumptions to align results and performance metric in [5]. In this paper, we run the link level simulation to evaluate PBCH cancellation based on the agreed working assumption in RAN4#64 AH meeting, the observation is that PBCH-IC can significantly improve the PBCH demodulation performance under FeICIC scenario or cannot meeting the requirement under 9dB CRE bias in Rel-11 FeICIC.(based on simulation results)

- Discussion

PBCH link level performance in terms of block error rate (BLER) vs. serving cell Es/Noc is investigated. The cases in table 1 are simulated to investigate performance of PBCH IC.

Table 1: Simulation Cases

| Case # | SNR for aggressor cell 1 (dB) | SNR for aggressor cell 2 (dB) | Cell ID (Serving cell, 1st aggressor cell, 2nd aggressor cell) |
| --- | --- | --- | --- |
| Case 1-1 | 5 | -∞ | (0,6) |
| Case 1-2 | 5 | -∞ | (0,1) |
| Case 2-1 | 5 | 1 | (0,6,2) |
| Case 2-2 | 5 | 1 | (0,1,2) |
| Case 3-1 | 6 | 3 | (0,6,2) |
| Case 3-2 | 6 | 3 | (0,1,2) |

The results for the PBCH interference cancellation are shown in Figure 1 to Figure 6, and the PBCH performance at 1% BLER for the above simulation cases is summarized in Table 2.

Figure 1: PBCH IC performance of Case 1-1: 1 interferer, 5dB, colliding CRS

Figure 2: PBCH IC performance of Case 1-2: 1 interferer, 5dB, non-colliding CRS

Figure 3: PBCH IC performance of Case 2-1: 2 interferers, (5, 1) dB, colliding CRS

Figure 4: PBCH IC performance of Case 2-2: 2 interferers, (5, 1) dB, non-colliding CRS

Figure 5: PBCH IC performance of Case 3-1: 2 interferers, (6, 3) dB, colliding CRS

Figure 6: PBCH IC performance of Case 3-2: 2 interferers, (6, 3) dB, non-colliding CRS

Table 2: Serving cell Es/Noc (dB) at 1% PBCH BLER and gain of PBCH IC

| Case # | Single cell | Without IC | With 1 cell IC | With 2 cells IC | gain |
| --- | --- | --- | --- | --- | --- |
| Case 1-1 | -5.8 | +1.45 | -1.6 | - | 3.05 |
| Case 1-2 | -5.8 | +2 | +0.3 | - | 1.7 |
| Case 2-1 | -5.8 | +2 | +0.2 | 0.4 | 1.8 |
| Case 2-2 | -5.8 | +2 | +2 | 1.9 | 0.1 |
| Case 3-1 | -5.8 | +3.1 | +1.7 | 1.4 | 1.7 |
| Case 3-2 | -5.8 | +3 | +3 | +3 | 0 |

It is observed from Figures 1-6 and Table 2 that PBCH performance with interference cancellation could not meet the Rel-8/9 requirement. The gain of PBCH IC is not obvious for all cases.

Proposals 1: PBCH IC capability should not be mandated and eNB signalling is needed to help the detection of PBCH for Rel-11 FeICIC.

- Conclusions

In this contribution, we provided link level simulation results for PBCH performance with/without IC. Observations have also been made.

Proposals 1: PBCH IC capability should not be mandated and eNB signalling is needed to help the detection of PBCH for Rel-11 FeICIC.

- References

- R1-123058, LS on MIB detection in FeICIC, RAN1

- R4-63AH-0010, On PBCH interference cancellation for FeICIC, Renesas Mobile Europe Ltd.

- R4-63AH-0163, PBCH interference cancellation, Ericsson, ST-Ericsson

- R4-63AH-0124, Response LS on MIB detection in FeICIC, Huawei, HiSilicon

- R4-63AH-0214, PBCH IC Simulation assumptions, Ericsson, ST-Ericsson , Huawei, HiSilicon

- Appendix

  - 5.1 Simulation assumptions

| Assumption | Value | Comment |
| --- | --- | --- |
| Number of interfering cells (N) | 0, 1, 2 | The final N for requirements, if the requirements are to be defined, is to be studied separately. |
| SNR for aggressor cell 1 (dB) | 6, 5, 3 |  |
| SNR for aggressor cell 2 (dB) | [3, 1, -∞] |  |
| Cell ID | (serving cell, 1st dominant interferer, 2nd dominant interferer) (0) (0, 1 , 2) (0, 6, 2) (0, 1) (0, 6) |  |
| Channel model | ETU, 30Hz |  |
| Carrier frequency | 2GHz |  |
| Antenna configuration | 2x2, low correlation |  |
| Subframe shifting | None |  |
| ABS configuration | Non ABS subframe |  |
| System bandwidth | 10MHz |  |
| Cyclic prefix | Normal |  |
| Power allocation (rhoA, rhoB) | -3dB |  |
| Serving cell SNR measured at CRS | -14 to 4dB, step size 1dB |  |
| Interference | Aggressor cell interference explicitly modeled |  |
| Tx EVM | 6% |  |
| Receiver | PBCH IC, PBCH no IC | CRS-IC should be performed at the same time. Companies encouraged to provide information on the cancellation principles (e.g. successive etc.) and equalizer used (e.g, MRC or IRC). |
| Simulation length | 40000 subframes minimum |  |
| Channel and interference estimation | Realistic |  |
| Aggressor PBCH decoding | Baseline: Practical Optional: Ideal |  |

The simulation outputs should be curves of BLER after 40msec PBCH decoding vs serving cell SNR for single cell, all single aggressor SNR values and all combinations of dual aggressor SNRs. The 1% BLER points should be noted.

  - 5.2 Criterion for feasibility of PBCH-IC

Two criteria should be used for deciding on the feasibility of PBCH IC

- SNR for 1% BLER PBCH decoding with IC falls within a typical operating range (to be defined)

- Gain in dB of PBCH IC compared with no IC at 1% BLER