TSG-RAN Working Group 4 (Radio) meeting #66	R4-130170

Malta, 28 January – 1 February 2013

Source:	TruePosition

Title:	Text Proposal for 36.111 Section 5 RF Requirements

Agenda item:	6.9.1

Document for:	Discussion and approval

# Introduction

In this contribution we propose to describe the LMU RF requirements in TS 36.111 section 5 as a set of tests that mirror section 7, Receiver Characteristics, of TS 36.104 [1] with UL-RTOA accuracy at a specified probability of detection as the requirement metric.

# Proposal

# 5	Receiver characteristics

## 5.1	General

The requirements in clause 5 are expressed for a single receiver antenna connector. For receivers with antenna diversity, the requirements apply for each receiver antenna connector.

Unless otherwise stated, the receiver characteristics are specified at the LMU antenna connector (test port A) in normal operating conditions. If any external apparatus such as a RX amplifier, a filter or the combination of such devices is used, requirements apply at the far end antenna connector (port B).

LMU

Test port A

Test port B

External

device

e.g.

RX filter

(if any

)

External

LNA

(if any)

From

antenna connector



Figure 5.1: Receiver test ports

When the LMU is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier. For ACS, blocking and intermodulation characteristics, the negative offsets of the interfering signal apply relative to the lower edge and positive offsets of the interfering signal apply relative to the higher edge.

## 5.2	Reference sensitivity level

The reference sensitivity power level PREFSENS is the minimum mean power received at the antenna connector at which an UL-RTOA accuracy with a specific probability of detection requirement shall be met for a specified reference measurement channel.

### 5.2.1	Minimum requirement

The LMU shall detect the reference measurement channel 90% of the time with a false alarm rate not greater than 1%.  The false alarm rate is the probability of detection noted when the SRS signal is turned off.  The reference measurement channel is specified in Annex A with parameters specified in Table 5.2.1-1.

Table 5.2.1-1: LMU reference sensitivity levels

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Reference sensitivity power level, PREFSENS [dBm] |
| --- | --- | --- |
| 1.4 | SRS with 4 RBs | -127.0 |
| 3 | SRS with 12 RBs | -127.0 |
| 5 | SRS with 24 RBs | -127.0 |
| 10 | SRS with 48 RBs | -127.0 |
| 15 | SRS with 72 RBs | -127.0 |
| 20 | SRS with 96 RBs | -127.0 |
| Note*: PREFSENS is the power level of a single instance of the reference measurement channel and each single instance of the SRS measured over the number of specified resource blocks. |  |  |

## 5.3	Dynamic range

The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal inside the received channel bandwidth. In this condition an UL-RTOA accuracy with a specific probability of detection requirement shall be met for a specified reference measurement channel. The interfering signal for the dynamic range requirement is an AWGN signal.

### 5.3.1	Minimum requirement

The LMU shall detect the reference measurement channel 90% of the time.  The reference measurement channel is specified in Annex A with parameters specified in Table 5.2.1-1.

Table 5.3.1-1: LMU dynamic range

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | SRS with 4 RBs | -93.0 | -73.7 | AWGN |
| 3 | SRS with 12 RBs | -93.0 | -69.7 | AWGN |
| 5 | SRS with 24 RBs | -93.0 | -67.5 | AWGN |
| 10 | SRS with 48 RBs | -93.0 | -64.5 | AWGN |
| 15 | SRS with 72 RBs | -93.0 | -62.5 | AWGN |
| 20 | SRS with 96 RBs | -93.0 | -61.4 | AWGN |

## 5.4	In-channel selectivity

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density. In this condition a UL-RTOA accuracy with a specific probability of detection requirement shall be met for a specified reference measurement channel. The interfering signal shall be an E-UTRA signal as specified in Annex C and shall be time aligned with the wanted signal.

5.4.1	Minimum requirement

The LMU shall detect the reference measurement channel 90% of the time.  The reference measurement channel is specified in Annex A with parameters specified in Table 5.2.1-1.

| E-UTRA channel bandwidth (MHz) / FFTSize | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | SRS with 4 RBs | PREFSENS + 3 dB | -87 | 1.4 MHz E-UTRA signal, 1 RB |
| 3 | SRS with 4 RBs | PREFSENS + 3 dB | -84 | 3 MHz E-UTRA signal, 6 RBs |
| 5 | SRS with 12 RBs | PREFSENS + 3 dB | -81 | 5 MHz E-UTRA signal, 10 RBs |
| 10 | SRS with 24 RBs | PREFSENS + 3 dB | -77 | 10 MHz E-UTRA signal, 25 RBs |
| 15 | SRS with 24 RBs | PREFSENS + 3 dB | -77 | 15 MHz E-UTRA signal, 25 RBs* |
| 20 | SRS with 48 RBs | PREFSENS + 3 dB | -77 | 20 MHz E-UTRA signal, 25 RBs* |
| Note: Wanted and interfering signal are placed adjacently around Fc |  |  |  |  |

Table 5.4.1-1 E-UTRA LMU in-channel selectivity

## 5.5  	Adjacent Channel Selectivity (ACS) and narrow-band blocking

Adjacent channel selectivity (ACS) is a measure of the receiver ability to receive a wanted signal at its assigned channel frequency in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system. The interfering signal shall be an E-UTRA signal as specified in Annex C.

### 5.5.1	Minimum requirement

The LMU shall detect the reference measurement channel 90% of the time.  The reference measurement channel is specified in Annex A with parameters specified in Table 5.2.1-1.

The wanted and the interfering signal coupled to the BS antenna input are specified in Tables 5.5.1-1 and 5.5.1-2 for narrowband blocking and in Table 5.5.1-3 for ACS. The reference measurement channel for the wanted signal is identified in Table 5.2.1-1 for each channel bandwidth and further specified in Annex A.

Table 5.5.1-1: Narrowband blocking requirement

| Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- |
| PREFSENS + 6dB | -49 | See Table 5.5.1-2 |

Table 5.5.1-2: Interfering signal for Narrowband blocking requirement

| E-UTRA channel BW of the lowest (highest) carrier received [MHz] | Interfering RB centre frequency offset to the lower (higher) edge [kHz] | Type of interfering signal |
| --- | --- | --- |
| 1.4 | ±(252.5+m*180), m=0, 1, 2, 3, 4, 5 | 1.4 MHz E-UTRA signal, 1 RB* |
| 3 | ±(247.5+m*180), m=0, 1, 2, 3, 4, 7, 10, 13 | 3 MHz E-UTRA signal, 1 RB* |
| 5 | ±(342.5+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz E-UTRA signal, 1 RB* |
| 10 | ±(347.5+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz E-UTRA signal, 1 RB* |
| 15 | ±(352.5+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz E-UTRA signal, 1 RB* |
| 20 | ±(342.5+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz E-UTRA signal, 1 RB* |
| Note*: Interfering signal consisting of one resource block is positioned at the stated offset; the channel bandwidth of the interfering signal is located adjacently to the lower (higher) edge. |  |  |  |

Table 5.5.1-3: LMU Adjacent channel selectivity

| E-UTRA channel bandwidth of the lowest (highest) carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset from the lower (higher) edge [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | PREFSENS + 8dB | -52 | ±0.7025 | 1.4MHz E-UTRA signal |
| 3 | PREFSENS + 7dB | -52 | ±1.5075 | 3MHz E-UTRA signal |
| 5 | PREFSENS + 5dB | -52 | ±2.5025 | 5MHz E-UTRA signal |
| 10 | PREFSENS + 3dB | -52 | ±2.5075 | 5MHz E-UTRA signal |
| 15 | PREFSENS + 3dB | -52 | ±2.5125 | 5MHz E-UTRA signal |
| 20 | PREFSENS + 3dB | -52 | ±2.5025 | 5MHz E-UTRA signal |

## 5.6	Blocking

### 5.6.1	General blocking requirement

The blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer, which are either a 1.4MHz, 3MHz or 5MHz E-UTRA signal for in-band blocking or a CW signal for out-of-band blocking. The interfering signal shall be an E-UTRA signal as specified in Annex C.

5.6.1.1	Minimum requirement

The LMU shall detect the reference measurement channel 90% of the time using the reference measurement channel, with a wanted and an interfering signal coupled to the LMU antenna input using the parameters in Table 5.6.1.1-1, 5.6.1.1-1a, 5.6.1.1-1b and 5.6.1.1-2. The reference measurement channel for the wanted signal is identified in Table 5.2.1-1 and 5.2.1-2 for each channel bandwidth and further specified in Annex A.

Table 5.6.1.1-1: LMU Blocking performance requirement

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower (higher) edge [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18,19, 21-23, 24, 33-43 | (FUL_low -20) | to | (FUL_high +20) | -43 | PREFSENS +6dB | See table 5.6.1.1-2 | See table 5.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 8 | (FUL_low -20) | to | (FUL_high +10) | -43 | PREFSENS +6dB | See table 5.6.1.1-2 | See table 5.6.1.1-2 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -43 | PREFSENS +6dB | See table 5.6.1.1-2 | See table 5.6.1.1-2 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -43 | PREFSENS +6dB | See table 5.6.1.1-2 | See table 5.6.1.1-2 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 20 | (FUL_low -11) | to | (FUL_high +20) | -43 | PREFSENS +6dB | See table 5.6.1.1-2 | See table 5.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +6dB |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -43 | PREFSENS +6dB | See table 5.6.1.1-2 | See table 5.6.1.1-2 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB |  | CW carrier |

NOTE:	Table 5.6.1.1-1 assumes that two operating bands, where the downlink operating band (see Table 5.5-1 of TS 36.104) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 5.6.1.1-2: Interfering signals for blocking performance requirement

| E-UTRA channel BW of the lowest (highest) carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower (higher) edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| 1.4 | ±2.1 | 1.4MHz E-UTRA signal |
| 3 | ±4.5 | 3MHz E-UTRA signal |
| 5 | ±7.5 | 5MHz E-UTRA signal |
| 10 | ±7.5 | 5MHz E-UTRA signal |
| 15 | ±7.5 | 5MHz E-UTRA signal |
| 20 | ±7.5 | 5MHz E-UTRA signal |

### 7.6.2	Co-location with other base stations

This additional blocking requirement may be applied for the protection of E-UTRA LMU receivers when GSM, CDMA, UTRA or E-UTRA BS operating in a different frequency band are co-located with an E-UTRA LMU. The requirement is applicable to all channel bandwidths supported by the E-UTRA LMU.

The requirements in this clause assume a 30 dB coupling loss between interfering transmitter and E-UTRA LMU receiver and are based on co-location with base stations of the same class as the LMU’s serving E-UTRA BS.

#### 7.6.2.1	Minimum requirement

The LMU shall detect the reference measurement channel 90% of the time using the reference measurement channel, with a wanted and an interfering signal coupled to LMU antenna input using the parameters in Table 5.6.2.1- The reference measurement channel for the wanted signal is identified in Table 5.2.1-1 and 5.2.1-2 for each channel bandwidth and further specified in Annex A.

Table 5.6.2.1-1: Blocking performance requirement for E-UTRA LMU when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Macro GSM850 or CDMA850 | 869 – 894 | +16 | PREFSENS + 6dB | CW carrier |
| Macro GSM900 | 921 – 960 | +16 | PREFSENS + 6dB | CW carrier |
| Macro DCS1800 | 1805 – 1880 | +16 | PREFSENS + 6dB | CW carrier |
| Macro PCS1900 | 1930 – 1990 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band I or E-UTRA Band 1 | 2110 – 2170 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band II or E-UTRA Band 2 | 1930 – 1990 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band III or E-UTRA Band 3 | 1805 – 1880 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band IV or E-UTRA Band 4 | 2110 – 2155 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band V or E-UTRA Band 5 | 869 – 894 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band VI or E-UTRA Band 6 | 875 – 885 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band VII or E-UTRA Band 7 | 2620 – 2690 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band VIII or E-UTRA Band 8 | 925 – 960 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 – 1879.9 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band X or E-UTRA Band 10 | 2110 – 2170 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 –1495.9 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band XII or E-UTRA Band 12 | 729 - 746 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band XIIII or E-UTRA Band 13 | 746 - 756 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band XIV or E-UTRA Band 14 | 758 - 768 | +16 | PREFSENS + 6dB | CW carrier |
| WA E-UTRA Band 17 | 734 - 746 | +16 | PREFSENS + 6dB | CW carrier |
| WA E-UTRA Band 18 | 860 - 875 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band XIX or E-UTRA Band 19 | 875 - 890 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band XX or E-UTRA Band 20 | 791 - 821 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band XXI or E-UTRA Band 21 | 1495.9 – 1510.9 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band XXII or E-UTRA Band 22 | 3510 – 3590 | +16 | PREFSENS + 6dB | CW carrier |
| WA E-UTRA Band 23 | 2180 - 2200 | +16 | PREFSENS + 6dB | CW carrier |
| WA E-UTRA Band 24 | 1525 – 1559 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA FDD Band XXV or E-UTRA Band 25 | 1930 – 1995 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA TDD Band a) or E-UTRA Band 33 | 1900-1920 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA TDD Band a) or E-UTRA TDD Band 34 | 2010-2025 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA TDD Band b) or E-UTRA Band 35 | 1850-1910 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA TDD Band b) or E-UTRA Band 36 | 1930-1990 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA TDD Band c) or E-UTRA Band 37 | 1910-1930 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA TDD Band d) or E-UTRA Band 38 | 2570-2620 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA TDD Band f) or E-UTRA Band 39 | 1880-1920 | +16 | PREFSENS + 6dB | CW carrier |
| WA UTRA TDD Band e) or E-UTRA Band 40 | 2300-2400 | +16 | PREFSENS + 6dB | CW carrier |
| WA E-UTRA Band 41 | 2496 - 2690 | +16 | PREFSENS + 6dB | CW carrier |
| WA E-UTRA Band 42 | 3400-3600 | +16 | PREFSENS + 6dB | CW carrier |
| WA E-UTRA Band 43 | 3600-3800 | +16 | PREFSENS + 6dB | CW carrier |
| Note*: PREFSENS depends on the channel bandwidth as specified in Table 7.2.1-1. |  |  |  |  |
| NOTE 1: Except for a LMU operating in Band 13, these requirements do not apply when the interfering signal falls within the uplink operating band or in the 10 MHz immediately outside the uplink operating band. For a LMU operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. For LMU operating in Band 42 or 43, the requirements do not apply when the interfering signal falls within the Band 42 or 43 uplink operating bands and the serving Base Stations are synchronized. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD with E-UTRA FDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [8]. |  |  |  |  |

## 5.7	Receiver spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the LMU receiver antenna connector.

For an LMU installed with a TDD BS with common RX and TX antenna port the requirement applies during the Transmitter OFF period. For an LMU installed with a FDD BS with common RX and TX antenna port the transmitter spurious emission as specified in clause 6.6.4[TBD] is valid.

### 5.7.1	Minimum requirement

The power of any spurious emission shall not exceed the levels in Table 5.7.1-1:

Table 5.7.1-1: General spurious emission minimum requirement

| Frequency range | Maximum level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30MHz - 1 GHz | -57 dBm | 100 kHz |  |
| 1 GHz – 12.75 GHz | -47 dBm | 1 MHz |  |
| 12.75 GHz - 5th harmonic of the upper frequency edge of the UL operating band in GHz | -47 dBm | 1 MHz | Applies only for Bands 22, 42 and 43. |
| NOTE: The frequency range between 2.5 * BWChannel below the first carrier frequency and 2.5 * BWChannel above the last carrier frequency transmitted by the BS, where BWChannel is the channel bandwidth according to Table 5.6-1, may be excluded from the requirement. However, frequencies that are more than 10 MHz below the lowest frequency of the BS downlink operating band or more than 10 MHz above the highest frequency of the BS downlink operating band shall not be excluded from the requirement. |  |  |  |

In addition to the requirements in Table 5.7.1-1, the power of any spurious emission shall not exceed the levels specified for Protection of the E-UTRA FDD BS receiver of own or different BS in subclause 6.6.4.2 [TBD] and for Co-existence with other systems in the same geographical area in subclause 6.6.4.3 [TBD]. In addition, the co-existence requirements for co-located base stations specified in subclause 6.6.4.4 [TBD] may also be applied.

## 5.8	Receiver intermodulation

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel.  Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal. Interfering signals shall be a CW signal and an E-UTRA signal as specified in Annex C.

### 5.8.1	Minimum requirement

The LMU shall detect the reference measurement channel 90% of the time using the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals coupled to the LMU antenna input, with the conditions specified in Tables 5.8.1-1 and 5.8.1-2 for intermodulation performance and in Table 5.8.1-3, 5.8.1-4 and Table 5.8.1-5 for narrowband intermodulation performance. The reference measurement channel for the wanted signal is identified in Table 5.2.1-1, Table 5.2.1-2 and Table 5.2.1-3 for each channel bandwidth and further specified in Annex A.

Table 5.8.1-1: Intermodulation performance requirement

| Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- |
| PREFSENS + 6dB* | -52 | See Table 5.8.1-2 |

Table 5.8.1-2: Interfering signal for Intermodulation performance requirement

| E-UTRA channel bandwidth of the lowest (highest) carrier received [MHz] | Interfering signal centre frequency offset from the lower (higher) edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| 1.4 | ±2.1 | CW |
|  | ±4.9 | 1.4MHz E-UTRA signal |
| 3 | ±4.5 | CW |
|  | ±10.5 | 3MHz E-UTRA signal |
| 5 | ±7.5 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| 10 | ±7. 375 | CW |
|  | ±17. 5 | 5MHz E-UTRA signal |
| 15 | ±7. 25 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| 20 | ±7. 125 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |

Table 5.8.1-3: Narrowband intermodulation performance requirement for LMU

| E-UTRA channel bandwidth of the lowest (highest) carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower (higher) edge [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | PREFSENS + 6dB | -52 | ±270 | CW |
|  |  | -52 | ±790 | 1.4 MHz E-UTRA signal, 1 RB** |
| 3 | PREFSENS + 6dB | -52 | ±270 | CW |
|  |  | -52 | ±780 | 3.0 MHz E-UTRA signal, 1 RB** |
| 5 | PREFSENS + 6dB | -52 | 360 | CW |
|  |  | -52 | ±1060 | 5 MHz E-UTRA signal, 1 RB** |
| 10 | PREFSENS + 6dB (***) | -52 | ±325 | CW |
|  |  | -52 | ±1240 | 5 MHz E-UTRA signal, 1 RB** |
| 15 | PREFSENS + 6dB (***) | -52 | ±380 | CW |
|  |  | -52 | ±1600 | 5MHz E-UTRA signal, 1 RB** |
| 20 / 2048 | PREFSENS + 6dB (***) | -52 | ±345 | CW |
|  |  | -52 | ±1780 | 5MHz E-UTRA signal, 1 RB** |
| Note**: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower (higher) edge. Note***: This requirement shall apply only for a FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals |  |  |  |  |

# Reference Measurement Channels

Table A-1: SRS Configuration for all except in-channel selectivity

| Table A-1. Reference Measurement Channel SRS Configuration Parameters |  |  |
| --- | --- | --- |
| Group hopping | disabled |  |
| Sequence hopping | disabled |  |
| NIDcell | 1 |  |
|  | 0 |  |
| cyclicShift | 0 |  |
| SRS-Bandwidth | 0 |  |
| SRS Bandwidth Configuration | Channel BW (MHz) | Csrs |
|  | 1.4 | 7 |
|  | 3 | 5 |
|  | 5 | 0 |
|  | 10 | 0 |
|  | 15 | 0 |
|  | 20 | 0 |

Table A-2: SRS Configuration for in-channel selectivity

| Table A-2. Simulation SRS Configuration Parameters for In-Channel Selectivity |  |  |
| --- | --- | --- |
| Group hopping | disabled |  |
| Sequence hopping | disabled |  |
| NIDcell | 0 |  |
|  | 0 |  |
| cyclicShift | 0 |  |
| SRS-Bandwidth | Channel BW (MHz) | Bsrs |
|  | 1.4 | 0 |
|  | 3 | 0 |
|  | 5 | 0 |
|  | 10 | 0 |
|  | 15 | 1 |
|  | 20 | 0 |
| SRS Bandwidth Configuration | Channel BW (MHz) | Csrs |
|  | 1.4 | 7 |
|  | 3 | 7 |
|  | 5 | 5 |
|  | 10 | 5 |
|  | 15 | 3 |
|  | 20 | 7 |

# Conclusion

In this contribution we propose to structure the LMU RF requirements as a set of tests that mirror the RF requirements in section 7[1] using the probability of detection of 90% as the conformance metric.

# References

- TS 36.104 “Base Station (BS) radio transmission and reception”

- R4-130168 “UTDOA Simulation Results (TS 36.111 Section 5 LMU RF Req)”