3GPP TSG-RAN WG4 Meeting #67  	                                    R4-132483

Fukuoka, Japan, 20 - 24 May, 2013

Agenda Item:	7.1.2

Source:	Samsung

Title:	Realistic Performance Evaluation of PUSCH and PUCCH with 1 Tx Antenna and 8 Rx Antennas

Document for:	Discussion

# Introduction

In RAN4 #66 meeting, the simulation assumption and test cases for PUSCH [1] and PUCCH format 1a [2] have decided. In RAN4#66bis meeting, first round of ideal result alignments was committed and companies are encouraged to submit impairment simulation results in this RAN4#67 meeting[3, 4]. This contribution presents the realistic results (with implementation margin) accordingly.

# PUSCH Performance for UL 8Rx with 1Tx

## PUSCH Simulation assumption

Table 1 provides the simulation assumptions for PUSCH in multipath fading propagation conditions [1].

Table 1	Simulation assumptions

| Parameters | Value |
| --- | --- |
| Propagation conditions | EVA, EPA, ETU |
| Antenna configuration | 1x8 |
| Correlation matrix for low correlation |  |
| Noise Model | AWGN |
| Channel Bandwidth* | 1.4MHz, 3MHz, 5MHz, 10 MHz, 15 MHz, 20 MHz |
| Channel estimation | Practical and realizable channel and noise estimates |
| Cyclic prefix | Normal, extended |
| Resource allocation | Full bandwidth allocation, single PRB allocation |
| Modulation scheme and code rate | QPSK 1/3, 16QAM 3/4, 64QAM 5/6 |
| Number of HARQ processes | 8 HARQ processes for FDD |
| Maximum number of HARQ transmissions | 4 |
| Redundancy version sequence | 0, 2, 3, 1, 0, 2, 3, 1 |
| HARQ combining | Incremental redundancy |
| Simulation length | 10000 subframes at minimum |

Note: * The simulation results of 5MHz, 10MHz, 15MHz and 20MHz are considered with higher priority for ideal result alignments in RAN4 #66bis meeting.

## Simulation Results of the PUSCH Test Cases

Table 2 ~ Table 7 provide the realistic results of test cases for 1Tx 8Rx PUSCH demodulation performance requirements in multipath fading propagation conditions.

Table 2 Minimum requirements for PUSCH, 1.4 MHz Channel Bandwidth, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix | FRC [6] | Fraction of maximum throughput | SNR [5] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | Normal | EPA 5Hz Low | A3-2 | 30% | -9.5 |
|  |  |  |  |  | 70% | -6.3 |
|  |  |  |  | A4-3 | 70% | 4.4 |
|  |  |  |  | A5-2 | 70% | 11.7 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -7.0 |
|  |  |  |  |  | 70% | -3.8 |
|  |  |  |  | A4-1 | 30% | -1.5 |
|  |  |  |  |  | 70% | 5.1 |
|  |  |  |  | A5-1 | 70% | 12.3 |
|  |  |  | EVA 70Hz Low | A3-2 | 30% | -9.4 |
|  |  |  |  |  | 70% | -5.7 |
|  |  |  |  | A4-3 | 30% | -2.1 |
|  |  |  |  |  | 70% | 5.3 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -6.8 |
|  |  |  |  |  | 70% | -3.4 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -6.6 |
|  |  |  |  |  | 70% | -2.9 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | -0.8 |
|  |  |  |  |  | 70% | 6.9 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |

Table 3 Minimum requirements for PUSCH, 3 MHz Channel Bandwidth, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix | FRC [6] | Fraction of maximum throughput | SNR [5] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | Normal | EPA 5Hz Low | A3-3 | 30% | -9.7 |
|  |  |  |  |  | 70% | -6.4 |
|  |  |  |  | A4-4 | 70% | 4.5 |
|  |  |  |  | A5-3 | 70% | 11.9 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -6.7 |
|  |  |  |  |  | 70% | -3.5 |
|  |  |  |  | A4-1 | 30% | -1.2 |
|  |  |  |  |  | 70% | 5.4 |
|  |  |  |  | A5-1 | 70% | 12.6 |
|  |  |  | EVA 70Hz Low | A3-3 | 30% | -9.6 |
|  |  |  |  |  | 70% | -5.9 |
|  |  |  |  | A4-4 | 30% | -2.1 |
|  |  |  |  |  | 70% | 5.4 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -6.8 |
|  |  |  |  |  | 70% | -3.2 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -6.5 |
|  |  |  |  |  | 70% | -2.8 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | -0.7 |
|  |  |  |  |  | 70% | 7.0 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |

Table 4 Minimum requirements for PUSCH, 5 MHz Channel Bandwidth, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions a and correlation matrix | FRC [6] | Fraction of maximum throughput | SNR [5] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | Normal | EPA 5Hz Low | A3-4 | 30% | -10.3 |
|  |  |  |  |  | 70% | -7.1 |
|  |  |  |  | A4-5 | 70% | 4.5 |
|  |  |  |  | A5-4 | 70% | 12.0 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -7.0 |
|  |  |  |  |  | 70% | -3.8 |
|  |  |  |  | A4-1 | 30% | -1.5 |
|  |  |  |  |  | 70% | 5.0 |
|  |  |  |  | A5-1 | 70% | 12.3 |
|  |  |  | EVA 70Hz Low | A3-4 | 30% | -10.2 |
|  |  |  |  |  | 70% | -6.6 |
|  |  |  |  | A4-5 | 30% | -2.2 |
|  |  |  |  |  | 70% | 5.3 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -6.8 |
|  |  |  |  |  | 70% | -3.3 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -6.6 |
|  |  |  |  |  | 70% | -2.9 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | -0.8 |
|  |  |  |  |  | 70% | 7.0 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |

Table 5 Minimum requirements for PUSCH, 10 MHz Channel Bandwidth, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix | FRC [6] | Fraction of maximum throughput | SNR [5] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | Normal | EPA 5Hz Low | A3-5 | 30% | -9.9 |
|  |  |  |  |  | 70% | -6.6 |
|  |  |  |  | A4-6 | 70% | 4.6 |
|  |  |  |  | A5-5 | 70% | 12.1 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -6.8 |
|  |  |  |  |  | 70% | -3.6 |
|  |  |  |  | A4-1 | 30% | -1.3 |
|  |  |  |  |  | 70% | 5.3 |
|  |  |  |  | A5-1 | 70% | 12.6 |
|  |  |  | EVA 70Hz Low | A3-5 | 30% | -9.8 |
|  |  |  |  |  | 70% | -6.0 |
|  |  |  |  | A4-6 | 30% | -2.2 |
|  |  |  |  |  | 70% | 5.4 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -6.8 |
|  |  |  |  |  | 70% | -3.3 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -6.6 |
|  |  |  |  |  | 70% | -2.9 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | -0.8 |
|  |  |  |  |  | 70% | 7.0 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |

Table 6 Minimum requirements for PUSCH, 15 MHz Channel Bandwidth, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix | FRC [6] | Fraction of maximum throughput | SNR [5] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | Normal | EPA 5Hz Low | A3-6 | 30% | -10.0 |
|  |  |  |  |  | 70% | -6.8 |
|  |  |  |  | A4-7 | 70% | 5.3 |
|  |  |  |  | A5-6 | 70% | 12.7 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -7.1 |
|  |  |  |  |  | 70% | -3.8 |
|  |  |  |  | A4-1 | 30% | -1.6 |
|  |  |  |  |  | 70% | 5.0 |
|  |  |  |  | A5-1 | 70% | 12.3 |
|  |  |  | EVA 70Hz Low | A3-6 | 30% | -10.1 |
|  |  |  |  |  | 70% | -6.5 |
|  |  |  |  | A4-7 | 30% | -2.0 |
|  |  |  |  |  | 70% | 5.8 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -6.9 |
|  |  |  |  |  | 70% | -3.4 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -6.6 |
|  |  |  |  |  | 70% | -2.9 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | -0.9 |
|  |  |  |  |  | 70% | 6.9 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |

Table 7 Minimum requirements for PUSCH, 20 MHz Channel Bandwidth, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix | FRC | Fraction of maximum throughput | SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | Normal | EPA 5Hz Low | A3-7 | 30% | -9.7 |
|  |  |  |  |  | 70% | -6.2 |
|  |  |  |  | A4-8 | 70% | 5.4 |
|  |  |  |  | A5-7 | 70% | 12.7 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -7.0 |
|  |  |  |  |  | 70% | -3.8 |
|  |  |  |  | A4-1 | 30% | -1.5 |
|  |  |  |  |  | 70% | 5.1 |
|  |  |  |  | A5-1 | 70% | 12.3 |
|  |  |  | EVA 70Hz Low | A3-7 | 30% | -9.7 |
|  |  |  |  |  | 70% | -5.9 |
|  |  |  |  | A4-8 | 30% | -2.0 |
|  |  |  |  |  | 70% | 5.8 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -6.8 |
|  |  |  |  |  | 70% | -3.3 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -6.6 |
|  |  |  |  |  | 70% | -2.9 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | -0.8 |
|  |  |  |  |  | 70% | 7.1 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |

# PUCCH Performance for UL 8Rx with 1Tx

## PUCCH Simulation assumption

Table 8 provides the simulation assumptions for single user PUCCH format 1a [2].

Table 8	PUCCH format 1a simulation assumptions

| Parameters | Value |
| --- | --- |
| Propagation conditions | EVA, EPA, ETU |
| Antenna configuration | 1x8 |
| Correlation matrix for low correlation |  |
| Noise Model | AWGN |
| Channel Bandwidth* | 1.4MHz, 3MHz, 5MHz, 10 MHz, 15 MHz, 20 MHz |
| Noise model | AWGN |
| Cyclic prefix | Normal, extended |
| Channel estimation | ML channel estimator with real noise estimation |
| Frequency domain equalizer | MMSE |
| Number of users | 1 |

Note: * The simulation results of 5MHz, 10MHz, 15MHz and 20MHz are considered with higher priority for ideal result alignments in RAN4 #66bis meeting.

## Simulation Results of the PUCCH Test cases

Table 9 provides realistic results of test cases for ACK missed detection requirements for single user PUCCH format 1a.

Table 9 Minimum requirements for single user PUCCH format 1a, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix | Channel Bandwidth / SNR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 1 | 8 | Normal | EPA 5 Low | -11.1 | -11.4 | -11.8 | -12 | -12.1 | -12 |
|  |  |  | EVA 5 Low | -11.7 | -11.8 | -11.9 | -11.8 | -12 | -11.9 |
|  |  |  | EVA 70 Low | -11.7 | -11.8 | -12 | -11.8 | -11.9 | -11.9 |
|  |  |  | ETU 300* Low | -11.4 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 |
|  |  | Extended | ETU 70* Low | -10.3 | -10.5 | -10.5 | -10.6 | -10.5 | -10.6 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |  |  |  |

# Reference

[1]	R4-130832, Simulation assumptions for PUSCH with 1Tx 8Rx. China Telecom,3GPP TSG-RAN WG4 Meeting #66,Malta, MT, 28 Jan - 1 Feb 2013.

[2]	R4-130833, Simulation assumptions for PUCCH with 1Tx 8Rx China Telecom,3GPP TSG-RAN WG4 Meeting #66,Malta, MT, 28 Jan - 1 Feb 2013.

[3]	R4-131889 and H. China Telecom, Work plan on performance requirements for UL 8 Rx antennas (version 2). 3GPP TSG-RAN WG4 meeting #66bis, Chicago, IL, US, Apr 15 - 19 , 2013.

[4]	R4-131894 and C.T. ZTE, LTE UL 8Rx ad-hoc agenda/minutes. 3GPP TSG-RAN WG4 meeting #66bis, Chicago, IL, US, Apr 15 - 19 , 2013.