3GPP TSG-RAN WG4 Meeting #67	R4-132724

# Fukuoka, Japan, 20 - 24 May, 2013

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.104 | CR | 655 | rev | - | Current version: | 11.5.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of BS performance requirements for HSUPA MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MIMO_64QAM_HSUPA-Perf |  |  |  |  |  |  | Date: |  | 2013-05-06 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | MIMO with 64QAM for HSUPA core part WI has been completed. Appropriate BS performance requirements have to be introduced to ensure correct functionality of the new feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | BS performance requirements are introduced for HSUPA MIMO transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | BS performance requirements are not introduced for HSUPA MIMO transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1; 8.11A; A.9; A.18; A.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

# 8	Performance requirement

## 8.1	General

Performance requirements for the BS are specified for the measurement channels defined in Annex A and the propagation conditions in Annex B. The requirements only apply to those measurement channels that are supported by the base station. For FRC8 in Annex 9 and Annex 17 the Non E-DPCCH boosting and E-DPCCH boosting requirement only apply for the option supported by the base station. The performance requirements for the high speed train conditions which scenarios defined in Annex B.4A are optional.

Unless stated otherwise, performance requirements apply for a single cell only. Performance requirements for a BS supporting DC-HSUPA are defined in terms of single carrier requirements. The requirements in clause 8 shall be met with the transmitter on.

NOTE:	In normal operating conditions the BS is configured to transmit and receive at the same time. The transmitter may be off for some of the tests as specifed in 25.141.

For BS with dual receiver antenna diversity, only the BS performance requirements with Rx diversity apply, the required Eb/N0 shall be applied separately at each antenna port.

For BS without receiver antenna diversity, only the BS performance requirements without Rx diversity apply, the required Eb/N0 shall be applied at the BS Rx antenna port.

The Eb/N0 used in this section is defined as:

Where:

is the received total energy of DPDCH, DPCCH, S-DPCCH, HS-DPCCH, E-DPDCH, S-E-DPDCH, E-DPCCH and S-E-DPCCH per PN chip per antenna from all paths.

is the total one-sided noise power spectral density due to all noise sources

is the number of chips per frame

is the number of information bits in DTCH excluding CRC bits per frame

# Table 8.1: Summary of Base Station performance targets

| Physical channel | Measurement channel | Static | Multi-path Case 1 | Multi-path Case 2* | Multi-path Case 3* | Moving* | Birth / Death* |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Performance metric |  |  |  |  |  |
| DCH | 12.2 kbps | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER< | BLER< |
|  | 64 kbps | BLER< 10-1,10-2 | BLER< 10-1, 10-2 | BLER< 10-1,10-2 | BLER< 10-1, 10-2,10-3 | BLER< | BLER< |
|  | 144 kbps | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1, 10-2,10-3 | - | - |
|  | 384 kbps | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1, 10-2,10-3 | - | - |
| * Not applicable for Home BS |  |  |  |  |  |  |  |

<Next change>

## 8.11	Demodulation of E-DPDCH in multipath fading condition

The performance requirement of the E-DPDCH in multi path fading condition is determined by the minimum throughput, R. For the test parameters specified in Table 8.18, the minimum requirements are specified in Table 8.19. For a BS supporting DC-HSUPA the requirements for FRC1, FRC2, FRC3 and FRC8 shall apply on each cell.

# Table 8.18: Test parameters for testing E-DPDCH

| Parameter | Unit | Test |  |
| --- | --- | --- | --- |
| RSN |  | {0, 1, 2, 3} |  |
| HARQ combining |  | IR |  |
| Maximum number of HARQ transmission |  | 4 |  |
| Power control |  | OFF |  |
| DPCCH slot format |  | FRC8 or BS supporting DC-HSUPA | 1 |
|  |  | otherwise | 0 |
| E-DPCCH # code words |  | 1024, no optimization based on prior knowledge of valid code words. |  |
| Physical channels to be turned on |  | DPCCH, E-DPDCH and E-DPCCH |  |

# Table 8.19 Minimum Requirement for E-DPDCH

| Fixed Reference Channel |  | Reference value, EC/N0 (dB), for R ≥ 30% and R ≥ 70% of maximum information bit rate |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation conditions |  | FRC1 | FRC2 | FRC3 | FRC4 | FRC5 | FRC6 | FRC7 | FRC8 |  |
|  |  |  |  |  |  |  |  |  | Non E-DPCCH boosting | E-DPCCH Boosting |
| Pedestrian A without RX diversity | 30% | -2.4 | 0.8 | 2.4 | -7.1 | -4.4 | -1.4 | -15.0 | NA | NA |
|  | 70% | 3.7 | 7.1 | 9.1 | -0.6 | 2.1 | 5.2 | -8.4 | 16.2 | 16.9 |
| Pedestrian A with RX diversity | 30% | -6.2 | -3.1 | -1.4 | -10.6 | -8.0 | -5.0 | -18.3 | NA | NA |
|  | 70% | -1.0 | 2.2 | 4.1 | -5.2 | -2.6 | 0.2 | -13.3 | 10.1 | 10.4 |
| Pedestrian B without RX diversity* | 30% | -2.5 | 1.1 | 3.5 | -7.5 | -4.7 | -1.3 | -13.6 | NA | NA |
|  | 70% | 3.9 | NA | NA | -2.1 | 0.9 | 5.3 | -10.1 | NA | NA |
| Pedestrian B with RX diversity* | 30% | -6.1 | -3.1 | -1.0 | -10.7 | -8.1 | -4.9 | -18.0 | NA | NA |
|  | 70% | -0.3 | 3.9 | 8.2 | -5.7 | -2.9 | 0.7 | -13.8 | 12.4 | 13.1 |
| Vehicular 30 without RX diversity* | 30% | -2.5 | 1.0 | 3.2 | -7.5 | -4.6 | -1.4 | -14.3 | NA | NA |
|  | 70% | 4.9 | NA | NA | -1.7 | 1.4 | 5.8 | -10.1 | NA | NA |
| Vehicular 30 with RX diversity* | 30% | -6.1 | -2.9 | -0.9 | -10.7 | -8.0 | -4.9 | -17.6 | NA | NA |
|  | 70% | 0.6 | 4.7 | 8.8 | -5.4 | -2.6 | 1.0 | -13.7 | 13.3 | 13.6 |
| Vehicular 120 without RX diversity* | 30% | -2.1 | 1.3 | 3.6 | -7.3 | -4.2 | -1.2 | -14.0 | NA | NA |
|  | 70% | 5.1 | NA | NA | -1.3 | 1.5 | 6.1 | -10.1 | NA | NA |
| Vehicular 120 with RX diversity* | 30% | -5.7 | -2.6 | -0.5 | -10.4 | -7.6 | -4.3 | -17.0 | NA | NA |
|  | 70% | 0.7 | 5.0 | 9.5 | -5.1 | -2.3 | 1.2 | -13.2 | NA | NA |
| * Not applicable for Home BS |  |  |  |  |  |  |  |  |  |  |

## 8.11A	Demodulation of E-DPDCH and S-E-DPDCH in multipath fading condition for UL MIMO

The performance requirements of the E-DPDCH and S-E-DPDCH in multi path fading condition for UL MIMO are determined by the minimum throughput, R. For the test parameters specified in Table 8.18A, the minimum requirements are specified in Table 8.19A.

# Table 8.18A: Test parameters for testing E-DPDCH and S-E-DPDCH for UL MIMO

| Parameter |  | Test |  |
| --- | --- | --- | --- |
| RSN |  | {0, 1, 2, 3} |  |
| HARQ combining |  | IR |  |
| Maximum number of HARQ transmission |  | 4 |  |
| Power control |  | OFF |  |
| TX weight vector selection | A fixed precoding weight vector |
| Physical channels to be turned on |  | DPCCH, S-DPCCH, E-DPCCH, S-E-DPCCH, E-DPDCH and S-E-DPDCH |  |

# Table 8.19A: Minimum Requirements for E-DPDCH and S-E-DPDCH for UL MIMO

| Fixed Reference Channel | Reference value, EC/N0 (dB), for R ≥ 70% of maximum information bit rate |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Propagation conditions | FRC9 |  |  | FRC10 |  |  |
|  | E-DPDCH | S-E-DPDCH | total | E-DPDCH | S-E-DPDCH | total |
| Ped A, 3 km/h | TBD | TBD | TBD | TBD | TBD | TBD |
| Ped B, 3 km/h | TBD | TBD | TBD | TBD | TBD | TBD |
| Veh A, 3 km/h | TBD | TBD | TBD | TBD | TBD | TBD |

<Next change>

# A.9	Summary of E-DPDCH Fixed reference channels

# Table A.9

| Fixed Ref Channel | TTI [ms] | NINF | SF1 | SF2 | SF3 | SF4 | NBIN | Coding rate | Max inf bit rate [kbps] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FRC1 | 2 | 2706 | 4 | 4 | 0 | 0 | 3840 | 0.705 | 1353.0 |
| FRC2 | 2 | 5412 | 2 | 2 | 0 | 0 | 7680 | 0.705 | 2706.0 |
| FRC3 | 2 | 8100 | 2 | 2 | 4 | 4 | 11520 | 0.703 | 4050.0 |
| FRC4 | 10 | 5076 | 4 | 0 | 0 | 0 | 9600 | 0.529 | 507.6 |
| FRC5 | 10 | 9780 | 4 | 4 | 0 | 0 | 19200 | 0.509 | 978.0 |
| FRC6 | 10 | 19278 | 2 | 2 | 0 | 0 | 38400 | 0.502 | 1927.8 |
| FRC7 | 10 | 690 | 16 | 0 | 0 | 0 | 2400 | 0.288 | 69.0 |
| FRC8 | 2 | 16218 | 2 | 2 | 4 | 4 | 23040 | 0.704 | 8109.0 |
| FRC9 | 2 | 16200 | 2 | 2 | 4 | 4 | 23040 | 0.703 | 8100.0 |
| FRC10 | 2 | 32436 | 2 | 2 | 4 | 4 | 46080 | 0.704 | 16218.0 |

# A.10	E-DPDCH Fixed reference channel 1 (FRC1)

# Table A.10

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 1353.0 |
| TTI | ms | 2 |
| Number of HARQ Processes | Processes | 8 |
| Information Bit Payload (NINF) | Bits | 2706 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 3840 |
| Coding Rate (NINF/ NBIN) |  | 0.705 |
| Physical Channel Codes | SF for each physical channel | {4,4} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 8.94 Non-diversity: 12.04 Diversity: 2.05 Non-diversity: 6.02 E-DPDCH /DPCCH power ratio is calculated for a single E-DPDCH. |
| E-DPCCH missed detection testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 8.94 Non-diversity: 12.04 Diversity: -1.94 Non-diversity: 0.00 |

# Figure A.10

# A.11	E-DPDCH Fixed reference channel 2 (FRC2)

# Table A.11

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 2706.0 |
| TTI | ms | 2 |
| Number of HARQ Processes | Processes | 8 |
| Information Bit Payload (NINF) | Bits | 5412 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 7680 |
| Coding Rate (NINF/ NBIN) |  | 0.705 |
| Physical Channel Codes | SF for each physical channel | {2,2} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 9.92 Non-diversity: 13.00 Diversity: 4.08 Non-diversity: 6.02 E-DPDCH /DPCCH power ratio is calculated for a single E-DPDCH. |

# Figure A.11

# A.12	E-DPDCH Fixed reference channel 3 (FRC3)

# Table A.12

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 4050.0 |
| TTI | ms | 2 |
| Number of HARQ Processes | Processes | 8 |
| Information Bit Payload (NINF) | Bits | 8100 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 11520 |
| Coding Rate (NINF/ NBIN) |  | 0.703 |
| Physical Channel Codes | SF for each physical channel | {2,2,4,4} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 6.02 Non-diversity: 8.94 Diversity: 0.0 Non-diversity: 2.05 E-DPDCH/DPCCH power ratio is calculated for a single E-DPDCH with SF 4. The power of an E-DPDCH with SF2 is twice that of an E-DPDCH with SF4. |

# Figure A.12

# A.13	E-DPDCH Fixed reference channel 4 (FRC4)

# Table A.13

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 507.6 |
| TTI | ms | 10 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload (NINF) | Bits | 5076 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 9600 |
| Coding Rate (NINF/ NBIN) |  | 0.529 |
| Physical Channel Codes | SF for each physical channel | {4} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 8.94 Non-diversity: 12.04 Diversity: -1.94 Non-diversity: 0.0 |
| E-DPCCH missed detection testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 8.94 Non-diversity: 12.04 Diversity: -7.96 Non-diversity: -5.46 |

# Figure A.13

# A.14	E-DPDCH Fixed reference channel 5 (FRC5)

# Table A.14

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 978.0 |
| TTI | ms | 10 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload (NINF) | Bits | 9780 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 19200 |
| Coding Rate (NINF/ NBIN) |  | 0.509 |
| Physical Channel Codes | SF for each physical channel | {4,4} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 8.94 Non-diversity: 12.04 Diversity: -1.94 Non-diversity: 0.0 E-DPDCH /DPCCH power ratio is calculated for a single E-DPDCH. |

# Figure A.14

# A.15	E-DPDCH Fixed reference channel 6 (FRC6)

# Table A.15

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 1927.8 |
| TTI | ms | 10 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload (NINF) | Bits | 19278 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 38400 |
| Coding Rate (NINF/ NBIN) |  | 0.502 |
| Physical Channel Codes | SF for each physical channel | {2,2} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 9.92 Non-diversity: 13.00 Diversity: -5.46 Non-diversity: -1.94 E-DPDCH /DPCCH power ratio is calculated for a single E-DPDCH. |

# Figure A.15

# A.16	E-DPDCH Fixed reference channel 7 (FRC7)

# Table A.16

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 69.0 |
| TTI | ms | 10 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload (NINF) | Bits | 690 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 2400 |
| Coding Rate (NINF/ NBIN) |  | 0.288 |
| Physical Channel Codes | SF for each physical channel | {16} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 6.02 Non-diversity: 8.94 Diversity: 0.0 Non-diversity: 4.08 |

# Figure A.16

# A.17	E-DPDCH Fixed reference channel 8 (FRC8)

# Table A.17

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation |  | 16QAM |
| Maximum. Inf. Bit Rate | kbps | 8109.0 |
| TTI | ms | 2 |
| Number of HARQ Processes | Processes | 8 |
| Information Bit Payload (NINF) | Bits | 16218 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 23040 |
| Coding Rate (NINF/ NBIN) |  | 0.704 |
| Physical Channel Codes | SF for each physical channel | {2,2,4,4} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio ΔT2TP E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB dB dB dB dB dB dB | Non E-DPCCH boosting Diversity: 4.09 Non-diversity: 6.98 Diversity: -9.54 Non-diversity: -5.46 E-DPCCH Boosting Diversity: 12 Non-diversity: 15 Diversity: 19.99 Non-diversity: 22.00 Diversity: 16.03 Non-diversity: 14.09 E-DPDCH/DPCCH power ratio is calculated for a single E-DPDCH with SF 4. The power of an E-DPDCH with SF2 is twice that of an E-DPDCH with SF4. |

# Figure A.17

# A.18	E-DPDCH Fixed reference channel 9 (FRC9)

# Table A.18

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation |  | QPSK |
| Maximum. Inf. Bit Rate | kbps | 8100 |
| TTI | ms | 2 |
| Number of HARQ Processes | Processes | 8 |
| Information Bit Payload (NINF) | Bits | 16200 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 23040 |
| Coding Rate (NINF/ NBIN) |  | 0.703 |
| Physical Channel Codes | SF for each physical channel | {2,2,4,4} |
| E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio S-DPCCH/DPCCH power ratio S-E-DPCCH/DPCCH power ratio S-E-DPDCH/DPCCH power ratio | dB dB dB dB dB | 6.02 -1.94 -1.94 0.00 6.02 E-DPDCH/DPCCH power ratio is calculated for a single E-DPDCH with SF 4. The power of an E-DPDCH with SF2 is twice that of an E-DPDCH with SF4. |

# Figure A.18

# A.19	E-DPDCH Fixed reference channel 10 (FRC10)

# Table A.19

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation |  | 16QAM |
| Maximum. Inf. Bit Rate | kbps | 16218 |
| TTI | ms | 2 |
| Number of HARQ Processes | Processes | 8 |
| Information Bit Payload (NINF) | Bits | 32436 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 46080 |
| Coding Rate (NINF/ NBIN) |  | 0.704 |
| Physical Channel Codes | SF for each physical channel | {2,2,4,4} |
| E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio S-DPCCH/DPCCH power ratio S-E-DPCCH/DPCCH power ratio S-E-DPDCH/DPCCH power ratio | dB dB dB dB dB | 19.99 16.03 16.03 [0.00] 19.99 E-DPDCH/DPCCH power ratio is calculated for a single E-DPDCH with SF 4. The power of an E-DPDCH with SF2 is twice that of an E-DPDCH with SF4. |

<End of changes>