3GPP TSG RAN WG4 Meeting #67	R4-133120

- Fukuoka, JAPAN, 20th – 24th May, 2013

- Agenda Item:	7.19

- Source:	LG Electronics and LG Uplus

- Title:	TP for Harmonics and IMD analysis for inter-band CA_1A-5A UE with 2ULs

Document for:	Approval

- Introduction

In the RAN Plenary #59 meetings, the study for the LTE-A dual uplink inter-band CA is approved. And the skeleton of technical report 36.8xx for Rel-12 was discussed in last RAN4 meeting [1].

The objective of work item on dual uplink inter-band CA is to gather the relevant background information and studies in order to address Dual Uplink Inter-band Carrier Aggregation requirements according to the CA class1 to 5.

In this contribution, we provide the harmonics and IMD analysis for CA_1A-5A UE with 2ULs.

- References

[1] R4-131929, “TR skeleton for inter-band CA with 2UL v.0.0.2,” Huawei

[2] R4-131416, “Protected Band for B1+B5 2 UP LINK CA,” LG Uplus

[3] TS 36.101 v11.4.0

[4] R4-133060, “General coexistence analysis table for inter-band CA with 2UL,” ZTE, LGE

Text proposal for TR 36.860

********************** Start of text proposal to TR 36.860 Chapter 6.2.1 ***********************

- 6.2.1	Inter-region OOBE requirements for Carrier Aggregation of Band 1 and Band 5

Table 6.2.1-1: 2UL inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FUL_low – FUL_high |  |  |  |  |
| CA_1-5 | 1 | 1920 MHz | – | 1980 MHz | 10 | 2110 MHz | – | 2170 MHz | 10 | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 10 | 869 MHz | – | 894 MHz | 10 |  |

      - List of specific combination issues

        - Channel bandwidths per operating band for CA

Table 6.2.1.1.1-1: 2UL inter-band CA operating bands

| E-UTRA band / channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Band | E-UTRA Bands | 1.4MHz | 3MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Bandwidth combination set |
| CA_1A-5A | 1 |  |  |  | Yes |  |  | 0 |
|  | 5 |  |  |  | Yes |  |  |  |

        - Co-existence studies for CA_1A-5A with 2ULs

For the analysis of 2UL inter-band CA 1A-5A UE, we should study and analyze the inter-modulation products and harmonics products when both Band 1 and Band 5 with 2ULs are transmitting simultaneously.

The combination of Band 1 and Band 5 has harmonics frequencies that are far away from the components receive and transmit frequencies, in the UL and in the DL of the CA combination, as shown in table 6.2.1.1.2-1. So there was no harmonic issue except Band 41. In the conventional SE for UE-to-UE coexistence requirements, the 3rd harmonics of Band 5 UE already fall into B41 DL band, but exceptions for SE measurements are permitted if there is at least one individual RB within the transmission bandwidth for which the 2nd, 3rd or 4th harmonic totally or partially overlaps the measurement bandwidth (MBW) [3].  And also, Band 41 is not defined as protected band when both Band1 and Band5 are operating.

Also, the inter-modulation products for CA_1A-5A UE with 2 ULs as shown in table 6.2.1.1.2-1. None of the IMD products fall into the own CA_1A-5A UE receive bands.  But, 2-tone 3rd IMD products may have impact on the Band 22 DL, Band 42 DL and 43 DL bands. But Band 22, Band 42 and Band 43 are far away from Band1 and Band 5, so we can assume that the UE front-end filter can effectively attenuate these IMD products.

Table 6.2.1.1.2-1: 2 UL B1 + B5 harmonic products and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 824 | 849 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 to 3960 |  |  | 1648 to 1698 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 to 5940 |  |  | 2472 to 2547 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1071 to 1156 |  |  | 2744 to 2829 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2991 to 3136 |  |  | 222 to 332 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4664 to 4809 |  |  | 3568 to 3678 |  |  |

        - ∆TIB and ∆RIB values

FFS

********************* End of text proposal to TR 36.860 Chapter 6.2.1 ************************