3GPP TSG-RAN WG4 Meeting #68	R4-133541

Barcelona, Spain, August 19th – 23rd, 2013

Agenda item:	8.19.1

Source: 	MediaTek Inc.

Title: 	CA_B39_B41 reference UE RF architecture for simultaneous Tx and Rx

Document for:	Discussion

## 1	Introduction

In RAN4 #66bis meeting, two reference UE RF architectures supporting CA_B39_B41 had been defined as way forward for further evaluations [1]. In RAN4 #67 meeting, most companies agreed to base on a single and more general reference architecture to derive the Tib and Rib requirements, and that tends to be the one supporting simultaneous Tx and Rx [2]. In this contribution, we suggest to modify this reference architecture slightly for it to be closer to practical implementation.

## 2	Discussion

Figure 2-1 shows the CA_B39_B41 reference UE RF architecture as defined in [1] which supports simultaneous Tx and Rx as well as dual uplink. To support this unique feature, a T/R switch is introduced between diplexer and Tx/Rx signal paths for each band, and that provides an excellent opportunity for Tx and Rx RF front-end to share the same band selection filter. In other words, two separate Tx and Rx RF filters can be replaced by a single filter located between diplexer and T/R switch.

Figure 2-1 CA_B39_B41 reference UE RF architecture supporting simultaneous Tx and Rx

Though having separate filters in Tx and Rx paths seems to be a more generalized topology from system requirements derivation perspective, in practice, it is likely the same filter would be used. Therefore, it would make no difference in terms of Tib and Rib requirements between the topologies having separate and shared RF filters. And most likely shared filter topology will be used in practical implementation as it is more cost-effective and area efficient.

On the other hand, in last RAN4 meeting, it was also suggested to include antenna switch in this reference architecture [3] as most UEs supporting this CA band combination are likely to support other bands as well.

Based on these two observations, we suggest modifying this reference architecture to have shared Tx/Rx RF filter for each band and include antenna switch, as shown in Figure 2-2.

Figure 2-2 Suggested CA_B39_B41 reference UE RF architecture supporting simultaneous Tx and Rx

## 3	Conclusion

In this contribution, we suggest modifying CA_B39_B41 reference UE RF architecture supporting simultaneously Tx and Rx as previously proposed in [1] to have shared Tx/Rx RF filter and include antenna switch for it to be closer to practical implementation.

## 4	References

- R4-131928, “UE requirement structure for CA_B39_B41 way forward”, CMCC, Renesas, CATT, Huawei, MediaTek, Qualcomm, TSG-RAN WG4 Meeting #66bis, Chicago, USA, April 15th – 19th, 2013

- R4-132764, “B39+B41 TDD inter-band CA UE considerations”, Qualcomm Incorporated, TSG-RAN WG4 Meeting #67, Fukuoka, Japan, May 20th – 24th, 2013

- R4-132710, “CA_39-41 reference architecture”, Renesas Mobile Europe Ltd., TSG-RAN WG4 Meeting #67, Fukuoka, Japan, May 20th – 24th, 2013