3GPP TSG RAN WG4 Meeting #68	R4-133757

- Barcelona, Spain, 19th – 23th August, 2013

- Agenda Item:	10.1

- Source:	LG Electronics

- Title:	TP for S-band UE Tx/Rx RF requirements for standalone Band

Document for:	Approval

- Introduction

In the RAN Plenary #57 meetings, the study for the LTE S-band in Korea was approved. And the UE-to-UE coexistence analysis and requirements was approved [1] for TR 36.861 v0.2.0 in RAN4 #67 meeting.

One of the objectives for this study item is to facilitate and harmonize the efficient use of these bands for terrestrial IMT, especially LTE, for Korea.

TDD Band 34 is not considered the coexistence band since SI rapporteurs provide the modified deployment plan and proposals [2]. Hence if TDD Band 34 and terrestrial S-band are deployed in same countries/regions, we need further discussion to protect Band 34 UE.

In this contribution, we provide the UE transmitter/Receiver RF requirements for the deployments plan of new standalone S-band.

- References

- R4-132400, “TR 36.861 v0.2.0,” SKT

- R4-133145, “Coexistence with Band 34,” KT, SK Telecom, LGU+, ETRI

- TS 36.101 v11.5.0

- R4-120891, “Information of KCC plan on 2.1GHz Spectrum in Korea,” TTA

- R4-131390, “TP for TR ab.cde : General issue for FS_LTE_1980_2170_Korea,” LG Electronics

Text proposal for TR 36.861

********************** Start of text proposal to TR 36.861 chapter 8 ***********************

- 8	Study of E-UTRA specific issues

This section summarizes issues and additional requirements when MS UE is deployed under Korean regulations.

- 8.1    UE Tx RF Requirements

- 8.1.1	New separate band (1980-2010 MHz and 2170-2200 MHz)

- 8.1.1.1 IMD and Harmonic analysis

In this section, we analyze the harmonic and IMD products which have impact on the bands in Korea.

Table 8.1.1.1-1 gives the harmonic products from S-band UE according to the non-CA UE. From Table 8.1.1.1-1, it can be seen that none of the harmonic products fall into the own S-band DL and the other deployed bands in Korea, which means there is no harmonic problem from S-band UE in Korea.

Table 8.1.1.1-1: S-band UE harmonic products

| UE UL carriers | f1_low | f1_high |
| --- | --- | --- |
| UL frequency (MHz) | 30MHz |  |
|  | 1980 | 2010 |
| 2nd order harmonics frequency range (MHz) | 3960 to 4020 |  |
| 3rd order harmonics frequency range (MHz) | 5940 to 6030 |  |

Table 8.1.1.1-2 shows the frequency ranges of IMD products for S-band UE according to multi-clustered transmission of non-CA UE. We can also see that there is no IMD products falling into the own S-band DL and the other deployed bands in Korea, which means there is no IMD problem from S-band UE in Korea.

Table 8.1.1.1-2: S-band UE IMD products

| UE UL carriers | f1_low | f1_high | f1_low | f1_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 10MHz |  | 20MHz |  |
|  | 1980 or 2000 | 1990 or 2010 | 1980 or 1990 | 2000 or 2010 |
| Two-tone 3rd order IMD products | ¦2*f1_low – f1_high¦ | ¦2*f1_high – f1_low¦ | ¦2*f1_low – f1_high¦ | ¦2*f1_high – f1_low¦ |
| IMD frequency range (MHz) | 1970 to 2000 or 1990 to 2020 |  | 1960 to 2020 or 1970 to 2030 |  |
| Two-tone 3rd order IMD products | (2*f1_low + f1_high) | (2*f1_high + f1_low) | (2*f1_low + f1_high) | (2*f1_high + f1_low) |
| IMD frequency range (MHz) | 5950 to 5960 or 6010 to 6020 |  | 5960 to 5980 or 5990 to 6010 |  |
| Two-tone 5th order IMD products | (3*f1_low – 2*f1_high) | (3*f1_high – 2*f1_low) | (3*f1_low – 2*f1_high) | (3*f1_high – 2*f1_low) |
| IMD frequency range (MHz) | 1960 to 2010 or 1980 to 2030 |  | 1940 to 2040 or 1950 to 2050 |  |
| Two-tone 7th order IMD products | (4*f1_low – 3*f1_high) | (4*f1_high -3*f1_low) | (4*f1_low – 3*f1_high) | (4*f1_high -3*f1_low) |
| IMD frequency range (MHz) | 1950 to 2020 or 1970 to 2040 |  | 1920 or 2060 or 1930 to 2070 |  |

8.1.1.2 	Spurious emission for UE co-existence

From the UE-to-UE coexistence analysis [5] and harmonic/IMD product analysis in this paper, we can see that there is no need to define A-MPR requirements and NS signalling to protect the bands in Korea.

As a conclusion of previous discussion, SE requirements for UE-to-UE coexistence can be defined as -50dBm/MHz for S-band UE as in Table 8.1.1.2-1.

Table 8.1.1.2-1: Spurious emission band UE co-existence of S-band UE

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Level (dBm) | Bandwidth (MHz) | Comment |
| XX | E-UTRA Band 1,3,5,8,26,XX,40 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1: This requirement is applicable for S-band UE of Band XX in Korea |  |  |  |  |  |  |  |

8.1.1.3	Other expected UE Tx RF requirements

In Table 8.1.1.3-1, detailed descriptions on UE transmitter RF requirements were proposed to predict any potential requirements changes needed for S-band UE when Band 34 is not considered as coexisting band for S-band deployments.

Table 8.1.1.3-1: Considering UE Tx RF requirements for S-band UE

| Description | Comments and Description of change |
| --- | --- |
| UE Maximum Output Power | No changes are needed for power class 3 due to the large duplex gap and Tx-Rx separation. |
| UE Maximum Output Power with additional requirements (A-MPR) | No changes are needed to protect adjacent coexistence bands due to no IMD and harmonics in Korea [1]. |
| Spurious emission/ additional Spurious emission | No changes are needed to protect whole frequency bands for S-band UE in Korea. |
| ACLR/SEM | No changes are needed in OOB ranges for S-band UE in Korea. |
| Additional Spectrum Emission Mask | No changes are needed to protect adjacent coexistence bands due to no IMD and harmonics in Korea [1]. |

8.1.2	New Extended band (1920-2010 MHz and 2110-2200 MHz)

< <text will be added>.

  - UE Rx RF Requirements

    - New separate band (1980-2010 MHz and 2170-2200 MHz)

- 8.2.1.1 REFSENS requirements for S-band UE

From the analysis of S-band UE co-existence and RF simulation results [5], there is no co-existence impact on band 1 UE. Also S-band UE have same Tx-Rx frequency separation and total bandwidth of S-band is smaller than that of Band 1, therefore S-band UE’s REFSENS level can be easily regarded as same as that of Band 1. To verify this assumption, we performed simple RF simulation, the result of which is shown in Fig. 8.2.1.1-1.

Figure8.2.1.1-1 show the redline is Tx leakage level of 10MHz/20MHz S-band UE into S-Band DL band with full RBs allocation.

- 20MHz S-band UE (fc=2000MHz)                      (b) 10MHz S-band UE (fc=2005MHz)

Figure 8.2.1.1-1 Simulation results of the Tx leakage level into S-band DL

The leakage level in DL band is about -75~-80dBm/MHz, which levels are attenuated by Duplexer Tx/Rx isolation level with 50dB. So the leakage level in DL band  is under -125dBm/MHz. Eq. 8-1 shows how to calculate REFSENS in general.

- REFSENS = kTB + 10log(CH_BW) + NF+ IM + SINR +Diversity gain + DFB (dBm)    (Eq. 8-1)

= -174 dBm/Hz + 10log(20MHz) + 9dB+2dB-1dB -3dB +0dB

= -104+11-4= - 94dBm/CH_BW

Where, kTB is thermal noise level, NF is noise figure, IM is impairment margin, SINR level is for QPSK modulation, Diversity gain is the receiver diversity gain and DFB means that Additional relaxation by reflection of design challenges for FE.

Eq. 8-2 shows the impact of REFSENS in S-band DL induced by Tx leakage.

- REFSENS for S-band UE = Conventional REFSENS + Tx_leakage levels                          (Eq. 8-2)

= -94dBm/CH_BW +  -125dBm/MHz

= -107dBm/MHz + -125dBm/MHz = - 106.93dBm/MHz

= -93.97dBm/CH_BW

From Eq. 8-1 and Eq. 8-2, we can see that the REFSENS in own Rx band by Tx leakage is negligible.

As a conclusion, REFSENS requirements for S-band UE can be defined as -94dBm/20MHz in Table 8.2.1.1-1 with the same uplink configurations as in Band1 [2].

Table 8.2.1.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| XX |  |  | -100 | -97 | -95.2 | -94 | FDD |

- 8.2.1.2 Other UE Rx RF requirements

For the receiver requirements for S-band UE, we propose preliminary descriptions on UE receiver requirements in Table 8.2.1.2-1 when Band 34 is not consider for S-band deployments. From the above REFSENS analysis, Tx leakage is expected to be negligible since the large duplexer spacing and same Tx-Rx frequency separation as Band1. Hence we can reuse the receiver requirements in Table 8.2.1.2-1 for S-band UE.

Table 8.2.1.2-1: Considering UE Rx RF requirements for S-band UE

| Description | Comments and Comparison to 2-band Solution |
| --- | --- |
| Adjacent Channel Selectivity | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements that can be larger than or equal to 95% with same condition in section 7.5.1 [2] |
| Blocking characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.6 [2] |
| Receiver inter-modulation characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.8.1 [2] |

8.2.2	New Extended band (1920-2010 MHz and 2110-2200 MHz)

< <text will be added>.

********************** End of text proposal to TR 36.861 Chapter 8 ***************************