3GPP TSG RAN WG4 Meeting #68	R4-13779

Barcelona, Spain, 19th – 23th August, 2013

Agenda Item:	8.22

Source:	LG Electronics

Title:	TP for general frequency bands for WiFi, GNSS and Digital TV for 2ULs inter-band CA

Document for:	Approval

- Introduction

In the RAN4 #67 meetings in Fukuoka, RAN4 had agreed that the IMD analysis should be studied for the LTE-A dual uplink inter-band CA into GNSS(GPS, Galileo, Glonass, Compass, etc), the connectivity radios (WLAN, BT etc in ISM bands) and Digital TV systems [1].

In this contribution, we propose the general IMD analysis table between LTE-A UE and other systems such as WLAN, GNSS and Digital TV for inter-band CA with 2ULs. This can help us to unify the analysis result.

- References

[1] TR 36.860 v0.1.0

[2] TS 36.101 v11.5.0

Text proposal for TR 36.860

********************** Start of text proposal to TR 36.860 Chapter 5.2.1 ***********************

5.2.1		UE Coexistence requirements for Dual UL CA when the aggregated bands are from different regions

A general harmonics and IMD coexistence analysis table for inter-band CA with 2UL is given in table 5.2.1-1. For UE-UE coexistence, it is sufficient to consider only up to 3rd order intermodulation.

Table 5.2.1-1: Band x and Band y UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) |  |  |  |  |  |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) |  |  |  |  |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) |  |  |  |  |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) |  |  |  |  |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) |  |  |  |  |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) |  |  |  |  |  |  |

When 2ULs inter-band CA UE is operated with the other systems such as WLAN, GNSS and DTV system, the intermodulation products can have impact on these systems, hence we provide a general IMD coexistence analysis table to protect other systems in table 5.2.1-2. From the analysis table, we can summarize the IMD problems.

Table 5.2.1-2: General The IMD analysis table to protect connectivity radios, DTV and positioning systems

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  | E2-L1-E1 |
|  | 1260 | - | 1279 |  |  | E6 |
|  | 1192 | - | 1215 |  |  | E5b |
| Galileo | 1544 | - | 1545 |  |  | SAR |
|  | 1559 | - | 1591 |  |  | E2-L1-E1 |
|  | 1260 | - | 1300 |  |  | E6 |
|  | 1164 | - | 1215 |  |  | E5A-E5B |
| GLONASS | 1591 | - | 1610 |  |  | L1 |
|  | 1240 | - | 1256 |  |  | L2 |
| GPS | 1563 | - | 1587 |  |  | L1 |
|  | 1215 | - | 1237 |  |  | L2 |
|  | 1164 | - | 1192 |  |  | L5 |
| WLAN 2.4GHz | 2400 | - | 2495 |  |  |  |
| WLAN 5GHz | 5150 | - | 5925 |  | US |  |
|  | 5150 | - | 5350 |  | Europe |  |
|  | 5470 | - | 5725 |  |  |  |
|  | 5150 | - | 5825 |  | Asia |  |
| Digital TV | 470 | - | 698 |  |  |  |

********************* End of text proposal to TR 36.860 Chapter 5.2.1 ************************