3GPP TSG-RAN4 Meeting #68	 R4-133995

# Barcelona, Spain, 19-23 August, 2013

Source: 	SONY Mobile Communications Japan, Inc.

Title: 	LTE-bands TRP/TRS measurement data from some UE:s.

Agenda Item: 	8.3.1.4

Document for:	Discussion

__________________________________________________________________________________________

1. 	Introduction

With reference to 3GPP TS 37.144 chapters 6.1.2 and 7.1.2 which aim at stipulating TRP/TRS requirements for E-UTRA FDD UE:s at “Beside Head”, this document provides measurement data from 4 Sony smart-phones, to serve as input/reference in discussions with respect to appropriate requirement levels.

The document R4-132117 submitted previously proposes requirement levels.

__________________________________________________________________________________

2.	Testing conditions

- All UE:s are multi-band, multi mode smart-phones LTE power class 3 devices with display sizes ranging from 4” to 5”.

- All tests were conducted at Sony facilities in calibrated though not accredited commercial test chambers, Bluetest RTS 60 or ETS-Lindgren AMS 8500.

- Measurement results presented are the average values taken from the high, mid, low channels per band on the left and right hand sides (BHH) at LTE band 3, 7 and 20 respectively.

- Phantom head and hands selection and device positioning are according to TR 25.914 and CTIA test plan V 3.2.

- TRP and TRS measured at 10 MHz BW and QPSK modulation.

- TRS figures are given separately for the primary and secondary receiver/antennas.

_______________________________________________________________________________

3.		Measurement data

| Band | Mode | UE, dBm |  |  |  | Proposed [1] Recommended requirement |
| --- | --- | --- | --- | --- | --- | --- |
| LTE 3 |  | A (Bluetest) | B (AMS8500) | C (AMS8500) | D (Bluetest) |  |
|  | FS TRP | 19.9 | 20.3 | 19.6 | 20.4 | 19 |
|  | FS TRS main |  |  |  |  | -91 |
|  | FS TRS 2:nd |  |  |  |  | -87 |
|  | BHH TRP | 17.2 | 13.8 | 15.1 |  | 13 |
|  | BHH TRS main | -87.7 | -86 | -91.4 | -87.1 | -88 |
|  | BHH TRS 2:nd | -83.8 | -81.3 | -84.8 | -84.6 | -84 |
|  | H TRP |  |  |  |  | 15 |
|  | H TRS main |  |  |  |  | -88 |
|  | H TRS 2:nd |  |  |  |  | -84 |
| LTE 7 | FS TRP | 21.1 | 18.7 | 17.3 | 17.4 | 18 |
|  | FS TRS main |  |  |  |  | -89 |
|  | FS TRS 2:nd |  |  |  |  | -85 |
|  | BHH TRP | 15.5 | 15.7 | 14.2 |  | 12 |
|  | BHH TRS main | -87.3 | -85 | -89.2 | -86.7 | -87 |
|  | BHH TRS 2:nd | -85.7 | -84.3 | -85.6 | -81.6 | -83 |
|  | H TRP |  |  |  |  | 12 |
|  | H TRS main |  |  |  |  | -87 |
|  | H TRS 2:nd |  |  |  |  | -83 |
| LTE 20 | FS TRP | 18.6 | 19.2 | 17.5 | 17.9 | 17 |
|  | FS TRS main |  |  |  |  | -89 |
|  | FS TRS 2:nd |  |  |  |  | -85 |
|  | BHH TRP | 11.1 | 10.1 | 7.5 |  | 9 |
|  | BHH TRS main | -85.7 | -87.3 | -82.2 | -83.9 | -83 |
|  | BHH TRS 2:nd | -77.2 | -74.3 | -80.8 | -83.7 | -79 |
|  | H TRP |  |  |  |  | 12 |
|  | H TRS main |  |  |  |  | -86 |
|  | H TRS 2:nd |  |  |  |  | -82 |

__________________________________________________________________________________________

4.	Conclusion

Although measurement data from only 4 smart-phones have been presented, it is apparent that it may be difficult to meet the proposed requirement levels for all bands and modes. The aim here is thus to highlight the need for realistic requirement levels on TRP/TRS for multi-mode UE:s and thereby contribute to the specification document 3GPP TS 37.144, chapter 6.1.2 and 7.1.2.

__________________________________________________________________________________

5.	References

[1]                   R4-132117 E-UTRA FDD UE TRP/TRS requirement data proposal.