3GPP TSG-RAN WG4 #68	R1-134003

Barcelona, Spain, 19th – 23th, Aug 2013

Source: 	Ericsson, ST-Ericsson

Title:	Link level simulation results and discussion for fading CQI test

Agenda Item:	7.8.2.2

Document for:	Discussion

# Introduction

In RAN4#66bis meeting, one WF paper on FeICIC CSI tests is agreed in [2].  In RAN4#67, the WF is refined in [1]. For fading CQI test, the group agrees to investigate:

- Propagation conditions and channel matrix

  - Option 1: Follow Rel-8 two-tap channel model, but the parameters are FFS (suggested below), and use 2x2 antenna configuration.

    - Serving cell: Td=0.45us, a=1, fd=+5Hz

    - Aggressor 1: Td=0.7us, a=0.8, fd=-5Hz

    - Aggressor 2: Td=0.8us, a=0.7, fd=+3Hz

  - Option 2: use two-tap channel model for serving cell, EVA5 for the aggressor cells

  - Other options are not precluded.

- Antenna configuration

  - Option 1: 1×2;

  - Option 2: 2×2;

In this paper, we provide link level simulation results for these options. Based on the link level simulation, we provide our view on the fading CSI test for FeICIC.

# Preliminary link level simulation for fading CQI test

In the link level simulation, simulation parameters are shown in Table 1. In the simulation, 2x2 antenna configuration is used, since it is widely used in current products. For propagation conditions and channel matrix, both option 1 and option 2 are simulated.

For fading CQI test of Rel-8/Rel-9/Rel-10, the following criterion is used:

- Criterion 1: a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

- Criterion 2: the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

- Criterion 3: when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to 0.05.

In Table 2 and Table 3, link level simulation results are shown for option 1 and option 2. In Table 2 and Table 3, the second column is corresponding to criterion 2, and the third column is corresponding to criterion 3, and the last 8 columns are corresponding to criterion 1. Based on Table 2 and Table 3, we have the following observations:

- Observation 1: Both options can meet criterion 1 and criterion 2.

- Observation 2: For low SNR region, criterion 3 can be met by option 1 and option 2, but for high SNR region, criterion 3 cannot be met.

Table 1: Parameters for fading CQI test setup

| Parameter | Unit | Value |
| --- | --- | --- |
| Bandwidth | MHz | 10 MHz |
| Transmission mode |  | 2 (port 0 and 1) |
| Propagation channel for serving cell |  | Clause B.2.4 with s, a = 1, Hz |
| Propagation channel for aggressor cells |  | Option 1: Aggressor 1: Td=0.7us, a=0.8, fd=-5Hz Aggressor 2: Td=0.8us, a=0.7, fd=+3Hz Option 2: EVA5 for both aggressors |
| Antenna configuration |  | 2 x 2 (low) |
| Reporting interval | ms | 5 |
| CQI delay | ms | 8 |
| Reporting mode |  | PUSCH 3-0 |
| Sub-band size | RB | 6 (full size) |
| Max number of HARQ transmissions |  | 1 |
| D1/Noc |  | 12 dB |
| D2/Noc |  | 10 dB |
| CRS configuration |  | The CRS of the first domain aggressor cells is colliding with the serving cell, and the CRS of the second aggressor cells is non-colliding with the serving cell |

Table 2: Fading CQI test based on two-tap channel (option 1)

Table 3: Fading CQI test based on two-tap channel (option 2)

For observation 2, the element filled by light red shows that the BLER is smaller than 5% in high SNR region. In other words, the reported CQI is conservative.  The conservation is explained in one companion paper [3]. For simplification, there always exists residual noise after CRS-IM in CRS REs, but in data REs in ABS protected subframes, it is clean. Hence, there is mismatch between the interference estimation in CRS REs and data REs. The mismatch becomes severe for high MCS which is corresponding to high SNR. Hence, we propose:

- Proposal 1: High SNR check point is not introduced for fading CQI test

Based on option 1 and option 2, the observations are very similar. But for option 1, the channel is deterministic channel. It is easy for UE cheating. Moreover, EVA5 is closer to practical channel but without too much impact on the UE fading CQI performance. Hence, EVA5 is more reasonable taken as the interference channel.

- Proposal 2: EVA5 with 2x2 low channel  is preferred for  aggressor cells in fading CQI test

# Conclusion

In this paper, link level simulation results are provided for fading CQI test. We have the following observations:

- Observation 1: Both options can meet criterion 1 and criterion 2.

- Observation 2: For low SNR region, criterion 3 can be met by option 1 and option 2, but for high SNR region, criterion 3 cannot be met.

We have the following proposals:

- Proposal 1: High SNR check point is not introduced for fading CQI test

- Proposal 2: EVA5 with 2x2 low channel  is preferred for  aggressor cells in fading CQI test

# Reference

R4-133037, WF on FeICIC CSI tests, Qualcomm, May 2013

R4-131898, WF on FeICIC demod and CSI, Qualcomm, Huawei, HiSilicon, NEC, May 2013

R1-134005, Link level simulation results and discussion for static CQI test, Ericsson, ST Ericsson, Aug 2013