TSG-RAN Working Group 4 meeting #68	R4-134093

Barcelona, Spain, August 19th - August 23rd, 2013

Source: 	US Cellular

Title: 	CA_B12_B25: Core requirement considerations and text proposal

Agenda Item:	8.17.5

Document for:	Approval

Introduction

The WI proposal to support the LTE Advanced Inter-band Carrier Aggregation (CA) of Band 12 and Band 25 was approved in RAN#60 [1]. The operating band ranges for this CA band are specified. The harmonic and Inter Modulation Distortion (IMD) products generated by the base station supporting this inter-band CA are proposed, based on [2].

With the objective of specifying the core requirements for this CA combination, which belongs to the A1 category in the CA framework, a TP (Text Proposal) is provided, for inclusion in [3].

Text Proposal

- < Start of TP segment in TR 36.851 - Unchanged sections omitted >

### 6.1.X	LTE Advanced Carrier Aggregation of Band 12 and Band 25 (1 UL)

Table 6.1.X-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_2-12 | 12 | 699 MHz | – | 716 MHz | 5, 10 | 729 MHz | – | 746 MHz | 5, 10 | FDD |
|  | 25 | 1850 MHz | – | 1915 MHz | 5, 10 | 1930 MHz | – | 1995 MHz | 5, 10 | FDD |

#### 6.1.X.1		List of specific combination issues

##### 6.1.X.1.1	Channel bandwidths per operating band for CA

Table 6.1.X.1.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| CA_2A-12A | 12 |  |  | Yes | Yes |  |  |
|  | 25 |  |  | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [4] when operating in single carrier mode.

##### 6.1.X.1.2	Co-existence studies for CA _12_25

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 12 and Band 25 DL carriers can be calculated as shown in Table 6.1.X.1.2-1 below:

Table 6.1.X.1.2-1: Band 12 and Band 25 DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 1930 | 1995 |
| 2nd harmonics frequency limits (MHz) | 1458 | 1492 | 3860 | 3990 |
| 3rd harmonics frequency limits (MHz) | 2187 | 2238 | 5790 | 5985 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 1184 | 1266 | 2659 | 2741 |
| 3rd order IMD products | (f2-low – 2*f1-high) | (f2-high – 2*f1-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 438 | 537 | 3114 | 3261 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 3388 | 3487 | 4589 | 4736 |
| 3rd order IMD products | (f1-low – f2-high + f2-low) | (f1-high + f2-high – f2-low) | (f2-low – f1-high + f1-low) | (f2-high + f1-high – f1-low) |
| IMD frequency limits (MHz) | 664 | 811 | 1913 | 2012 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 719 | 756 | 1920 | 2005 |

It can be seen from Table 6.1.X.1.2-1 that the 2nd harmonics of Band 12 carriers may fall into the BS receive band of Band 21, while the 2nd IMD products caused by BS supporting carrier aggregation of Band 12 and Band 25 may fall into the BS receive band of Band 41, and the 3rd IMD products may fall into the BS receive band of Bands 1, 12, 13, 14, 17, 22, 23, 25, 28, 33, 36, 37, 39, 42 and 44. Note that the calculation in Table 6.1.X.1.2-1 (except the last row) assumes the BS is transmitting with the whole 60 MHz DL frequency of Band 25 and the whole 17 MHz DL frequency of Band 12. If the BS is only transmitting an up to 10 MHz DL in Band 25 and Band 12 as stated in the WIDS, then the 3rd IMD products will not fall into the BS receive band of Band 12, 13, 14, 17, 23, 25, 33 or 39 as shown in the last row in Table 6.1.X.1.2-1.

It should be noted that Bands 1, 21, 28, and 44 are not intended for use in the same geographical area as Bands 25 and 12. Moreover, a co-location of Band (12 + 25) transmitter and Band 36 or 37 transceivers implies FDD/TDD co-location on adjacent frequencies which requires the use of certain site-engineering solutions to avoid mutual interference. Therefore, the focus here will be on the harmonics and IMD falling into Bands 22, 41 and 42.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 22, 41 or 42 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 2 and 12 BS transmitters do not share the same antenna with Band 22, 41 or 42 BS receiver.

Therefore, it is recommended that Bands 12 and 25 BS transmitters should not share the same antenna with Band 22, 41 or 42 BS receivers; unless the antenna path meets very stringent 2nd and 3rd order PIM specification so that the PIM will not cause Band 22, 41 or 42 BS receiver de-sensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

< End of TP segment in TR 36.851 – Unchanged sections omitted >

Reference

[1]	RP-130832, “New WI proposal: LTE Advanced inter-band Carrier Aggregation of Band 12 and Band 25”, US Cellular.

[2]	R4-131197, “Text Proposal on Coexistence Studies of Harmonics and Intermodulation Products for Band Combination (2 + 12)” Alcatel-Lucent, US Cellular.

[3]	R4-132863, “Rel-12 Inter-band Carrier Aggregation TR 36.851 V0.5.0”, Ericsson, ST-Ericsson.