3GPP TSG RAN WG4 Meeting #68                                      	      R4-134095

Barcelona, Spain

August 19-23, 2013

Agenda Item:	6.1

Source:	Motorola Mobility

Title:	Consistent Handling of Tolerances with P-MPR

Document for:   Discussion and Approval

# Introduction

In the PCMAX equations in Section 6.2.5 and 6.2.5A of the specification TS 36.101, the tolerances ΔTC and ΔTIB are handled inconsistently with respect to P-MPR.  In particular, the tolerance ΔTC is added to P-MPR while ΔTIB is not.  Conversely, both tolerances are added in determining PCMAX when P-MPR is zero.  In this contribution, it is proposed that the tolerances be handled consistently with respect to P-MPR.  Furthermore, since there is no restriction on the value of P-MPR that the UE may apply, it is proposed that the tolerances are ΔTC and ΔTIB not be added to P-MPR in the calculation of PCMAX.

# Simulation Results

In Section 6.2.5 of TS 36.101, PCMAX_L is given as

PCMAX_L = MIN {PEMAX – ΔTC, PPowerClass – MAX(MPR + A-MPR + ΔTIB,c, P-MPR) – ΔTC}

which can also be written as

PCMAX_L = MIN {PEMAX – ΔTC, PPowerClass – MAX(MPR + A-MPR + ΔTIB,c + ΔTC, P-MPR + ΔTC)}

Thus, in the computation of PCMAX_L the tolerance ΔTC is added to P-MPR while the tolerance ΔTIB,c is not.  Conversely, it can be noted that both tolerances ΔTC and ΔTIB,c are added to MPR + A-MPR.

More specifically, we can consider an example in which PEMAX = PPowerClass, so that

PCMAX_L = PPowerClass – MAX(MPR + A-MPR + ΔTIB,c + ΔTC, P-MPR + ΔTC)

Now, if

MPR + A-MPR + ΔTIB,c + ΔTC > P-MPR + ΔTC,

then

PCMAX_L = PPowerClass – MPR - A-MPR - ΔTIB,c - ΔTC

so that PCMAX_L is reduced by both ΔTC and ΔTIB,c.  Conversely, if

MPR + A-MPR + ΔTIB,c + ΔTC < P-MPR + ΔTC,

then

PCMAX_L = PPowerClass – P-MPR - ΔTC

so that PCMAX_L is reduced by ΔTC but not by ΔTIB,c.

Thus, the setting of PCMAX_L can be seen to be inconsistent in that both tolerances are applied in setting  PCMAX_L when P-MPR is 0, but only ΔTC is applied in setting PCMAX_L when the value of P-MPR is larger than MPR + A-MPR + ΔTIB,c.

# Modification of PCMAX_L

In order to maintain consistent handling of the tolerances ΔTC and ΔTIB with the use of P-MPR, either both of the tolerances should be added to P-MPR, or neither tolerance should be added. Furthermore, since there is no restriction on the value of P-MPR that the UE may apply, it is proposed that neither tolerance be added to P-MPR in the calculation of PCMAX. Thus, we have the following:

Proposal:

In Section 6.2.5, PCMAX_L shall be defined as

PCMAX_L = MIN {PEMAX – ΔTC, PPowerClass – MAX(MPR + A-MPR + ΔTIB,c + ΔTC, P-MPR) }

and similar modifications shall be made in Section 6.2.5A.

# Conclusion

In the PCMAX equations in Section 6.2.5 and 6.2.5A of the specification, the tolerances ΔTC and ΔTIB are handled inconsistently with respect to P-MPR.  It is proposed that the PCMAX equations in Section 6.2.5 and 6.2.5A be modified so that neither tolerance be added to P-MPR in the calculation of PCMAX

# References

- TS 36.101, v11.5.0.