3GPP TSG RAN WG4 Meeting #68	R4-134388

- Barcelona, Spain, 19th – 23th August, 2013

- Agenda Item:	10.1

- Source:	LG Electronics

- Title:	TP for S-band UE Tx/Rx RF requirements for the extended Band

Document for:	Approval

- Introduction

In the RAN Plenary #57 meetings, the study for the LTE S-band in Korea was approved. And the UE-to-UE coexistence analysis and requirements was approved [1] for TR 36.861 v0.2.0 in RAN4 #67 meeting.

One of the objectives for this study item is to facilitate and harmonize the efficient use of these bands for terrestrial IMT, especially LTE, for Korea.

TDD Band 34 is not considered the coexistence band since SI rapporteurs provide the modified deployment plan and proposals [2]. Hence if TDD Band 34 and terrestrial S-band are deployed in same countries/regions, we need further discussion to protect Band 34 UE.

In this contribution, we provide the UE transmitter/Receiver RF requirements for the deployments plan of new extended S-band.

- References

- R4-132400, “TR 36.861 v0.2.0,” SKT

- R4-133145, “Coexistence with Band 34,” KT, SK Telecom, LGU+, ETRI

- TS 36.101 v11.5.0

- R4-120891, “Information of KCC plan on 2.1GHz Spectrum in Korea,” TTA

- R4-131390, “TP for TR ab.cde : General issue for FS_LTE_1980_2170_Korea,” LG Electronics

Text proposal for TR 36.861

********************** Start of text proposal to TR 36.861 Chapter 8 ***********************

- 8.1    UE Tx RF Requirements

- 8.1.1	New separate band (1980-2010 MHz and 2170-2200 MHz)

< <text will be added>.

- 8.1.2	New Extended band (1920-2010 MHz and 2110-2200 MHz)

- 8.1.2.1 IMD and Harmonic analysis

In this section, we analyze the harmonic and IMD products which have impact on the bands in Korea.

Table 8.1.2.1-1 gives the harmonic products from S-band UE according to multi-clustered transmission of non-CA UE. From Table 8.1.2.1-1, it can be seen that none of the harmonic products fall into the own S-band DL and the other deployed bands in Korea, which means there is no harmonic problem from S-band UE in Korea.

Table 8.1.2.1-1: S-band UE harmonic products

| UE UL carriers | f1_low | f1_high |
| --- | --- | --- |
| UL frequency (MHz) | 90MHz |  |
|  | 1920 | 2010 |
| 2nd order harmonics frequency range (MHz) | 3840 to 4020 |  |
| 3rd order harmonics frequency range (MHz) | 5760 to 6030 |  |

Table 8.1.2.1-2 show the frequency ranges of IMD products for S-band UE according to multi-clustered transmission of non-CA UE. We can see that the 7th order IMD products falling into the Band 3 DL band but the 7th order IMD level is less than -50dBm/MHz, so it can be not impact the UE to UE co-existence requirements.  Therefore there is no IMD problem from S-band UE in Korea.

Table 8.1.2.1-2: S-band UE IMD products

| UE UL carriers | f1_low | f1_high | f1_low | f1_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 10MHz |  | 20MHz |  |
|  | 1920 or 2000 | 1930 or 2010 | 1920 or 1990 | 1940 or 2010 |
| Two-tone 3rd order IMD products | ¦2*f1_low – f1_high¦ | ¦2*f1_high – f1_low¦ | ¦2*f1_low – f1_high¦ | ¦2*f1_high – f1_low¦ |
| IMD frequency range (MHz) | 1910 to 1940 or 1990 to 2020 |  | 1900 to 1960 or 1970 to 2030 |  |
| Two-tone 3rd order IMD products | (2*f1_low + f1_high) | (2*f1_high + f1_low) | (2*f1_low + f1_high) | (2*f1_high + f1_low) |
| IMD frequency range (MHz) | 5770 to 5780 or 6010 to 6020 |  | 5780 to 5800 or 5990 to 6010 |  |
| Two-tone 5th order IMD products | (3*f1_low – 2*f1_high) | (3*f1_high – 2*f1_low) | (3*f1_low – 2*f1_high) | (3*f1_high – 2*f1_low) |
| IMD frequency range (MHz) | 1900 to 1950 or 1980 to 2030 |  | 1880 to 1980 or 1950 to 2050 |  |
| Two-tone 7th order IMD products | (4*f1_low – 3*f1_high) | (4*f1_high -3*f1_low) | (4*f1_low – 3*f1_high) | (4*f1_high -3*f1_low) |
| IMD frequency range (MHz) | 1890 to 1960 or 1970 to 2040 |  | 1860 to 2000 or 1930 to 2070 |  |

- 8.1.2.2 	Spurious emission for UE co-existence

For the coexistence requirements, a single duplexer approach for 2x90MHz arrangements is considered. Dual duplexer approach also can be considered, but the requirements are defined as single requirements and should be met by either a single duplexer or a split-duplexer solution.

Even though the duplexer gap is decreased from 130MHz to 100MHz and pass band is increased from 60MHz to 90MHz, the single duplexer can keep the conventional duplexer filter characteristics when we consider the protect band list without Band 34. So the UE-to-UE coexistence analysis [5] is applicable to the extended S-band UE. We can also see that there is no need to define A-MPR requirements and NS signalling to protect the bands in Korea.

As a conclusion of previous discussion, SE requirements for UE-to-UE coexistence can be defined as -50dBm/MHz for the extended S-band UE as in Table 8.1.2.2-1.

Table 8.1.2.2-1: Spurious emission band UE co-existence of the extended S-band UE

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Level (dBm) | Bandwidth (MHz) | Comment |
| XX | E-UTRA Band 1,3,5,8,26,XX,40 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1: This requirement is applicable for S-band UE of Band XX in Korea |  |  |  |  |  |  |  |

- 8.1.2.3	Other expected UE Tx RF requirements

In Table 8.1.2.3-1, detailed descriptions on UE transmitter RF requirements were proposed to predict any potential requirements changes needed for the extended S-band UE.

Table 8.1.2.3-1: Considering UE Tx RF requirements for S-band UE

| Description | Comments and Description of change |
| --- | --- |
| UE Maximum Output Power | Additional insertion loss will be added to calculate the UE MOP due to the decreased duplex gap and increased Pass band. |
| UE Maximum Output Power with additional requirements (A-MPR) | No changes are needed to protect adjacent coexistence bands due to no IMD and harmonics in Korea [1]. |
| Spurious emission/ additional Spurious emission | No changes are needed to protect whole frequency bands for S-band UE in Korea. |
| ACLR/SEM | No changes are needed in OOB ranges for S-band UE in Korea. |
| Additional Spectrum Emission Mask | No changes are needed to protect adjacent coexistence bands due to no IMD and harmonics in Korea [1]. |

  - UE Rx RF Requirements

    - New separate band (1980-2010 MHz and 2170-2200 MHz)

< <text will be added>.

    - New Extended band (1920-2010 MHz and 2110-2200 MHz)

- 8.2.2.1 REFSENS requirements for S-band UE

From the previous Rx RF simulation results, we can follow same REFSENS requirements for the extended S-band DL band since single duplexer can achieve same duplexer attenuation levels even though duplexer gap is reduced from 130MHz to 100MHz and pass band is increased from 60MHz to 90MHz. However, additional insertion loss of the extended S-band Duplexer is needed which is compared to conventional Band 1 Duplexer

As a conclusion, REFSENS requirements for S-band UE can be reduced by the factor of additional insertion loss of new duplexer of the extended S-band UE.

- 8.2.2.2 Other UE Rx RF requirements

For the receiver requirements for the extended S-band UE, we propose that we can reuse the conventional receiver requirements as shown in table 8.2.1.2-1.

********************** End of text proposal to TR 36.861 Chapter 8 ***************************