3GPP TSG RAN WG4 Meeting #68bis	R4-135500

Riga, Latvia, 7th – 11st October, 2013

Agenda item:		10.4.3

Source:	Renesas Mobile Europe Ltd.

Title:	TP for L2S modeling of WLMMSE-IRC

Document for:		Approval

# 1	Introduction

This document contains text proposal on link to system mapping of WLMMSE-IRC receiver. This TP is proposed to be included in TR36.863.

< start of text proposal >

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	RP-130404, "Study on Network Assisted Interference Cancellation and Suppression for LTE".

[2]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[y3]	3GPP TR36.829, “Enhanced performance requirement for LTE User Equipment (UE)”.

< text omitted >

## 9.1	System-level Modelling Methodologies

[Editor's note: This section will capture the system-level modeling for different types of receivers.]

### 9.1.1	System-level Methodology for WLMMSE-IRC Receiver

This section defines system level modelling for the WLMMSE-IRC receiver by starting from definition of general linear model and then defining the SINR calculation including modelling of estimation errors at the system level. Finally, the methodology is summarized.

#### 9.1.1.1	Signal model and receiver algorithm

A general linear model may be defined, where the received signal is given by:

r=H0x0+n

The linear model can be rewritten in augmented form as:

rA=H0Ax0A+nA

rA=I(r)Q(r)

H0A=I(H0)-Q(H0)Q(H0)I(H0)

x0A=I(x0)Q(x0)

nA=I(n)Q(n)

where all variables are real valued. Note that the interference vector n can be colored and operators I(·) and Q(·) take the real and imaginary parts of their arguments. As for any LMMSE receiver, the symbol estimate of the WLMMSE-IRC receiver equals:

| x0A=CrrA-1H0ACxxArA=grA | (x1) |
| --- | --- |

where the matrices CrrA, H0A and CxxA are estimates of covariance of the received signal, the channel and the covariance of transmitted symbols, respectively. The matrix CxxA is related to the selected modulation in the serving cell and is known by the UE through signalling. In other words,

CxxA=0.5I00I if QAM modulation is transmitted

and

CxxA=I00O0 if PAM modulation is transmitted.

#### 9.1.1.2	System-level modeling Methodology

There are two main issues in the system modelling methodology that needs to be solved. First, assuming system simulations are performed by using SINR to packet error rate mapping, the calculation of the SINR at the output of the receiver needs to be defined. Secondly, if practical non-idealities are to be taken into account, estimation errors need to be modeled.

##### Calculation of post-equalization SINR

Considering first the post-equalization SINR calculation, let us define:

- The usefull signal contribution at the output of the receiver filter:

ΨIΨQ=EdiaggH0Ax0AH0Ax0ATgT=diaggH0ACxxAH0ATgT

- The interference contribution at the output of the receiver filter:

ξIξQ=ξ1⋮ξn

where

ξi=Egi,:H0Ax0A'H0Ax0A'T+ nAnATgTi,:

=gi,:H0ACxx,iA'H0ATgTi,:+gi,:CnnAgTi,:

The matrix Cxx,iA' equals the covariance of the transmitted signal where the i:th diagonal element is zeroed out like in MIMO systems. Finally, the SINR of the received and filtered output symbol in the case of real valued modulation where e.g. a PAM modulated symbol is mapped to the I branch equals

| ΓI=ΨIξI | (x2) |
| --- | --- |

where the division is computed element wise. If complex valued modulation e.g. QAM have been transmitted one can combine the real and imaginary part and obtain the SINR of the complex symbol (assuming element wise division) as

| ΓIQ=ΨI+ΨQξI+ξQ | (x3) |
| --- | --- |

Because e.g. for scalar  x0= Ix0+jQx0, x0=xr+nr+jxq+nq and

Ex0x0*=Exr+nr+jxq+nqxr+nr+jxq+nq*

= Exrxr*+Exqxq*+Enrnr*+Enqnq*.

##### Modeling of Estimation Errors

As mentioned in the previous section, information on matrices CrrA, H0A and CxxA is required by the receiver. The matrix CxxA depends on the applied modulation format at the serving cell, which is signaled over PDCCH. Hence, no additional error model is needed in relation to CxxA. The channel estimate H0A can be estimated in complex domain as in the conventional system e.g. from DM-RS. Hence, generation of channel estimation error is not discussed in this paper. However, the covariance of the received signal CrrA needs to be calculated in augmented real valued format. In this document we assume DM-RS based covariance estimation.

Earlier studies, as in [y3], have used estimation error model based on Wishart random matrices and the extension to augmented matrices is now discussed further.

Assuming CrrA=H0ACxxAH0AH+CiiA one can focus on estimating the covariance of interference CiiA from the residual signal obtained by subtracting the serving cell DM-RS pilot replica p from the received signal. In other words, the residual signal at the DM-RS RE positions is calculated following a sample covariance matrix calculation i.e.

CiiA=1NDM-RSrAn-H0AIpnQpn,…, rAn+NDM-RS-H0AIpn+NDM-RSQpn+NDM-RS

×rAn-H0AIpnQpn,…, rAn+NDM-RS-H0AIpn+NDM-RSQpn+NDM-RST

As also discussed in [y3] the matrix CiiA may be modelled as a Wishart distributed random matrix i.e. CiiA~WnNDM-RS,CiiA which is based on the ideal information on the interference covariance matrix CiiA=CrrA-H0ACxxAH0AT which should easily be available in system level simulators.

The Wishart distributed random matrix can be generated from ideal information i.e. CiiA=1NDM-RSQAATQTwhere [y3]

| Q=cholCiiA A=c10n21c20…00…0n31n32⋮⋮nN1nN2c2…0⋮⋱⋮nN3…cN ci=χ2NDM-RS-i-1 nij~N(0,1) | (x4) |
| --- | --- |

Note that the matrix is real valued in this case.

#### 9.1.1.3	Summary of system modelling

In this section we summarize the steps in the system modelling methodology to calculate modulation specific post-equalization SINR at the output of the WLMMSE-IRC receiver including practical non-idealities:

- Generate serving cell channel matrix H0A and interference covariance matrix CiiAsamples

- Add channel estimation error and calculate H0A.

- Generate estimated interference covariance matrix CiiAby using the Wishart random matrices as in (x4).

- Calculate WLMMSE-IRC receiver filter as in (x1).

- Calculate receiver ouput SINR as in (x2) or (x3) depeding on the scheduled modulation.

< end of text proposal >