3GPP TSG RAN WG4 Meeting #70	R4-141252

Prague, Czech Republic, 10th – 14th Feb., 2014

- Agenda Item:	9.1

- Source:	LG Electronics

- Title:	TP for UE TX requirements for S-band UE

Document for:	Approval

- Introduction

In the RAN4 #69 meetings, the way forward on the channel arrangements are agreed and interested companies encouraged to provide the required A-MPR and RB restriction simulation results to protect band 34 when S-band and Band 34 deployed in same geometrical regions.

In this contribution, we provide the revised UE transmitter RF requirements based on the agreed way forwards.

- References

- R4-137089, “Way forward on how to define a new band for MSS 2GHz,” NTT DOCOMO, Qualcomm, Nokia corporation, Ericsson, KT

- R4-137153, “Way forward on MSS co-existence with Band 34,” Ericsson, NTT DOCOMO

- TS 36.101 v12.2.0

- R4-135186, “UE coexistence analysis to protect Band 34,” LG Electronics

Text proposal for TR 36.861

********************** Start of text proposal to TR 36.861 chapter 8 ***********************

- 8	Study of E-UTRA specific issues

For the channel arrangements for S-band UE, RAN4 agreed to study the restricted channel arrangements as follow,

- UL: (2010 – X) to 2010 MHz, DL: (2200 – X) to 2200 MHz

- 30 MHz ≦ X ≦ 70 with 10 MHz step

- The channel arrangement for the fully overlapped with Band1 is not a candidate band due to the duplexer characteristics.

Note that the channel arrangement for X = 30 is the same as the one originally proposed in the SI(FS_LTE_1980_2170_Korea) [5].

- 8.1    UE Tx RF Requirements

- 8.1.1 	Spurious emission for UE co-existence

When S-band UE is allocated in the same country and region where Band34 UE is deployed, RAN4 can define network signalling such as NS_XX to apply A-MPR mask with guard band to protect band 34 for S-band UE. SE requirements for UE-to-UE coexistence can be defined as in Table 8.1.1-1. The SE requirements level and detail test methodology to protect band 34 are for further study.

Table 8.1.1-1: Spurious emission band UE co-existence of S-band UE for Korea

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Level (dBm) | Bandwidth (MHz) | Comment |
| XX | E-UTRA Band 1,3,5,7,8,26,27,XX,40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | 2010 | - | 2025 | TBD | TBD | 1 |
|  |  |  | - |  |  |  |  |
| NOTE 1: This requirement is only applicable for S-band UE, when Band34 and S-band are deployed in same region. |  |  |  |  |  |  |  |

[Editorial Note]: The spurious emission band UE coexistence of S-band UE for region3 will be added in future.

********************** End of text proposal to TR 36.861 Chapter 8 ***************************