3GPP TSG-RAN WG4 Meeting #70bis                                                                   R4-141828

# San Jose Del Cabo, Mexico, 31 Mar - 4 Apr, 2014

Source:	LG Electronics

Title:	Link performance for NAICS receiver with blind detection under CRS based transmission mode

Agenda item:	7.14.2.1

Document for:	Discussion

Introduction

In last RAN Plenary, Work Item for NAICS [1] was approved. RAN4’s objective is to identify and agree on the parameter combinations that could be jointly blind detected jointly, including if under any subset restriction for any parameters. Considering parameters for blind detection are as follows:

    - Presence or absence of interference

    - Transmission modes (TM)

    - For DMRS-based TMs: DMRS ports, modulation order, Virtual cell ID, nSCID, Cell ID, CRS ports, and MBSFN pattern

    - For CRS-based TMs: PMI, RI, modulation order, Cell ID, CRS ports, and MBSFN pattern, ρA

    - CFI (if not coordinated and required by receiver implementation)

To cancel or suppress aggressor PDSCH, higher-layer signaling of parameters are beneficial to reduce the blind detection complexity or performance degradation and desirable blind detection parameters to reduce scheduling restriction and signaling overhead were itemized in last RAN1 meeting [2]. In this contribution, we discuss blind detection for dynamic parameters and provide link level performance for blind detection under CRS based transmission mode.

Performance of blind detection

For CRS based transmission mode, some candidate parameters for blind detection are as follows:

    - Presence or absence of interference

    - Transmission modes (TM)

    - PMI

    - RI

    - Modulation order

    - PA

In this contribution, we evaluate blind detection performance for combination of transmission mode (TM), PMI, RI, and modulation order under follow interference conditions:

| TM | Serving MCS | Interference MCS |  | INR |  | Interference pattern |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Interference 1 | Interference 2 | Interference 1 | Interference 2 | Interference 1 | Interference 2 |
| 4 | 5 | 5 | 5 | 7.68 | 2.16 | ON | ON/OFF |
|  |  | 14 | 14 |  |  |  |  |
|  |  | 25 | 25 |  |  |  |  |
|  | 14 | 5 | 5 |  |  |  |  |
|  |  | 14 | 14 |  |  |  |  |
|  |  | 25 | 25 |  |  |  |  |

Those parameters are blindly detected per PRB pair. By working assumption in last RAN4 meeting, maximum 3 layers and one interference cell to cancel/suppress PDSCH interference are considered. For TM blind detection, TM2, 3, 4, and 6 are considered. To reduce UE complexity, reduced TM set is considered, i.e., separated CRS based TM (TM2, 3, 4, and 6) and DMRS based TM (TM8, 9, 10).

Figure 2-1 ~ 2-3 show throughput performance for blind detection for SLIC receiver under rank one.  When interference pattern is [ON OFF], blind detection performances are degraded by 1.5~2.5dB in comparison with genie-aid performance under QPSK, 16QAM, and 64QAM modulation order. However, when interference pattern is [ON ON], blind detection performances are similar to those for baseline IRC receiver if we cancel only one interference cell based on RAN4 working assumption. So we need to consider two interference cell cancellations for performance improvement.

In general, performance for joint blind detection of dynamic parameters shows relatively large degradation in comparison with those with single parameter detection. To improve performance with joint blind detection, increase of NAICS receiver complexity is inevitable. From UE implementation perspective, increased complexity and corresponding power consumption are main concerning points.

- Observation1: Blind detection performances for combination of dynamic parameters such as TM, PMI, RI, and modulation order are degraded by 1.5~2.5dB in comparison with genie-aid performance under interference patter [ON OFF].

-  Observation2: To improve performance gain for NAICS receiver, two cell interference cancellations need to be considered.

Figure 2-1 SLIC receiver throughput performance with rank one and QPSK for interference cells

Figure 2-2 SLIC receiver throughput performance with rank one and 16QAM for interference cells

Figure 2-3 SLIC receiver throughput performance with rank one and 64QAM for interference cells

Discussion for further simulation assumption

To analyze and align company observations on blind detection performance for dynamic parameters, common simulation assumption should be considered. Based on agreement in the last meeting, we propose the following simulation assumptions for fair comparison among companies from next meeting (Table 3-1~3-3).

Table 3-1 Simulation assumption for CRS-based transmission (Common parameters)

| Common Parameters |  | Unit | Serving | I1 | I2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 | -3 | -3 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cell Id |  |  | 0 | 6 | 1 |
| Propagation channel |  |  | EPA5 |  |  |
| Correlation and antenna configuration |  |  | 2X2 Low |  |  |
| MCS |  |  | 5 | 5 | 5 |
|  |  |  |  | 14 | 14 |
|  |  |  | 14 |  |  |
|  |  |  |  | 25 | 25 |
| Transmission mode |  |  | 4 | 4 | 4 |
| PDSCH Allocation |  | PRBs | 1 (Note1) | 50 | 50 |
| Interference pattern |  |  |  | ON | ON |
|  |  |  |  |  | OFF |
| Number of control OFDM symbols |  |  | 2 | 2 | 2 |
| Note1: to avoid channel effect, middle PRB should be considered. For example, selected PRB index 20 among PRB index 0~49. Note2: the precoder shall be updated in each TTI (1 ms granularity) Note3: no timing and frequency offset |  |  |  |  |  |

For low (5~25%) SINR geometry and RU 40%,

Table 3-2 Simulation parameters for low SINR geometry

| Parameters | Unit | Serving | I1 | I2 |
| --- | --- | --- | --- | --- |
| E/Noc | dB | Sweep E/Noc to cover SINR range | 50% I1/Noc | Conditional median I2/Noc |
|  |  |  | 80% I1/Noc | Conditional median I2/Noc |
| Rank |  | 1 | 1 | 1 |
|  |  |  | 2 | 2 |
| Note1: Both layers of rank2 transmissions use the same MCS |  |  |  |  |

For medium (40~60%) SINR geometry and RU 40%,

Table 3-3 Simulation parameters for medium SINR geometry

| Parameters | Unit | Serving | I1 | I2 |
| --- | --- | --- | --- | --- |
| E/Noc | dB | Sweep E/Noc to cover SINR range | 50% I1/Noc | Conditional median I2/Noc |
| Rank |  | 2 | 1 | 1 |
| Note1: Both layers of rank2 transmissions use the same MCS |  |  |  |  |

Based on above simulation assumption, we suggest that blind detection performance for combination of dynamic parameters is evaluated, and SNR gain and loss at 70% of maximum throughput in comparison with baseline IRC and genie-aid NAICS throughput can be considered for evaluation metric.

Conclusion

In this contribution, we provide link level simulation results for blind detection performance under CRS-based transmission mode, and our observations are as follows:

- Observation1: Blind detection performances for combination of dynamic parameters such as TM, PMI, RI, and modulation order are degraded by 1.5~2.5dB in comparison with genie-aid performance under interference patter [ON OFF].

-  Observation2: To improve performance gain for NAICS receiver, two cell interference cancellations need to be considered.

And we propose common simulation assumptions to have fair comparison among companies on blind detection performance for dynamic parameters as Table 3-1~3-3. For evaluation metric, SNR gain and loss at 70% of maximum throughput in comparison with baseline IRC and genie-aid NAICS throughput can be considered.

Reference

- RP-140519, “New work item proposal for network assistance interference cancellation and suppression for LTE,” MediaTek Inc.

- 3GPP TR 36.866 V2.0.0.