3GPP TSG-RAN WG4 (Radio) Meeting #70bis	R4-141870

San Jose Del Cabo, Mexico, 31 March – 4 April 2014

Agenda Item:	7.42.1

Source: 	Alcatel-Lucent

Title: 	TP for TR 36.835: Specific BS RF requirements for LTE Advanced 3 Band Carrier Aggregation (3DL/1UL)

Document for:	Approval

# 1.	Introduction

Numerous WI proposals to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) have been approved in RAN. One of the objectives of these WI is to specify the band-combination specific RF requirements for the corresponding 3 band carrier aggregation (CA) scenarios.

In this contribution, we discuss the BS RF requirements for supporting 3 band CA (3DL/1UL), and provide we provide a text proposal to record the findings in the Technical Report for 3 Band Carrier Aggregation with Single Uplink [1].

# 2.	Discussion

For BS capable of multi-carrier and/or CA operation in contiguous and non-contiguous spectrum in single band, the core and test requirements currently specified in the BS specifications (TS 36.104 and TS 36.141 [3, 4]) are agnostic to the number of CC in the operating band or each of the two sub-blocks within the operating band supported by the BS in intra-band or inter-band CA operation. This can be seen by studying the test configurations defined for multi-carrier and/or CA operation in contiguous and non-contiguous spectrum in single band in sub-clauses 4.10.1, 4.10.2 and 4.10.3 in TS 36.141.

Similarly, for BS capable of multi-band operation, the core and test requirements currently specified in the BS specifications (TS 36.104 and TS 36.141 [3, 4]) are agnostic to the number of CC in each operating band or each of the two sub-blocks within each operating band supported by the BS in intra-band or inter-band CA operation. This can be seen by studying the test configurations defined for multi-carrier and/or CA operation in contiguous and non-contiguous spectrum in multi-band in sub-clauses 4.10.4 and 4.10.5 in TS 36.141. Also the core and test requirements are agnostic to the number of operating bands supported by the BS, except in subclause 4.7.1 in TS 36.141 (extracted below) where the ‘RF bandwidth position for multi-carrier and/or CA testing’ is specified only for ‘BS capable of dual-band operation’.

<Start of extraction>

For BS capable of dual-band operation, unless otherwise stated, the test shall be performed at BRFBW_T’RFBW and B’RFBW_TRFBW defined as following:

BRFBW_ T’RFBW: the RF bandwidths located at the bottom of the supported frequency range in the lower operating band and at the highest possible simultaneous frequency position, within the maximum radio bandwidth, in the upper operating band.

B’RFBW_TRFBW: the RF bandwidths located at the top of the supported frequency range in the upper operating band and at the lowest possible simultaneous frequency position, within the maximum radio bandwidth, in the lower operating band.

NOTE: BRFBW_T’RFBW = B’RFBW_TRFBW = BRFBW_TRFBW when the declared maximum radio bandwidth spans both operating bands. BRFBW_TRFBW means the RF bandwidths are located at the bottom of the supported frequency range in the lower operating band and at the top of the supported frequency range in the upper operating band.

<End of extraction>

In order to cover the testing for BS capable of 3 bands operation, the section above should be changed as follow:

<Start of extraction>

For BS capable of multi-band operation, unless otherwise stated, the test shall be performed at BRFBW_T’RFBW and B’RFBW_TRFBW defined as following:

BRFBW_ T’RFBW: the RF bandwidths located at the bottom of the supported frequency range in the lowest operating band and at the highest possible simultaneous frequency position, within the maximum radio bandwidth, in the highest operating band.

B’RFBW_TRFBW: the RF bandwidths located at the top of the supported frequency range in the highest operating band and at the lowest possible simultaneous frequency position, within the maximum radio bandwidth, in the lowest operating band.

NOTE: BRFBW_T’RFBW = B’RFBW_TRFBW = BRFBW_TRFBW when the declared maximum radio bandwidth spans all operating bands supported by the BS. BRFBW_TRFBW means the RF bandwidths are located at the bottom of the supported frequency range in the lowest operating band and at the top of the supported frequency range in the highest operating band.

<End of extraction>

Otherwise, the current requirements can be applied for supporting 3 band CA (3DL/1UL). Moreover, a new table should be added in TS 36.104 and TS 36.141 (as shown below) to list the 3 band CA (3DL/1UL) supported in the specifications.

Table 5.5-5. 3 band carrier aggregation (3DL/1UL) bands

| CA Band | E-UTRA operating band |
| --- | --- |
| CA_x-y-z | x |
|  | y |
|  | z |

# 3.	Conclusion

The current requirements specified in TS 36.104 and TS 36.141 for BS capable of multi-carrier and/or CA operation in contiguous or non-contiguous spectrum in single or multi-band are agnostic to the number of CC in each operating band or each of the two sub-blocks within each operating band supported by the BS in intra-band or inter-band CA operation, and thus can be applied for supporting 3 band CA (3DL/1UL), except in subclause 4.7.1 in TS 36.141 where the ‘RF bandwidth position for multi-carrier and/or CA testing’ should be changed to cover the testing for BS capable of 3 bands operation.

Moreover, a new table should be added in TS 36.104 and TS 36.141 to list the 3 band CA (3DL/1UL) supported in the specifications.

# References

[1]	R4-137075, “Draft New Technical Report 'LTE Advanced 3 Band Carrier Aggregation (3DL/1UL)'”, AT&T.

[2]	3GPP TS 36.104 v12.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.

[3]	3GPP TS 36.141 v12.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

# Text Proposal

<Start of change>

## 5.3	Specific BS RF requirements

The current requirements specified in TS 36.104 and TS 36.141 for BS capable of multi-carrier and/or CA operation in contiguous or non-contiguous spectrum in single or multi-band are agnostic to the number of CC in each operating band or each of the two sub-blocks within each operating band supported by the BS in intra-band or inter-band CA operation, and thus can be applied for supporting 3 band CA (3DL/1UL), except in subclause 4.7.1 in TS 36.141 where the ‘RF bandwidth position for multi-carrier and/or CA testing’ should be changed (as shown below) to cover the testing for BS capable of 3 bands operation.

<Start of extraction>

For BS capable of multi-band operation, unless otherwise stated, the test shall be performed at BRFBW_T’RFBW and B’RFBW_TRFBW defined as following:

BRFBW_ T’RFBW: the RF bandwidths located at the bottom of the supported frequency range in the lowest operating band and at the highest possible simultaneous frequency position, within the maximum radio bandwidth, in the highest operating band.

B’RFBW_TRFBW: the RF bandwidths located at the top of the supported frequency range in the highest operating band and at the lowest possible simultaneous frequency position, within the maximum radio bandwidth, in the lowest operating band.

NOTE: BRFBW_T’RFBW = B’RFBW_TRFBW = BRFBW_TRFBW when the declared maximum radio bandwidth spans all operating bands supported by the BS. BRFBW_TRFBW means the RF bandwidths are located at the bottom of the supported frequency range in the lowest operating band and at the top of the supported frequency range in the highest operating band.

<End of extraction>

Moreover, a new table should be added in TS 36.104 and TS 36.141 (as shown below) to list the 3 band CA (3DL/1UL) supported in the specifications.

Table 5.5-5. 3 band carrier aggregation (3DL/1UL) bands

| CA Band | E-UTRA operating band |
| --- | --- |
| CA_x-y-z | x |
|  | y |
|  | z |

<End of change>