3GPP TSG-RAN WG4 RAN4 #70Bis                                      R4-142054

San Jose del Cabo, Mexico, 31 Mar - 4 Apr, 2014

Agenda item:	7.14.2.1

Source: 	Samsung

Title:			   On blind detection feasibility of parameter combination for CRS-based TM

Document for:	Discussion

# 1	Introduction

In 3GPP RAN #63 meeting, it was agreed to set up a work item targeted on specifying advanced non-linear receiver in Rel-12, based on the extensive study during study item. Consequently, RAN4 are expected to identify and agree on the parameter combinations that could be blindly detected, including:

- Presence or absence of interference

- Transmission modes (TM)

- For DMRS-based TMs: DMRS ports, modulation order, Virtual cell ID, nSCID, Cell ID, CRS ports, and MBSFN pattern

- For CRS-based TMs: PMI, RI, modulation order, Cell ID, CRS ports, and MBSFN pattern, ρA

- CFI (if not coordinated and required by receiver implementation)

In this paper, we provide our views and analysis on dynamic parameters for CRS based TM, including

- Presence or absence of interference

- For CRS-based TMs: PMI, RI, modulation order

The analysis on dynamic parameters for DMRS-based TM are provided in companion papers[2].

# Simulation Assumptions

In this contribution, we focus on the blind detection performance of R-ML receiver. Especially, we evaluate the performance loss caused by

- Blind detection of interference presence or absence

For blind detection of interference presence or absence, the estimation can be done based on the measured covariance matrix of received signal.

Note that for the interference ON/OFF detection, it is not only to evaluate the performance loss caused by ON->OFF misdetection for R-ML receiver when interference presence, but also necessary to guarantee enough good performance of false alarm, i.e. OFF->ON misdetection when interference is absence.

- Blind detection of TF (Transmission Format)

For blind detection of TF (including RI, PMI and Modulation), a similar classification algorithm as in [2] can be applied.

Furthermore, we evaluated the performance under TM4 with cell ID [0, 6, 1] with a set of interference profiles listed below. Detailed assumptions are shown in Annex.

- Interference profiles:  (1) INR1 = 7.77dB, INR2 = 2.29dB; (2) INR1 = 13.91dB, INR2 = 3.34dB

- Interference combinations:

Table 1 Simulated combinations

| TM (Serving/I1/I2) | Interference cell RI | Interference cell MCS | Serving cell RI | Serving cell MCS |
| --- | --- | --- | --- | --- |
| TM4/TM4/TM4 ON/ON/ON pattern | 1 | 5 | 1 | {5} |
|  |  |  |  | {14} |
|  | 1 | 14 | 1 | {5} |
|  |  |  |  | {14} |
|  | 2 | 5 | 1 | {5} |
|  |  |  |  | {14} |
|  | 2 | 14 | 1 | {5} |
|  |  |  |  | {14} |

# Simulation Results

In this contribution, we focus on the performance evaluation of R-ML receiver without losing the generality. Especially, three types of R-ML receiver are evaluated:

- Genie-aided R-ML receiver

- R-ML receiver with blind detection of RI, PMI and modulation format, termed as TF(Transmission Format) in short  hereafter

- R-ML receiver with joint blind detection of TF and interference presence

- Additionally, the performance of MMSE-IRC receiver is also presented as a baseline receiver.

The throughput performances of all cases are listed in Figure 1 to Figure 4.

- Figure 1: Performance under Rank 1 interference and INR1 = 7.77dB, INR2 = 2.29dB

- Figure 2: Performance under Rank 2 interference and INR1 = 7.77dB, INR2 = 2.29dB

- Figure 3: Performance under Rank 1 interference and INR1 = 13.91dB, INR2 = 3.34dB

- Figure 4: Performance under Rank 2 interference and INR1 = 13.91dB, INR2 = 3.34dB

Figure 1: Performance under Rank 1 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 2: Performance under Rank 2 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 3: Performance under Rank 1 interference and INR1 = 13.91dB, INR2 = 3.34dB

Figure 4: Performance under Rank 2 interference and INR1 = 13.91dB, INR2 = 3.34dB

# Observations

Based on the simulation results presented in Section 3, the performance for different receivers under different interference profiles are summarized in Table 1 . Furthermore, the average performance gain is also provided compared with MMSE-IRC receiver.

Table 1: Required SNR @ 70% Throughput [dB]

| INF Level 1 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} | AVG Gain | Max Gain |
| --- | --- | --- | --- | --- | --- | --- |
| MMSE | 5.5 | 5.6 | 12.3 | 12.3 |  |  |
| R-ML without BD | 2.1 | 3.7 | 9.9 | 11.0 | 2.2 | 3.5 |
| R-ML with TF BD (1RB) | 2.8 | 5.0 | 10.8 | 12.2 | 1.2 | 2.7 |
| R-ML with TF+INF ON/OFF BD (1RB) | 3.2 | 5.1 | 11.1 | 12.3 | 1.0 | 2.3 |
| R-ML with TF BD (3RB) | 2.3 | 4.5 | 10.2 | 11.8 | 1.7 | 3.3 |
| R-ML with TF+INF ON/OFF BD (3RB) | 2.8 | 4.7 | 10.6 | 11.9 | 1.4 | 2.8 |
| INF Level 1 + Rank 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} | AVG Gain | Max Gain |
| MMSE | 5.5 | 5.5 | 12.3 | 12.3 |  |  |
| R-ML without BD | 4.5 | 5.3 | 11.9 | 12.3 | 0.4 | 1.0 |
| R-ML with TF BD (1RB) | 5.8 | 6.5 | 13.0 | 13.3 | -0.7 | -0.2 |
| R-ML with TF+INF ON/OFF BD (1RB) | 5.8 | 6.5 | 13.0 | 13.2 | -0.7 | -0.2 |
| R-ML with TF BD (3RB) | 5.5 | 6.3 | 12.8 | 13.2 | -0.5 | 0.1 |
| R-ML with TF+INF ON/OFF BD (3RB) | 5.4 | 6.3 | 12.7 | 13.1 | -0.5 | 0.1 |
| INF Level 2 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} | AVG Gain | Max Gain |
| MMSE | 9.4 | 9.4 | 16.0 | 16.0 |  |  |
| R-ML without BD | 1.9 | 4.1 | 9.3 | 12.3 | 5.8 | 7.5 |
| R-ML with TF BD (1RB) | 2.3 | 5.0 | 10.0 | 13.8 | 5.0 | 7.2 |
| R-ML with TF+INF ON/OFF BD (1RB) | 3.5 | 5.8 | 11.5 | 14.5 | 3.9 | 6.0 |
| R-ML with TF BD (3RB) | 2.0 | 4.4 | 9.4 | 12.9 | 5.6 | 7.4 |
| R-ML with TF+INF ON/OFF BD (3RB) | 3.2 | 5.2 | 10.8 | 13.6 | 4.5 | 6.2 |
| INF Level 2 + Rank 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} | AVG Gain | Max Gain |
| MMSE | 9.3 | 9.3 | 15.9 | 15.9 |  |  |
| R-ML without BD | 4.0 | 8.0 | 13.2 | 15.6 | 2.4 | 5.4 |
| R-ML with TF BD (1RB) | 5.3 | 9.8 | 15.0 | 16.9 | 0.8 | 4.1 |
| R-ML with TF+INF ON/OFF BD (1RB) | 5.3 | 9.8 | 15.0 | 16.9 | 0.8 | 4.1 |
| R-ML with TF BD (3RB) | 4.4 | 9.3 | 13.8 | 16.6 | 1.6 | 4.9 |
| R-ML with TF+INF ON/OFF BD (3RB) | 4.4 | 9.3 | 13.8 | 16.6 | 1.6 | 4.9 |

Based on the results, it is observed that for TM4 2Tx case,

- Compared with genie-aided R-ML receiver, blind detection of RI, PMI and modulation format suffer acceptable performance loss, i.e. up to 0.9dB in terms of average performance loss when blind detection granularity is 3RB, under all simulated interference scenario.

- On top of modulation format blind detection, the additional performance loss caused by blind detection of interference presence is typically less than 0.5dB in most cases.

- Regarding blind detection granularity, 3RB granularity shows noticeable performance gain (at around 0.5dB~1.0dB) in most cases over 1RB granularity.

- In addition, Figure 5 provides the results when both interference cell are turn off. The results confirm no false alarm issue (i.e. OFF->ON misdetection) for interference absence detection.

- With joint blind detection of RI, PMI and modulation with 3RB granularity, R-ML receiver provides 1.5dB~4.5dB average performance gain for high interference level and low interference level with rank 1 cases, compared with MMSE-IRC receiver.

  - Noted, because both interference cell are turn off in simulation, the interference level is underestimated by UE, i.e. interference level seen by UE could be even higher in realistic network.

  - For low interference level with rank 2, there is 0.5dB performance loss compared with MMSE-IRC receiver.

Figure 5: Performance when both interference cell are turn off

Based on the analysis above, our proposals are:

Proposal 1: For TM4 case with 2Tx case, the joint blind detection of RI, PMI, modulation and interference presence, R-ML receiver with blind detection still provide promising performance gain in most cases.

Proposal 2: For blind detection granularity, blind detection granularity of multiple RB (e.g. 3RBs) could be considered to improve the blind detection performance and to reduce UE implementation complexity.

Proposal 3: It is beneficial if network could semi-statically coordinate to avoid scheduling high rank and high modulation interference for NAICS receiver.

# Conclusion

In this contribution, we investigate the performance of blind detection of dynamic parameters for TM4 mode,

Based on our results and analysis above, our proposals are:

Proposal 1: For TM4 case with 2Tx case, the joint blind detection of RI, PMI, modulation and interference presence, R-ML receiver with blind detection still provide promising performance gain in most cases.

Proposal 2: For blind detection granularity, blind detection granularity of multiple RB (e.g. 3RBs) could be considered to improve the blind detection performance and to reduce UE implementation complexity.

Proposal 3: It is beneficial if network could semi-statically coordinate to avoid scheduling high rank and high modulation interference for NAICS receiver.

# Reference

- R4-142055, On blind detection feasibility of parameter combination for DMRS-based TM, Samsung

- D. Bai, et al, “Near ML Modulation Classification”, in Proc. of IEEE VTC 2012, pp.1-5.

# Annex

Table 5-1: Simulation Assumptions

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| RB allocation | 6 |
| Cell ID | [0, 6, 1] |
| Transmission mode on Serving cell | TM4 |
| Transmission mode on Interference cell | TM4 |
| MIMO configuration | 2x2 and low correlation |
| Channel model and Doppler frequency for target and interference cells | EPA 5Hz Use different channel seed for between cells |
| CRS configuration | 2 CRS ports. CRS is colliding between serving cell and interference cells |
| PA | -3dB |
| PB | 0dB |
| CSI-RS configuration | None |
| Channel Estimation | Realistic Channel Estimation |
| PMI | Random PMI |
| H-ARQ | 8 HARQ processes |
| PCFICH | CFI = 2 |
| PCFICH/PDCCH detection | Not considered |