3GPP TSG-RAN WG4 (Radio) Meeting #70bis	R4-142202

San Jose Del Cabo, Mexico, 31 March – 4 April 2014

Agenda Item:	7.21.2

Source: 	Alcatel-Lucent

Title: 	TP for TR 36.833-5-40: Specific BS RF requirements for LTE Advanced Carrier Aggregation in Band 40 for 3DL

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced Carrier Aggregation in Band 40 for 3DL was approved in RAN#63 [1]. One of the objectives of this WI is to specify Specify the band specific RF requirements for band 40 intra-band contiguous carrier aggregation for 3-DL CCs [1].

In this contribution, we discuss the BS RF requirements for band 40 intra-band contiguous carrier aggregation for 3-DL CC, and provide a text proposal to record the findings in the TR36.833-5-40 [2].

# 2.	Discussion

For BS capable of multi-carrier and/or CA operation in contiguous spectrum in single band, the core and test requirements currently specified in the BS specifications (TS 36.104 and TS 36.141 [3, 4]) are agnostic to the number of CC supported by the BS in intra-band contiguous CA operation. This can be seen by studying the test configurations defined for multi-carrier and/or CA operation in contiguous spectrum in single band in sub-clauses 4.10.1 and 4.10.2 in TS 36.141. Also Table 5.5-2 in TS 36.104 and TS 36.141 (copied below) do not specify the number of CC in the intra-band contiguous carrier aggregation bands.

Table 5.5-2. Intra-band contiguous carrier aggregation bands

| CA Band | E-UTRA operating band |
| --- | --- |
| CA_1 | 1 |
| CA_7 | 7 |
| CA_38 | 38 |
| CA_40 | 40 |
| CA_41 | 41 |

Moreover, it was agreed in RAN4 and recorded in Annex A.2 of TR 36.808 [5] (extracted below) that ‘BS requirements will be developed for both 50 MHz and 40 MHz aggregated channel BWs for the CA_40 scenario in release-10 timeframe’. Thus Band 40 intra-band contiguous carrier aggregation for 3-DL and 2-DL CCs have already been supported in the BS specifications since Release 10.

<Start of extraction>

# A.2	Intra – band Contiguous CA

Intra-band contiguous scenarios for the Rel-10 CA WI are shown in Table A.2-1.

Table A.2-1 Intra-band contiguous CA

| E-UTRA CA Band | E-UTRA operating Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UE transmit / BS receive |  |  | Channel BW MHz | UE receive / BS transmit |  |  | Channel BW MHz |  |
|  |  | FUL_low (MHz) – FUL_high (MHz) |  |  |  | FDL_low (MHz) – FDL_high (MHz) |  |  |  |  |
| CA_40 | 40 | 2300 | – | 2400 | 501 | 2300 | – | 2400 | 501 | TDD |
| CA_1 | 1 | 1920 | – | 1980 | 40 | 2110 | – | 2170 | 40 | FDD |
| Note 1: BS requirements will be developed for both 50 MHz and 40 MHz aggregated channel BWs for the CA_40 scenario in release-10 timeframe |  |  |  |  |  |  |  |  |  |  |

<End of extraction>

Therefore, the current requirements can be applied for 3-CC contiguous intra-band DL CA in Band 40, and hence no change in the BS specifications is required.

# 3.	Conclusion

The current requirements specified in TS 36.104 and TS 36.141 for BS capable of multi-carrier and/or CA operation in contiguous spectrum in single band are agnostic to the number of CC supported by the BS in intra-band contiguous CA operation, and thus can be applied for 3-CC contiguous intra-band DL CA in Band 40. Indeed Band 40 intra-band contiguous carrier aggregation for 3-DL and 2-DL CCs have already been supported in the BS specifications since Release 10. Hence no change in the BS specifications is required.

# References

[1]	RP-140453, “New WI proposal: LTE Advanced intra-band contiguous Carrier Aggregation in Band 40 for 3DL”, CMCC.

[2]	R4-141561, “TR 36.833-5-40 skeleton”, CMCC.

[3]	3GPP TS 36.104 v12.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.

[4]	3GPP TS 36.141 v12.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[5]	3GPP TS 36.808 v10.1.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Carrier Aggregation; Base Station (BS) radio transmission and reception”.

# Text Proposal

<Start of change>

## 5.3	Specific BS RF requirements

The current requirements specified in TS 36.104 and TS 36.141 for BS capable of multi-carrier and/or CA operation in contiguous spectrum in single band are agnostic to the number of CC supported by the BS in intra-band contiguous CA operation, and thus can be applied for 3-CC contiguous intra-band DL CA in Band 40. Indeed Band 40 intra-band contiguous carrier aggregation for 3-DL and 2-DL CCs have already been supported in the BS specifications since Release 10. Hence no change in the BS specifications is required.

<End of change>