3GPP TSG-RAN Working Group 4 Meeting #72                    R4-144513

Dresden, Germany, 18-22 Aug, 2014

Source:	Samsung

Title:	View on specification changes for introducing LTE_CA_B1_B41_B41

Agenda item:	7.7.5.3

Document for:	Discussion

- 1	Introduction

According to WF[1], the objective of TDD-FDD CA WI is revised to completing since one CA combination case of Band 8 + Band 40 has been completed, but Band 1 + Band 41, one of the other example combinations has not been completed. For those band combinations not completed by RAN#64, their WI should be newly proposed at RAN#64, and be handled in Rel-13 specification. Then in RAN#64, the new WI was approved for LTE advanced 3 band carrier aggregation (3DL/1UL) of Band 1, Band 41 and Band 41[2], in order to continue RAN4 work for Band 1 + Band 41.

TS36.101 Rel-12 will be finished this year and then Rel-13 specification will be created. So in this contribution, specification changes will be summarized and discussed for CA_1A_41A_41A and CA_1A-41A.

- 2	Discussion

2.1 Operating bands for CA

Operating bands for B1 and B41 have been introduced in previous specification, but how to introduce the operating bands of 3DL/1UL CA into specification is still in discussion. According to offline discussion, there are 2 options on how to add 3DL/1UL CA combination into specification for inter-band CA + intra-band Non-contiguous CA. And option2 is seemed to be captured in WF which will be discussed in this meeting.

Option1: Put at the end of the existing 3DL on numeric sequence

Option2: Insert in the middle of the existing 2DL tables on numeric sequence

Therefore based on option2, CA_1A_41A_41A was inserted in the middle of Inter-band CA operating bands Table(Table 2.1-1).

Table 2.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-5 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
| CA_1-8 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
| CA_1-18 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz |  |
| CA_1-19 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |
| CA_1-21 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
| CA_1-26 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz |  |
| CA_1-41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| CA_1-41-41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| CA_2-4 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz |  |
| …… |  |  |  |  |  |  |  |  |
| CA_39-41 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | 40 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz |  |

2.2 Channel bandwidths and bandwidth combinations

The CA configurations and bandwidth combination sets for CA_1A-41A_41A has been defined in WI[2]. The combinations were as shown as in Table 2.2-1.

Table 2.2-1: Supported E-UTRA bandwidths per CA configuration for CA_1A_41A_41A

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-41A-41A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |

Regarding fallback mode for 3DL/1UL CA, the bandwidth combinations for Band1 +Band 41 and Intra-band Non-contiguous Band 41 following CA_1A_41A_41A should be supported but not limited in CA_1A-41A case and CA_41A-41A case in specification. For example, Table 2.2-1 shows supported CA bandwidth combinations for CA_1A_41A_41A, and then following fallback mode, CA_1A-41A and CA_41A-41A should at least separate support bandwidth combination of (5, 10, 15, 20MHz), (5, 10, 15, 20MHz) in specification, while DL CA combinations for CA_41A-41A are already defined in Rel-12 specification, and do include above combinations as combination set 1 as shown in Table 2.2-2. If CA_1A-41A don’t support this CA combination, then have to add this bandwidth combination set as set 2 for CA_41A-41A in specification. Due to CA_1A-41A has not introduced in introduction yet, so could be introduced with CA_1A_41A_41A together with defined combination sets as shown in Table 2.2-3.

Table 2.2-2: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRACA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_2A-2A | 5, 10, 15, 20 | 5, 10, 15, 20 | 40 | 0 |
| CA_3A-3A | 5, 10, 15, 20 | 5, 10, 15, 20 | 40 | 0 |
| CA_4A-4A | 5, 10, 15, 20 | 5, 10, 15, 20 | 40 | 0 |
| CA_7A-7A | 5 | 15 | 40 | 0 |
|  | 10 | 10, 15 |  |  |
|  | 15 | 15, 20 |  |  |
|  | 20 | 20 |  |  |
| CA_23A-23A | 5 | 10 | 15 | 0 |
| CA_25A-25A | 5, 10 | 5, 10 | 20 | 0 |
| CA_41A-41A | 10, 15, 20 | 10, 15, 20 | 40 | 0 |
|  | 5, 10, 15, 20 | 5, 10, 15, 20 | 40 | 1 |

Table 2.2-3: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-5A | 1 |  |  |  | Yes |  |  | 20 | 0 |
|  | 5 |  |  |  | Yes |  |  |  |  |
| CA_1A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  | 8 |  |  | Yes | Yes |  |  |  |  |
|  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-18A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  | 18 |  |  | Yes | Yes | Yes |  |  |  |
|  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  | 18 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-19A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  | 19 |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-21A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  | 21 |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-26A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  | 26 |  |  | Yes | Yes | Yes |  |  |  |
|  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  | 26 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-41A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-41A-41A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  | 41 | See Table 2.2-2 CA_41A-41A Bandwidth combination set 1 |  |  |  |  |  |  |  |
| CA_2A-4A | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  | 4 |  |  | Yes | Yes |  |  |  |  |
| …… |  |  |  |  |  |  |  |  |  |
| CA_39A-41A | 39 |  |  |  | Yes | Yes | Yes | 40 | 0 |
|  | 41 |  |  |  |  |  | Yes |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal NOTE 4: For the CA_1A_41A_41A, only specify Band 1 as one uplink component carrier to be used in this CA combination |  |  |  |  |  |  |  |  |  |

2.3 Relaxation values

The relaxation values for CA_1A-41A and CA_1A_41A_41A mostly depend on the reference TDD-FDD RF architecture, whether using triplexer architecture or dedicated architecture as reference architecture is not decided yet, thus relaxation values are left FFS as below.

Table 2.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-5A | 1 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-8A | 1 | 0.3 |
|  | 8 | 0.3 |
| CA_1A-18A | 1 | 0.3 |
|  | 18 | 0.3 |
| CA_1A-19A | 1 | 0.3 |
|  | 19 | 0.3 |
| CA_1A-21A | 1 | 0.3 |
|  | 21 | 0.3 |
| CA_1A-26A | 1 | 0.3 |
|  | 26 | 0.3 |
| CA_1A-41A | 1 | FFS |
|  | 41 | FFS |
| CA_1A_41A_41A | 1 | FFS |
|  | 41 | FFS |
| CA_2A-4A | 2 | [0.5] |
|  | 4 | 0.5 |
| …… |  |  |
| CA_39A-41A | 39 | 04 |
|  | 41 | 04 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances above, truncated to one decimal place for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance above that applies for that operating band among the supported CA configurations NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 2.3-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-5A | 1 | 0 |
|  | 5 | 0 |
| CA_1A-8A | 1 | 0 |
|  | 8 | 0 |
| CA_1A-18A | 1 | 0 |
|  | 18 | 0 |
| CA_1A-19A | 1 | 0 |
|  | 19 | 0 |
| CA_1A-21A | 1 | 0 |
|  | 21 | 0 |
| CA_1A-26A | 1 | 0 |
|  | 26 | 0 |
| CA_1A-41A | 1 | FFS |
|  | 41 | FFS |
| CA_1A_41A_41A | 1 | FFS |
|  | 41 | FFS |
| CA_2A-4A | 2 | 0.3 |
|  | 4 | 0.3 |
| …… |  |  |
| CA_39A-41A | 39 | 0.24 |
|  | 40 | 0.24 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in intra-band and non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances in Table 7.3.1-1A, truncated to one decimal place that would apply for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance in Table 7.3.1-1A that would apply for that operating band among the supported CA configurations NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

2.4 REFSENS

Due to this WI only specify the uplink requirements for Band1 in CA_1A-41A and CA_1A_41A_41A, so the harmonic products for both cases are summarized as Table 2.4-1. It can be seen that none of the harmonic products fall into the B1 or B41 UE receiving bands. Therefore, REFSENS level of CA_1A-41A and CA_1A_41A_41A only follow REFSENS levels of single band.

Table 2.4-1: B1 + B41 harmonic products with B1 UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 |
| 41 | 2496 | 2690 | 2496 | 2690 |  |  |  |  |

- 3 Conclusion

Due to TS36.101 Rel-13 is not created yet, all related specification changes for introducing CA_1A_41A_41A are discussed in this contribution.

- References

- R4-141230, “Agreement for TDD-FDD band combinations and handling of WI”, Nokia Coporation

- RP-140963, " New Work Item proposal: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 1, Band 41 and Band 41 " KDDI