3GPP TSG-RAN WG4 Meeting #72	 R4-144520

# Dresden, Germany, 18 – 22 Aug, 2014

Agenda Item:	7.5.2

Source: 	Samsung

Title: 	Simulation results for eDL-MIMO PMI test

Document for:	Discussion

1	Introduction

Performance requirements for eDL-MIMO work item in Rel-12 were widely discussed in recently RAN4 meeting. Regarding PMI test cases, such tests were agreed in last meeting:

    - Single PMI test

- PUCCH 1-1 submode 1, EPA 5Hz, QPSK ½, XP High with Beam-S (Model given in R4-141077)

    - Multiple PMI test

- PUSCH 1-2 rank 2, EVA 5 Hz, 16 QAM ½ , XP High with Beam-S (Model given in R4-141077)

In this contribution, we provide simulation results for PMI test cases.

- Analysis

2.1 Simulation assumption

The detailed simulation assumption for single PMI test and multiple PMI test cases were agreed in [2] and [3].

2.2 Simulation results

Figure 1 and figure 2 below give absolute throughput curves vs. SNR and relative throughput ratios for single PMI test. Figure 3 and figure 4 show the performance for multiple PMI test cases.

Single PMI test

Figure 1: Throughput vs. SNR for single PMI test

Figure 2: TP ratio Vs SNR for single PMI test

Multiple PMI test

Figure 3: Throughput Vs SNR for multiple test

Figure 4: TP ratio Vs SNR for multiple PMI test

Based on the simulation results, reference SNR point and throughput ratio at reference SNR point based on legacy test metric, were summarized in table 1:

:the maximum throughput obtained at  using the precoders configured according to the UE reports, and is the throughput measured at with random precoding.

For XP High channel: corresponding to: using the precoders configured according to the UE reports capture [70] % of the maximum throughput obtained at which SNR point

Table 1: Reference SNR point and TP ratio for PMI test cases

| Reference point (relative throughput with following PMI) | Single PMI test |  | Multiple PMI test |  |
| --- | --- | --- | --- | --- |
|  | SNR [dB] | TP ratio | SNR [dB] | TP ratio |
| 60% | -5.1 | 2.9 | 6.5 | 1.5 |
| 65% | -4.7 | 2.8 | 7.2 | 1.5 |
| 70% | -4.2 | 2.7 | 7.8 | 1.5 |
| 75% | -3.7 | 2.6 | 8.4 | 1.5 |
| 80% | -3.1 | 2.4 | 8.9 | 1.5 |

- Conclusion

In this contribution, we provide simulation results for PMI test cases. Based on simulation results, we propose to take below values as minimum requirements for single PMI and multiple PMI test cases:

Proposal1: for single PMI test (FDD mode), for multiple PMI test (FDD mode).

- Reference

- R4-142361, “Simulation assumptions for eDL-MIMO single PMI test”, Samsung

- R4-142362, “Simulation assumptions for eDL-MIMO multiple PMI test”, Huawei