3GPP TSG-RAN WG4 Meeting #72	 R4-144527

# Dresden, Germany, 18 – 22 Aug, 2014

Agenda Item:	7.10.4

Source: 	Samsung

Title: 	Simulation results for TP identification based on CSI-RS

Document for:	Discussion

# 1	Introduction

A new work item regarding Small cell enhancements for E-UTRA and E-UTRAN was approved in RAN#62 plenary. A new type of discovery reference signal was designed and introduced to facilitate Cell/TP identification and small on/off operation in RAN1. In RAN4#71 RRM AH, DRS based on cell/TP identification were widely discussed. A simulation for Cell/TP identification based on CRS or CSI-RS was agreed in [1].

In this contribution, we provide initial evaluation results for CSI-RS based on TP identification.

# Analysis

## 2.1 Simulation assumption

The detailed simulation assumption for CSI-RS acquisition was agreed in [1], as shown in table below.

Table 1: Simulation parameters for CSI-RS acquisition

| Parameter | Unit | TP1 | TP2 | TP3(Desired TP) |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 | Channel 1 |
| Data and Control PSD relative to RS PSD | dB | 0 | 0 | 0 |
| CSI-RS PSD relative to RS PSD | dB | 0 | 0 | 0 |
| Carrier frequency | GHz | 3.5 | 3.5 | 3.5 |
| Number of RB’s |  | 6RB, 50RB |  |  |
| RB Utilization | % | 100 | 100 | 100 |
| Data Modulation | - | QPSK | QPSK | QPSK |
| Frame Structure Type | - | FDD | FDD | FDD |
| CP Length | - | Normal | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 | 0 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | 0 | CP/2 |
| Es/Noc | dB | 10 | 5 | Test 1: -4dB Test 2: -3dB Test 3: 0dB Test 4: 3dB |
| ( equals to) |  | 3 | 3 | 3 |
| Number of Tx antennas | - | 1 | 1 | 1 |
| Antenna port | - | 15 | 15 | 15 |
| CSI-RS configuration (Defined in TS36.211) | - | 10 | 5 | 0 |
| CSI-RS periodicity and subframe offset TCSI-RS / ICSI-RS | - | 40/3, 80/3, 160/3 |  |  |
| Muting for CSI-RS |  | Yes |  |  |
| Propagation Condition | - | AWGN, EPA5, ETU30 |  |  |
| Noc Model | - | AWGN |  |  |
| CSI-RS periodicity | - | 40ms, 80ms, 160ms |  |  |
| NOTE1 : Noc value doesn’t include the three simulated TP signals’ power NOTE2 : ICSI-RS in this table is the subframe offset within each CSI-RS periodicity. 40/3 means the third subframe within 40ms periodicity, 80/3 means the third subframe within 80ms periodicity, and 160/3 means the third subframe within 160ms periodicity. |  |  |  |  |

Table 2: Other simulation assumption parameters for TP identification

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of TP1 and TP2 by the UE | Yes |
| TP 1, 2, 3 carrier frequency | Same |
| False detect threshold | Required as in a real UE implementation |
| Duty cycle | 100% (to represent non-DRX case) |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” TP identification of CSI-RS configuration |
| CP length detection | Both short or long CP may be present, hence UE needs to detect CP length |
| Receive antennas | - (uncorrelated) |

Performance metric for CSI-RS acquisition

- Criterion of successful event: (correct CSI-RS acquisition)

- Required successful event ratio: 90%

- Expected outcome from simulator: (CSI-RS acquisition delay)

## 2.2 Simulation results

It’s worth noting that during simulation UE is assumed to be aware of the full CSI-RS transmission information (subframe, RE configuration, scrambling initialization) of three TPs in this simulation.

The intention of this evaluation was to justify whether UE can correctly detect CSI-RS absence in corresponding target resources. Generally two performance metrics can be derived for evaluation:

- False Alarm: UE wrongly detect the absence of CSI-RS when NW don’t transmit corresponding CSI-RS resource (1%)

- Miss detection: UE fail to detect the corresponding CSI-RS when NW transmit (10%)

False alarm detection threshold was related to UE implementation. First of all, threshold was derived based on false alarm probability less than 1% during our evaluation.

Then we evaluate the CSI-RS TP identification time delay, i.e. the probability of UE correctly detect the absence of CSI-RS configuration during acquisition time should larger than 90%.

Table 1 below summarized our simulation results under different bandwidth and DRS periodicity.

Based on the results, it can be observed that:

Observation 1: For 6RB case, the maximum CSI-RS acquisition delay is scaled with DRS periodicity. Even under good channel condition, up to 3 DRS periodicity is required to acquire CSI-RS when 1 CSI-RS port is configured.

Observation 2: With extending measurement bandwidth to 50RB, CSI-RS acquisition delay is significantly decreased. 1 DRS periodicity is enough to acquire CSI-RS.

Table 1 CSI-RS acquisition time delay under 6RB and 50RB

| SNR[dB] | 6RB |  |  | 50RB |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DRS Periodicity | 40ms | 80ms | 160ms | 40ms | 80ms | 160ms |
| -4 | >400ms | >800ms | >1600ms | 80ms(2) | 160ms(2) | 320ms(2) |
| -3 | 360ms(9) | 720ms(9) | 1440ms(9) | 40ms(1) | 80ms(1) | 160ms(1) |
| 0 | 200ms(5) | 400ms(5) | 800ms(5) | 40ms(1) | 80ms(1) | 160ms(1) |
| 3 | 160ms(4) | 320ms(4) | 640ms(4) | 40ms(1) | 80ms(1) | 160ms(1) |
| 6 | 120ms(3) | 240ms(3) | 480ms(3) | 40ms(1) | 80ms(1) | 160ms(1) |

# Conclusion

In this contribution, we provide the initial evaluation results for CSI-RS based TP identification. Our observation is

Observation 1: For 6RB case, the maximum CSI-RS acquisition delay is scaled with DRS periodicity. Even under good channel condition, up to 3 DRS periodicity is required to acquire CSI-RS when 1 CSI-RS port is configured.

Observation 2: With extending measurement bandwidth to 50RB, CSI-RS acquisition delay is significantly decreased. 1 DRS periodicity is enough to acquire CSI-RS.

CSI-RS acquisition time delay under 6RB and 50RB

| SNR[dB] | 6RB |  |  | 50RB |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DRS Periodicity | 40ms | 80ms | 160ms | 40ms | 80ms | 160ms |
| -4 | >400ms | >800ms | >1600ms | 80ms(2) | 160ms(2) | 320ms(2) |
| -3 | 360ms(9) | 720ms(9) | 1440ms(9) | 40ms(1) | 80ms(1) | 160ms(1) |
| 0 | 200ms(5) | 400ms(5) | 800ms(5) | 40ms(1) | 80ms(1) | 160ms(1) |
| 3 | 160ms(4) | 320ms(4) | 640ms(4) | 40ms(1) | 80ms(1) | 160ms(1) |
| 6 | 120ms(3) | 240ms(3) | 480ms(3) | 40ms(1) | 80ms(1) | 160ms(1) |

# Reference

- R4-71AH-0092, “Simulation assumption for small cell discovery based on DRS”, Huawei

- R4-144534, ” System level simulation results for CRS/CSI-RS based SINR”, Samsung