3GPP TSG-RAN WG4 (Radio) Meeting #72bis	R4-146173

Singapore, 6 – 10 October 2014

Agenda Item:	7.36.2.2

Source: 	Alcatel-Lucent, KDDI

Title: 	TP for TR 36.851: Updates of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (26 + 41)

Document for:	Approval

# 1.	Introduction

The WI proposal to support LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 26, Band 41 and Band 41was approved in RAN#64 [1]. One objective of the WI is to specify the inter-band carrier aggregation (CA) of Band 26 and Band 41 which are not specified.

The impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting carrier aggregation of this new bandwidth combination to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to update the coexistence studies in the Inter-band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<Start of change>

##### 7.2.5.1.2	Co-existence studies for CA_26-41

Table 7.2.5.1.2-1gives the intermodulation products for Band 26 + Band 41 CA with 2 DLs. For the IMD analysis the maximum transmission as defined in table 7.2.5.1.1-1 is considered. It can be seen from the table that the 2nd harmonics of BS transmitting in Band 26 may fall into the BS receive band of Bands 3, 4, 9 and 10 and the 3rd harmonics of BS transmitting in Band 26 may fall into the BS receive band of Bands 38 and 41, while the 2nd IMD products caused by BS supporting CA of Band 26 and Band 41 may fall into the BS receive band of Band 3, 4, 9, 10, 22, 24 and 42, and the 3rd IMD products may fall into the BS receive band of Bands 5, 6, 7, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 38, 41 and 44. Note that Bands 7, 8, 20 and 38 are not intended for use in the same geographical area as Bands 26 and 41. Therefore, sharing antenna for CA operation between Band 26 and Band 41 is not recommended.  In addition, when one deploys CA_26-41 operation in certain region, these co-existence studies should be taken into account because harmonics and IMDs could fall into twenty bands in total.

Table 7.2.5.1.2-1: 2DLs B26 + B41 harmonics and IMD products frequency limits

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 859 | 894 | 2496 | 2690 |
| 2nd order harmonics frequency range (MHz) | 1718 | 1788 | 4992 | 5380 |
| 3rd order harmonics frequency range (MHz) | 2577 | 2682 | 7488 | 8070 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 1602 | 1831 | 3355 | 3584 |
| 3rd order IMD products | ¦f2_high – 2*f1_low¦ | ¦f2_low – 2*f1_high¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 708 | 972 | 4098 | 4521 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 4214 | 4478 | 5851 | 6274 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 665 | 1088 | 2461 | 2725 |
| 3rd order IMD products (with maximum channel bandwidth) | ¦f1_low – max BW f2¦ | ¦f1_high + max BW f2¦ | ¦f2_low – max BW f1¦ | ¦f2_high + max BW f1¦ |
| IMD frequency limits (MHz) | 839 | 914 | 2486 | 2700 |

<End of change>

# References

[1]	RP-140964, “New WI: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 26, Band 41 and Band 41”, KDDI.

[2]	R4-145496, “	TP for TR36.851: On Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (26 + 41)”, KDDI.

[3]	R4-14xxxx, “TR 36.851 V0.12.0: Rel-12 Inter-band Carrier Aggregation”, Ericsson.