3GPP TSG-RAN WG4 (Radio) Meeting #72bis	R4-146174

Singapore, 6 – 10 October 2014

Agenda Item:	7.36.2.3

Source: 	Alcatel-Lucent, T-Mobile USA, Nokia Networks

Title: 	TP for TR 36.853: Updates of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (4 + 4 + 12)

Document for:	Approval

# 1.	Introduction

The revised WI proposal to support LTE Advanced 3 Band CA (3DL/1UL) of Band 4, Band 4 and Band 12 was approved in RAN#65 [1]. One objective of the WI is to specify the band-combinations specific Radio Frequency (RF) requirements for this new Carrier Aggregation (CA) band combination.

The impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting carrier aggregation of this new bandwidth combination to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to update the coexistence studies in the 3 Band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<Start of change>

#### 6.36.3.1		For the BS

Table 6.36.3.1-1 shows harmonics frequency limits and Table 6.36.3.1-2 intermodulation products frequency limits for CA of Band 4, Band 4 and Band 12, respectively. Harmonics and intermodulation product problems are summarized below the relevant Table.

Table 6.36.3.1-1: DL harmonics frequency limits for CA of Band 4, Band 4 and Band 12

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 2110 | 2155 |
| 2nd order harmonics frequency range (MHz) | 1458 to 1492 |  | 4220 to 4310 |  |
| 3rd order harmonics frequency range (MHz) | 2187 to 2238 |  | 6330 to 6465 |  |

As shown in Table 6.36.3.1-2, second and third harmonics will not fall to any UL frequencies for Bands specified in 3GPP specifications.

Table 6.36.3.1-2: DL intermodulation products frequency limits for CA of Band 4, Band 4 and Band 12

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 2110 | 2155 |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 1364 to 1426 |  | 2839 to 2901 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2* f2_high – f1_low |
| IMD frequency range (MHz) | 618 to 697 |  | 3474 to 3581 |  |
| Two-tone 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency range (MHz) | 3568 to 3647 |  | 4949 to 5056 |  |
| Three-tone 3rd order IMD products | (f1_low – f2_high + f2_low) | (f1_high + f2_high – f2_low) | (f2_low – f1_high + f1_low) | (f2_high + f1_high – f1_low) |
| IMD frequency range (MHz) | 684 to 791 |  | 2093 to 2172 |  |

As shown in Table 6.36.3.1-2, second order intermodulation products will not fall to any UL frequencies for Bands specified in 3GPP specifications.

Third intermodulation products may fall to UL frequencies of Band 12-14, 17, 22, 28 or 42-44. As third intermodulation products may fall to own receive block of Band 12 (highlighted in Table 6.x.3.1-2), the desensitisation may be an issue.

Assume that the 3 carriers transmitted by the Band (4 + 4 + 12) BS are a, b and c MHz in channel bandwidth and d, e and f MHz from the lower edges of Bands 4 and 12 DL frequency bands as shown in Figure 6.36.3.1-1 below:

a MHz

b MHz

3rd CC

2nd CC

c MHz

1st CC

2110

729

d MHz

MHz

e MHz

f MHz

f1-low

f1-high

f2-low

f2-high

f3-low

f3-high

Figure 6.36.3.1-1: 3 carriers transmitted by the Band (4 + 4 + 12) BS

And the corresponding BS receive blocks are as shown in Figure 6.36.3.1-2 below:

a MHz

b MHz

3rd CC

2nd CC

c MHz

1st CC

1710

699

d MHz

MHz

e MHz

f MHz

f1-low

f1-high

f2-low

f2-high

f3-low

f3-high

Figure 6.36.3.1-2: 3 carriers received by the Band (4 + 4 + 12) BS

Then the 3rd order IMD products caused in the BS by transmitting of the 3 CC can be calculated as shown in Table 6.36.3.1-3 below:

Table 6.36.3.1-3: Band (4 + 4 + 12) BS DL 3rd order IMD products

| 3rd order IMD products | IMD frequency limits (MHz) |
| --- | --- |
| (f1-low + f2-low – f3-high) | 729 + d + e – f – c |
| (f1-high + f2-high – f3-low) | 729 + d + a + e + b – f |
| (f1-low + f3-low – f2-high) | 729 + d + f – e – b |
| (f1-high + f3-high – f2-low) | 729 + d + a + f + c – e |
| (f2-low + f3-low – f1-high) | 3491 + e + f – d – a |
| (f2-high + f3-high – f1-low) | 3491 + e + b + f + c – d |
| (f1-low + f2-low + f3-low) | 4949 + d + e + f |
| (f1-high + f2-high + f3-high) | 4949 + d + a + e + b + f + c |

Comparing the IMD frequency limits in Table 6.36.3.1-3 with the BS receive blocks in Figure 6.36.3.1-2, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of the 3 CC will only fall into the BS own receive block of Band 12 under one of the transmit configurations shown in Table 6.36.3.1-4 below:

Table 6.36.3.1-4: Band (4 + 4 + 12) BS transmit configurations with 3rd IMD within Band 12 BS own receive blocks

| 1st CC channel bandwidth (a MHz) | 2nd CC channel bandwidth (b MHz) | 3rd CC channel bandwidth (c MHz) | CC position (d, e and f MHz) |
| --- | --- | --- | --- |
| 5 | 5 | 5 | 20 < f – e < 40 |
| 5 | 5 | 10 | 15 < f – e <= 35 |
| 5 | 10 | 5 | 20 < f – e <= 40 |
| 10 | 5 | 5 | 15 < f – e <= 40 |
| 5 | 5 | 15 | 10 < f – e <= 30 |
| 5 | 10 | 10 | 15 < f – e <= 35 |
| 5 | 15 | 5 | 20 < f – e <= 40 |
| 10 | 5 | 10 | 10 < f – e <= 35 |
| 10 | 10 | 5 | 15 < f – e <= 40 |
| 5 | 5 | 20 | 5 < f – e <= 25 |
| 5 | 10 | 15 | 10 < f – e <= 30 |
| 5 | 15 | 10 | 15 < f – e <= 35 |
| 5 | 20 | 5 | 20 < f – e <= 40 |
| 10 | 5 | 15 | 5 < f – e <= 30 |
| 10 | 10 | 10 | 10 < f – e <= 35 |
| 10 | 15 | 5 | 15 < f – e <= 40 |
| 5 | 10 | 20 | 10 < f – e <= 25 |
| 5 | 15 | 15 | 15 < f – e <= 30 |
| 5 | 20 | 10 | 20 < f – e <= 35 |
| 10 | 5 | 20 | 5 < f – e <= 25 |
| 10 | 10 | 15 | 10 < f – e <= 30 |
| 10 | 15 | 10 | 15 < f – e <= 35 |
| 10 | 20 | 5 | 20 < f – e <= 40 |
| 5 | 15 | 20 | 15 < f – e <= 25 |
| 5 | 20 | 15 | 20 < f – e <= 30 |
| 10 | 10 | 20 | 10 < f – e <= 25 |
| 10 | 15 | 15 | 15 < f – e <= 30 |
| 10 | 20 | 10 | 20 < f – e <= 35 |
| 5 | 20 | 20 | 20 < f – e <= 25 |
| 10 | 15 | 20 | 15 < f – e <= 25 |
| 10 | 20 | 15 | 20 < f – e <= 30 |
| 10 | 20 | 20 | 20 < f – e <= 25 |

It is suggested BS transmitters supporting CA of Band 4, Band 4 and Band 12 should not share the same antenna with Band 12-14, 17, 22, 28 or 42-44 BS receiver, unless the antenna path meets very stringent third order PIM specification so that the PIM will not cause Band 12-14, 17, 22, 28 or 42-44 BS receiver desensitization.

<End of change>

# References

[1]	RP-141709, “Revised WID: LTE Advanced 3 Band CA (3DL/1UL) of Band 4, Band 4 and Band 12”, T-Mobile USA.

[2]	R4-144006, “	TP for TR 36.853: Harmonics and intermodulation products generated by the BS supporting LTE-A CA of Band 4, Band 4 and Band 12”, NSN, T-Mobile USA.

[3]	RP-141217, “TR 36.853 v2.0.0 on LTE Advanced 3 Band Carrier Aggregation (3DL/1UL)”, AT&T.