3GPP TSG-RAN WG4 Meeting #72bis	R4-146438

# Singapore, 6th – 10th October, 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.133 | CR | 1377 | rev |  | Current version: | 12.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Increased number of carriers to monitor in Cell_DCH mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12 |  |  |  |  |  |  | Date: |  | 2014-10-06 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Modifications to Cell_DCH requirements to support increased number of carriers to monitor |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modifications to Cell_DCH requirements to support increased number of carriers to monitor |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New Cell_DCH requirements to support increased number of carriers to monitor will not captured in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |

## <Start of Changes>

## 8.1	General Measurement Requirements in CELL_DCH State

### 8.1.1	Introduction

This section contains requirements on the UE regarding measurement reporting in CELL_DCH state. The requirements are split in FDD intra frequency, FDD inter frequency, TDD, GSM and E-UTRAN measurements. These measurements may be used by the UTRAN, e.g. for handover decisions. The measurements are defined in TS 25.215, the measurement model is defined in TS 25.302 and measurement accuracies are specified in section 9. Control of measurement reporting is specified in TS 25.331 and parallel measurements are specified in section 8.2. Compressed mode is specified in TS 25.215.

### 8.1.2	Requirements

#### 8.1.2.1	UE Measurement Capability

In CELL_DCH state, when a single uplink carrier frequency is configured, the UE shall be able to monitor up to

-	32 intra frequency FDD cells (including active set); and

-	32 inter frequency cells, including

-	FDD cells distributed on up to 2 additional FDD carriers; and

-	Depending on UE Capability, TDD cells, distributed on up to 3 TDD carriers; and

-	Depending on UE capability, 32 GSM cells distributed on up to 32 GSM carriers; and

-	Depending on UE capability, 4 E-UTRA FDD cells per E-UTRA FDD carrier for up to 4 E-UTRA FDD carriers; and

-	Depending on UE capability, 4 E-UTRA TDD cells per E-UTRA TDD carrier for up to 4 E-UTRA TDD carriers; and

-	Depending on UE capability, the UE shall be able to monitor up to 16 intra frequency cells during IPDL gaps.

In addition to the requirements defined above, a UE supporting E-UTRA measurements shall be capable of monitoring a minimum total of at least 8 carrier frequency layers, including the intra-frequency serving layer and comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD and GSM layers (one GSM layer corresponds to 32 cells).

In CELL_DCH state, when a single uplink carrier frequency is configured, UEs which indicate support for Increased UE carrier monitoring UTRA (either default or signalled) according to capabilities in [16] shall be capable of monitoring up to

-	32 intra frequency FDD cells (including active set); and

-	80 inter frequency cells with maximum of 32 cells per frequency, including

-	FDD cells distributed on up to 5 additional FDD carriers; and

-	Depending on UE Capability, TDD cells, distributed on up to 3 TDD carriers; and

-	Depending on UE capability, 32 GSM cells distributed on up to 32 GSM carriers; and

-	Depending on UE capability, 8 E-UTRA FDD cells per E-UTRA FDD carrier for up to 4 E-UTRA FDD carriers; and

-	Depending on UE capability, 8 E-UTRA TDD cells per E-UTRA TDD carrier for up to 4 E-UTRA TDD carriers; and

-	Depending on UE capability, the UE shall be able to monitor up to 16 intra frequency cells during IPDL gaps.

In addition to the requirements defined above, a UE supporting E-UTRA measurements shall be capable of monitoring a minimum total of at least 13 carrier frequency layers, including the intra-frequency serving layer and comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD and GSM layers (one GSM layer corresponds to 32 cells).

In CELL_DCH state, when dual uplink carrier frequencies are configured, the UE shall be able to monitor up to

-	32 intra frequency FDD cells (including active set) per intra frequency carrier, and

-	32 inter frequency cells, including

-	FDD cells distributed on up to 2 additional FDD carriers and

-	Depending on UE Capability, TDD cells, distributed on up to 3 TDD carriers and

-	Depending on UE capability, 32 GSM cells distributed on up to 32 GSM carriers and

-	Depending on UE capability, 4 E-UTRA FDD cells per E-UTRA FDD carrier for up to 4 E-UTRA FDD carriers and

-	Depending on UE capability, 4 E-UTRA TDD cells per E-UTRA TDD carrier for distributed on up to 4 E-UTRA TDD carriers and

-	Depending on UE capability, the UE shall be able to monitor up to 16 intra frequency cells during IPDL gaps.

In addition to the requirements defined above, a UE supporting E-UTRA measurements shall be capable of monitoring a minimum total of at least 9 carrier frequency layers, including the two intra-frequency carriers and comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD and GSM layers (one GSM layer corresponds to 32 cells).

In CELL_DCH state, when dual uplink carrier frequencies are configured, UEs which indicate support for Increased UE carrier monitoring UTRA (either default or signalled) according to capabilities in [16] shall be capable of monitoring up to

-	32 intra frequency FDD cells (including active set) per intra frequency carrier, and

-	80 inter frequency cells with maximum of 32 cells per frequency, including

-	FDD cells distributed on up to 5 additional FDD carriers and

-	Depending on UE Capability, TDD cells, distributed on up to 3 TDD carriers and

-	Depending on UE capability, 32 GSM cells distributed on up to 32 GSM carriers and

-	Depending on UE capability, 8 E-UTRA FDD cells per E-UTRA FDD carrier for up to 4 E-UTRA FDD carriers and

-	Depending on UE capability, 8 E-UTRA TDD cells per E-UTRA TDD carrier for distributed on up to 4 E-UTRA TDD carriers and

-	Depending on UE capability, the UE shall be able to monitor up to 16 intra frequency cells during IPDL gaps.

In addition to the requirements defined above, a UE supporting E-UTRA measurements shall be capable of monitoring a minimum total of at least 13 carrier frequency layers, including the two intra-frequency carriers and comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD and GSM layers (one GSM layer corresponds to 32 cells).

## <End of Changes>

## <Start of Changes>

#### 8.1.2.3	FDD inter frequency measurements

In the CELL_DCH state when a transmission gap pattern sequence with the "FDD measurements" purpose is provided by the network the UE shall continuously measure identified inter frequency cells and search for new inter frequency cells indicated in the measurement control information.

In order for the requirements in the following subsections to apply the UTRAN must provide a transmission gap pattern sequence with measurement purpose FDD measurement using the following combinations for TGL1, TGL2, TGD and Max TGPL:

Table 8.1

| TGL1 [slots] |  | TGL2 [slots] |  | TGD [slots] |  | Max TGPL [frames] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  | - |  | undefined |  | 18 |  |
| 14 |  | - |  | undefined |  | 36 |  |
| 10 |  | - |  | undefined |  | 24 |  |
| 7 |  | 7 |  | 15…269 |  | 18 + ceil(TGD/15) |  |
| 14 |  | 14 |  | 45…269 |  | 36 + ceil(TGD/15) |  |

##### 8.1.2.3.1	Identification of a new cell

The UE shall be able to identify a new detectable cell belonging to the monitored set within

for normal performance group and

the UE shall be able to identify a new detectable cell belonging to the monitored set within

for reduced performance group

and UE that supports inter frequency detected set CPICH measurements shall also be able to identify a new detectable cell belonging to the detected set within Tidentify inter detected set = 30·(s/(s-1)) ·NNorm,freq  s for normal performance group and 30·s ·NReduced,freq  s for reduced performance group. UE which do not indicate support for Increased UE carrier monitoring UTRA (either default or signalled) according to capabilities in [16] shall set NReduced,freq  = 0.

If the UE does not need compressed mode to measure cells on a frequency adjacent to the intra-frequency (irrespective of the number of serving cells configured in the UE) according to its measurement capability in the IE “Adjacent Frequency measurements without compressed mode” defined in section 10.3.3.21 of TS 25.331 [16], and if the Adjacent Frequency Index [16] is configured, the UE shall be able to identify a new detectable cell on the indicated adjacent frequency belonging to the monitored set within Tidentify,inter,optional and UE that supports inter frequency detected set CPICH measurements shall be able to identify a new detectable cell on the indicated adjacent frequency within Tidentify,inter detected set  = 30s.

If the UE does not need compressed mode to measure cells on a frequency in a band jointly supported with the band of intra-frequency according to its measurement capability in the IE “Inter-band Frequency measurements without compressed mode” defined in section 10.3.3.21 of TS 25.331 [16], the UE shall be able to identify a new detectable cell on that frequency belonging to the monitored set within Tidentify,inter,optional  and UE that supports inter frequency detected set CPICH measurements shall be able to identify a new detectable cell on that frequency within Tidentify,inter detected set  = 30s.

If the UE is able to search enhanced inter-frequency measurements without compressed mode according to its measurement capability in the IE “Enhanced inter-frequency measurements without compressed mode” defined in section 10.3.3.21 of TS 25.331 [16],

- when a single uplink frequency is configured, the UE shall be able to identify a new detectable cell each of the 2 indicated frequencies belonging to the monitored set within Tidentify,inter,optional; or

- when dual uplink frequencies are configured, the UE shall be able to identify a new detectable cell on the indicated frequency belong to the monitored set within Tidentify,inter,optional,

where the indicated frequenc(ies) together with intra-frequenc(ies) shall be within UE RF capability as per Band Combination and Supported Carrier Combination List in [16] and UE that supports inter frequency detected set CPICH measurements shall be able to identify a new detectable cell on the indicated frequenc(ies) within Tidentify,inter detected set  = 30s.

If DL_DRX_Active = 0,

.

If DL_DRX_Active = 1, the UE shall be able to identify and decode the SFN of a new detectable cell belonging to the monitored set according to the figures in Table 8.1a

Table 8.1a: Cell identification times, inter-frequency measurements, Enhanced IF measurements, No CM, DL DRX active

| DRX cycle length (subframes) | Tidentify, inter, optional (s) |
| --- | --- |
| DRX-cycle length <10 | 0.8 |
| 10 ≤ DRX-cycle length ≤ 20 | 1.5 Note 1 |
| 20 < DRX-cycle length ≤ 80 | [3.0] Note 1 |
| Note1: The requirements for DRX cycle lengths of greater than 20 subframes are only applicable for UEs that support DRX cycle 2 according to its capability in IE “Enhanced Downlink DRX” defined in section 10.3.3.25 of TS 25.331 [16]. |  |

If the UE does not need compressed mode to measure cells on configured carrier frequencies corresponding to secondary serving HS-DSCH cells according to its measurement capability in the IE, “Inter-frequency measurements on configured carriers without compressed mode” defined in section 10.3.3.21 of TS 25.331 [16], the UE shall be able to identify a new detectable cell on the configured carrier frequency belonging to the monitored set within Tidentify,inter,optional where:

If DL_DRX_Active = 0, Tidentify, inter, optional = 2.2* (s/(s-1)) * N Norm,freq seconds for normal performance group and 2.2* s * N Reduced,freq  seconds for reduced performance group.

If DL_DRX_Active = 1 and the UE DRX cycle < 10 subframes, Tidentify, inter, optional = 2.2*(s/(s-1)) * N Norm,freq seconds for normal performance group and 2.2* s * N Reduced,freq  seconds for reduced performance group.

If DL_DRX_Active = 1 and the UE DRX cycle ≥ 10 subframes, Tidentify, inter, optional = 4.4*(s/(s-1)) * N Norm,freq seconds for normal performance group and 4.4* s * N Reduced,freq  seconds for reduced performance group.

If DL_DRX_Active = 1, the value of Tidentify, inter, optional is according to the figures in Table 8.1b.

Table 8.1b: Cell identification times, inter-frequency measurements, IF measurements on configured carriers, No CM, DL DRX active,

| DRX cycle length (subframes) | Tidentify, inter, optional (s) (normal performance) | Tidentify, inter, optional (s) (reduced performance) |
| --- | --- | --- |
| DRX-cycle length <10 | 2.2* s * N Reduced,freq | 2.2* s * N Reduced,freq |
| 10 ≤ DRX-cycle length ≤ 20 | 4.4* s * N Reduced,freq Note 1 | 4.4* s * N Reduced,freq |
| 20 < DRX-cycle length ≤ 80 | 6.6* s * N Reduced,freq | 6.6* s * N Reduced,freq |
| Note1: The requirements for DRX cycle lengths of greater than 20 subframes are only applicable for UEs that support DRX cycle 2 according to its capability in IE “Enhanced Downlink DRX” defined in section 10.3.3.25 of TS 25.331 [16]. |  |  |

A cell shall be considered detectable when

-	CPICH Ec/Io > -20 dB,

-	SCH_Ec/Io > -17 dB for at least one channel tap and SCH_Ec/Ior is equally divided between primary synchronisation code and secondary synchronisation code. When L3 filtering is used an additional delay can be expected.

##### 8.1.2.3.2	UE CPICH measurement capability

When transmission gaps are scheduled for FDD inter frequency measurements the UE physical layer shall be capable of reporting measurements to higher layers with measurement period given by

for normal performance group and

for reduced performance group

If the UE does not need compressed mode to perform inter-frequency measurements, the measurement period for inter frequency measurements is 480 ms. UE which do not indicate support for Increased UE carrier monitoring UTRA (either default or signalled) according to capabilities in [16] shall set NReduced,freq  = 0.

The UE shall be capable of performing CPICH measurements for Xbasic measurement FDD inter inter-frequency cells per FDD frequency of the monitored set or the virtual active set, or the detected set for UE that supports inter frequency detected set meaurements, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of TMeasurement_Inter. If the UE has identified more than Xbasic measurement FDD inter cells, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased.

Xbasic measurement FDDinter = 6

TMeasurement_Period Inter = 480 ms. The period used for calculating the measurement period Tmeasurement_inter for inter frequency CPICH measurements.

TInter:: This is the minimum time that is available for inter frequency measurements , during the period TMeasurement_Period inter with an arbitrarily chosen timing. The minimum time per transmission gap is calculated by using the actual idle length within the transmission gap as given in the table 11 of Annex B in TS 25.212 and by assuming 2*0.5 ms for implementation margin and after that taking only full slots into account in the calculation.

Tbasic_identify_FDD,inter = 300 ms. This is the time period used in the inter frequency equation where the maximum allowed time for the UE to identify a new FDD cell is defined.

Tbasic_measurement_FDD inter = 50 ms. This is the time period used in the equation for defining the measurement period for inter frequency CPICH measurements.

NNorm,Freq: Number of FDD frequencies in the normal performance group indicated in the inter frequency measurement control information.

NReduced,Freq: Number of FDD frequencies in the reduced performance group indicated in the inter frequency measurement control information.

s = scale factor used to scale the cell identification/measurement delay of reduced performance group and takes the values {1.667, 4, 8, 16}.

For UE that supports inter frequency detected set CPICH measurements, out of the Xbasic measurement FDD inter inter-frequency cells per FDD frequency for which the UE is capable of performing CPICH measurements, the UE shall be capable of measuring the CPICH of at least one detected set cell.

If the UE does not need compressed mode to measure cells on a frequency adjacent to the intra-frequency (irrespective of the number of serving cells configured in the UE) according to its measurement capability in the IE “Adjacent Frequency measurements without compressed mode” defined in section 10.3.3.21 of TS 25.331 [16], and if the Adjacent Frequency Index [16] is configured,

- when DL_DRX_Active=0, the UE shall be capable of performing CPICH measurements for 8 cells on the indicated adjacent frequency, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of 200ms. For UE that supports inter frequency detected set CPICH measurements, out of 8 cells on the indicated adjacent frequency, the UE shall be capable of measuring the CPICH of at least one detected set cell. If the UE has identified more than 8 cells, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased; or

- when DL_DRX_Active=1, the UE shall be capable of performing CPICH measurements for at least Ymeasurement intra cells on the indicated adjacent frequency, where Ymeasurement intra is defined in sub-clause 8.1.2.2.2. If the UE has identified more than Ymeasurement intra cells, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased. For UE that supports inter frequency detected set CPICH measurements, out of Ymeasurement intra cells on the indicated adjacent frequency, the UE shall be capable of measuring the CPICH of at least one detected set cell. If the UE has identified more than Ymeasurement intra cells, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased.

If the UE does not need compressed mode to measure cells on a frequency in a band jointly supported with the band of intra-frequency according to its measurement capability in the IE “Inter-band Frequency measurements without compressed mode” defined in section 10.3.3.21 of TS 25.331 [16],

- when DL_DRX_Active=0, the UE shall be capable of performing CPICH measurements for 8 cells on that frequency, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of 200ms. For UE that supports inter frequency detected set CPICH measurements, out of 8 cells on that frequency, the UE shall be capable of measuring the CPICH of at least one detected set cell. If the UE has identified more than 8 cells, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased; or

- when DL_DRX_Active=1, the UE shall be capable of performing CPICH measurements for at least Ymeasurement intra cells on that frequency. If the UE has identified more than Ymeasurement intra cells, where Ymeasurement intra is defined in sub-clause 8.1.2.2.2, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased. For UE that supports inter frequency detected set CPICH measurements, out of Ymeasurement intra cells on that frequency, the UE shall be capable of measuring the CPICH of at least one detected set cell. If the UE has identified more than Ymeasurement intra cells, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased.

If the UE is able to search enhanced inter-frequency measurements without compressed mode according to its measurement capability in the IE “Enhanced inter-frequency measurements without compressed mode” defined in section 10.3.3.21 of TS 25.331 [16], and DL_DRX_Active = 0,

- when a single uplink frequency is configured, the UE shall be capable of performing CPICH measurements for 8 cells each on 2 indicated frequencies, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of 200ms. For UE that supports inter frequency detected set CPICH measurements, out of 8 cells each on 2 indicated frequencies, the UE shall be capable of measuring the CPICH of at least one detected set cell each on 2 indicated frequencies. If the UE has identified more than 8 cells on each frequency, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased; or

- when dual uplink frequencies are configured, the UE shall be capable of performing CPICH measurements for 8 cells on the indicated frequency, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of 200ms. For UE that supports inter frequency detected set CPICH measurements, out of 8 cells on the indicated frequency, the UE shall be capable of measuring the CPICH of at least one detected set cell. If the UE has identified more than 8 cells, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased.

If the UE is able to search enhanced inter-frequency measurements without compressed mode according to its measurement capability in the IE “Enhanced inter-frequency measurements without compressed mode” defined in section 10.3.3.21 of TS 25.331 [16], and DL_DRX_Active = 1,

- when a single uplink frequency is configured, the UE shall be capable of performing CPICH measurements for at least Ymeasurement intra cells each on 2 indicated frequencies, where Ymeasurement intra is defined in sub-clause 8.1.2.2.2. If the UE has identified more than Ymeasurement intra cells on the indicated frequency, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased for that frequency. For UE that supports inter frequency detected set CPICH measurements, out of Ymeasurement intra cells each on 2 indicated frequencies, the UE shall be capable of measuring the CPICH of at least one detected set cell each on 2 indicated frequencies. If the UE has identified more than Ymeasurement intra cells on each frequency, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased; or

- when dual uplink frequencies are configured, the UE shall be capable of performing CPICH measurements for at least Ymeasurement intra cells on the indicated frequency, where Ymeasurement intra is defined in sub-clause 8.1.2.2.2. If the UE has identified more than Ymeasurement intra cells on the indicated frequency, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased for that frequency. For UE that supports inter frequency detected set CPICH measurements, out of Ymeasurement intra cells on the indicated frequency, the UE shall be capable of measuring the CPICH of at least one detected set cell. If the UE has identified more than Ymeasurement intra cells, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased.

If the UE does not need compressed mode to measure cells on configured carrier frequencies corresponding to secondary serving HS-DSCH cells according to its measurement capability in the IE, “Inter-frequency measurements on configured carriers without compressed mode” defined in section 10.3.3.21 of TS 25.331 [16]:

-	when DL_DRX_Active=0, or DL_DRX_Active=1 and the UE DRX cycle < 10 subframes, the UE shall be capable of performing CPICH measurements for 6 cells per configured carrier frequency belonging to the monitored set or the virtual active set, or the detected set for UE that supports inter frequency detected set measurements on the configured carrier frequency, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of 1000*(s/(s-1))* NNorm,Freq ms for normal performance group and 1000*s* NReduced,Freq ms for reduced performance group. For UE that supports inter frequency detected set CPICH measurements, out of 6 cells per configured carrier frequency, the UE shall be capable of measuring the CPICH of at least one detected set cell. If the UE has identified more than 6 cells, the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased; or

-	when DL_DRX_Active=1 and the UE DRX cycle ≥ 10 subframes, the UE shall be capable of performing CPICH measurements for at least 6cells per configured carrier frequency belonging to the monitored set or the virtual active set, or the detected set for UE that supports inter frequency detected set measurements on the configured carrier frequency, and the UE physical layer shall be capable of reporting measurements to high layers with the measurement period of 2000* (s/(s-1))* N Norm,Freq ms for normal performance group and 2000*s* NReduced,Freq ms for reduced performance group. For UE that supports inter frequency detected set CPICH measurements, out of 6 cells per configured carrier frequency, the UE shall be capable of measuring the CPICH of at least one detected set cell. If the UE has identified more than 6cells then the UE shall perform measurements of all identified cells but the reporting rate of CPICH measurements of cells from UE physical layer to higher layers may be decreased.

The measurement accuracy for all measured cells shall be as inter frequency measurement accuracy specified in the sub-clause 9.1.1 and 9.1.2.

## <End of Changes>

## <Start of Changes>

#### 8.1.2.6	E-UTRAN measurements

The requirements in this section apply only to UEs supporting UTRAN FDD and E-UTRAN FDD and TDD.

1)	In CELL_DCH state when a transmission gap pattern sequence is provided by the UTRAN the UE shall continuously measure previously detected E-UTRAN cells and search for new E-UTRAN cells.

In order for the requirements in the following subsections to apply the UTRAN must provide a transmission gap pattern sequence with measurement purpose "E-UTRAN measurement" using the following combinations for TGL1, TGL2, TGD and Max TGPL:

Table 8.8a

| TGL1 [slots] |  | TGL2 [slots] |  | TGD [slots] |  | Max TGPL [frames] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ≥10 |  | - |  | undefined |  | 12 |  |

2)	If the UE does not need compressed mode to perform E-UTRAN measurements:

-	the UE shall measure either all E-UTRAN cells present in the monitored set or, if only frequencies are provided in the neighbour cell list, the strongest cells present in the detected set, up to the monitoring capabilities of the UE.

-	the relevant requirements for E-UTRAN RRC_CONNECTED mode specified in 3GPP TS 36.133 [24] shall apply. This is further detailed in the following subclauses.

##### 8.1.2.6.1	Identification of a new cell

When compressed mode gaps are used for E-UTRAN measurements, the UE shall be able to identify a new detectable E-UTRAN cell within

for normal performance group and

the UE shall be able to identify a new detectable E-UTRAN cell within

for reduced performance group;

If the UE does not need compressed mode to perform E-UTRAN measurements, the UE shall be able to identify a new detectable E-UTRAN cell within  ms.

Where:

TE-UTRAN: This is the minimum time that is available for E-UTRAN measurements during a 480ms period with an arbitrarily chosen timing. The minimum time per compressed mode gap is calculated by subtracting 2*0.5 ms from the length of the gap.

NNorm,freq,E-UTRAN :This is the number of E-UTRAN carriers being monitored in the normal performance group

NReduced,freq,E-UTRAN :This is the number of E-UTRAN carriers being monitored in the reduced performance group. UE which do not indicate support for Increased UE carrier monitoring UTRA (either default or signalled) according to capabilities in [16] shall set NReduced,freq  = 0.

s = scale factor used to scale the cell identification delay of reduced performance group and takes the values {1.667, 4, 8, 16}.

TBasic_Identify_E-UTRAN = 480ms

When L3 filtering is used an additional delay can be expected.

An E-UTRANcell shall be considered detectable when

-	RSRP|dBm RSRP Ês/Iot according to Annex B.2.1 for a corresponding Band

-	other RSRP related side condition given in Section 9.1 of [24] are fulfilled,

-	SCH_RP|dBm and SCH_RP/Iot according to Annex B.2.1 for a corresponding Band

##### 8.1.2.6.2	E-UTRAN  RSRP and RSRQ measurement period

When compressed mode gaps are scheduled for E-UTRAN measurements the UE physical layer shall be capable of reporting measurements to higher layers with measurement accuracy as specified in sub-clause 9.1.4a  and 9.1.4bwith measurement period of TMeasurement_Period_E-UTRAN =480 x (s/(s-1)) x NNorm,Freq ms for normal performance group and TMeasurement_Period_E-UTRAN =480 x s x NReduced,Freq ms for reduced performance group where NNorm,Freq is the number of E-UTRAN frequencies in the normal performance group indicated in the inter-RAT measurement control information and NReduced,Freq is the number of E-UTRAN frequencies in the reduced performance group indicated in the inter-RAT measurement control information. ‘s’ is scale factor used to scale the measurement delay of reduced performance group and takes the values {1.667, 4, 8, 16}. UE which do not indicate support for Increased UE carrier monitoring UTRA (either default or signalled) according to capabilities in [16] shall set NReduced,freq  = 0.

The UE shall be capable of performing RSRP and RSRQ measurements of at least 4 E-UTRAN cells per E-UTRAN frequency, according to its supported UE capabilities. Additionally, for a UE supporting E-UTRA measurements in Cell-DCH state, the UE shall be capable of monitoring a minimum total of at least 8 carrier frequency layers, including the intrafrequency serving layer and comprising of any allowed combination of E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD and GSM layers (one GSM layer corresponds to 32 cells).

## <End of Changes>