3GPP TSG RAN WG4 Meeting #72bis			R4-146441

# Singapore, 6th – 10th October 2014

Title:	[Draft] Reply LS on CSG Cell Reselection Performance Requirements

Source:	3GPP WG RAN4

To:	3GPP WG RAN5

Cc:	3GPP WG RAN2

Contact Person:

#### Name:	Amith Chincholi

Tel. Number:	+1 408 533 9615

E-mail Address:	amithc@qti.qualcomm.com

1. Overall Description:

RAN4 has discussed CSG-to-inter-frequency neighbour-list CSG cell reselection requirements and CSG to inter RAT neighbour-list EUTRA CSG cell reselection requirements and come to the conclusion that CSG-to-neighbour-list-CSG cell reselection requirements are required.

RAN4 would like to inform RAN5 that performance requirements related to CSG to CSG reselection are required. There are none currently and RAN4 will introduce the following requirements to support CSG to CSG reselection:

- Reselection from a CSG to an inter-frequency CSG cell

If 0< Sx <= Sintersearch or when Sintersearch is not specified (when no absolute priorities are configured), and the serving cell is the highest ranked cell on the serving frequency, the UE shall perform search and reselection to an allowed inter-frequency CSG cell that has met CSG reselection criterion defined in [1] and that is in its whitelist within 6 minutes, provided that this inter-frequency CSG cell is listed in system information without absolute priority configured.

If 0< Srxlevservingcell <= Sprioritysearch1 or 0< Squalservingcell <= Sprioritysearch2, or Sprioritysearch,1 and Sprioritysearch,2 not specified (when absolute priorities are configured) and the serving cell is the highest ranked cell on the serving frequency, the UE shall perform search and reselection to an allowed inter-frequency CSG cell that has met CSG reselection criterion defined in [1] and that is in its whitelist within 6 minutes, provided that this inter-frequency CSG cell is listed in system information with absolute priority.

If Sx<=0 or the serving cell is not the highest ranked cell on the serving frequency, the requirements in section 4.2.2.3 applies.

- Reselection from a CSG to an inter-RAT E-UTRA CSG cell

If 0< Srxlevservingcell <= Sprioritysearch1 or 0< Squalservingcell <= Sprioritysearch2, or Sprioritysearch,1 and Sprioritysearch,2 not specified (when absolute priorities are configured) and the serving cell is the highest ranked cell on the serving frequency, the UE shall perform search and reselection to an allowed inter-RAT E-UTRA CSG cell that has met CSG reselection criterion defined in [1] and that is in its whitelist within 6 minutes, provided that this inter-RAT E-UTRA CSG cell is listed in system information with absolute priority.

If Sx<=0 or the serving cell is not the highest ranked cell on the serving frequency, the requirements in section 4.2.2.5a applies.

2. Actions:

To 3GPP WG RAN5:

ACTION: 	RAN4 kindly asks RAN5 to take note of the agreements on CSG-to-CSG cell reselection requirements as discussed in [1] and TS25.133 CR [2].

3. References:

- R4-146440, “New requirements for reselection from a CSG to an inter-frequency CSG cell or inter-RAT E-UTRA         CSG cell”

- R4-146442, “CR for new requirements for reselection from a CSG to an inter-frequency CSG cell or inter-RAT E-UTRA CSG cell”

4. Date of Next 3GPP RAN4 Meetings:

3GPP RAN4 Meeting #73           17th – 21th November 2014		San Francisco, USA

3GPP RAN4 Meeting #74           9th – 13th February 2015		Athens, Greece