3GPP TSG-RAN WG4 Meeting #72bis	R4-146735

Singapore, 6 – 10 Oct, 2014

Agenda item:	7.26.1

Source: 	Intel Corporation

Title: 	TP for TR 36.851: LTE_CA_B5_B13

Document for:	Approval

## 1	Introduction

1UL/2DL CA of B5+B13 has been discussed in previous meeting [1]. This contribution provides some further consideration on quadplexer implementation.

## 2	Discussion

Both B5 and B13 are both low bands. A quadplexer is needed for B5+B13 CA. There is limited information on the development of B5+B13 quadplexer at this moment. Table 2.1 lists the additional insertion loss from vendor 1. All the data are typical. Due to the reason that ETC data are not available, a compromising alternative is to leverage on relaxation values for CA B5+B12 and B5+B17 as discussed in [1], as shown in Table 2.2 and 2.3.

Table 2.1 B5+B13 quadplexer specification

| Company | Additional Insertion Loss (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | B5 UL | B5 DL | B13 UL | B13 DL |
| Vendor1 | 0.4 | 0.5 | 0.5 | 0.6 |
| Vendor2 |  |  |  |  |

Table 2.2: ΔTIB,c for B5+B12/17

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_5-12 | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_5-17 | 5 | 0.8 |
|  | 17 | 0.4 |

Table 2.3: ΔRIB,c for B5+B12/17

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_5-12 | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_5-17 | 5 | 0.5 |
|  | 17 | 0.3 |

During the meeting time, the generic approach of handling 2DL CA relaxation values has been discussed and adopted as the way moving forward. [3]

- Single and dual UL assumed

- Low band < 1 GHz

- Low-Low band combination dTib = 0.5 dB for both bands and dRib = 0 dB for both bands

Following this approach, updated relaxation values are listed in Table 2.4 and Table 2.5.

- Table 2.4 ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_5A-13A | 5 | 0.5 |
|  | 13 | 0.5 |

- Table 2.5 ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_5A-13A | 5 | 0 |
|  | 13 | 0 |

## 3	Conclusion

In this contribution, relaxation values for CA B5+B13 has been discussed.

## 4	References

- R4-145256, “Consideration for 2DL CA B5+B13”, Intel Corporate, Verizon Wireless.

- TS36.101

- R4-145843, “Interband CA relaxations”, Nokia Corporation

# Text Proposal for TR36.851 (based on v13.0)

----- Start of TP -----

# 6.3.9 LTE Advanced Carrier Aggregation of Band 5 and Band 13

[Unchanged sections omitted]

##### 6.3.9.1.3		ΔTIB,c and ΔRIB (1 UL)

For the UE which supports CA_5A-13A the ΔTIB,c is defined for applicable bands in table 6.3.9.1.3-1.

Table 6.3.9.1.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_5A-13A | 5 | 0.5 |
|  | 13 | 0.5 |

For the UE which supports CA_5A-13A the ΔRIB is defined for applicable bands in table 6.3.9.1.3-2.

Table 6.3.9.1.3-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_5A-13A | 5 | 0 |
|  | 13 | 0 |