3GPP TSG-RAN4 Meeting #73	 R4-146903

# San Fransisco, USA, 17-21 Novembre, 2014

Source: 	SONY Mobile Communications Japan, Inc.

Title: 	Several UE device types UTRA and E-UTRA bands TRP/TRS Requirements Proposal and in Response to GCF LS

Agenda Item: 	7.1

Document for:	Discussion

_______________________________________________________________________________________________________________________________

- Introduction

In response to the GCF LS R4 146601 this is a proposal on OTA performance figures for all device types and bands. The phone figures have been derived from the current TS 37.144 GSM and UTRA bands Beside Head figures but with an average hand loss added to the min. and avg. numbers. The hand loss figures are obtained from the average difference between BH and BHH results from TRP and TRS measurements respectively on 20 different mobile phones, see R4-146902 for details. The resulting figures were rounded to the nearest 0.5 dB. Recommended figures are aligned close to GSMA recommendations.

For example: TS 37.144 TRP Avg. GSM 900 BH: +20.5 dBm => new spec. BHH: 20.5-4.6=16 dBm.

For UTRA band V and VIII, the figures were adjusted up to be more in line with typical device performance based on submissions.

CTIA work is ongoing to define phantom hands for devices of 72-96 mm in width. Preliminary indications from simulations point to similar loss levels in these hands relative to existing hands. We thereby assume the same OTA requirements will apply regardless of the standardized hand which must be used for testing.

For E-UTRA the nominal UE output power is 1 dB less than for UTRA (23 vs. 24 dBm). This is reflected in the E-UTRA TRP requirements which are adjusted accordingly.

The proposed figures are valid for non CA devices.

For LEE and LME the negotiation process already started at R4 #72bis and the proposed UTRA band I and VIII figures from Intel are presented in this contribution for reference.

GSM bands are not addressed by the RAN 4 WI however requested by GCF.

- TRP, dBm

| Band | Phone, BHH |  |  | Phone, Hand |  |  | Tablet, FS |  |  | LEE/LME, FS |  |  |  |  |  | M2M, FS |  |  | Wearable’s On phantom arm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec |
| GSM 850 | 13.5 | 15.5 | 19 | 19 | 21 | 23 | 25 | 27 | 29 |  |  |  |  |  |  |  |  |  |  |  |  |
| GSM 900 | 14 | 16 | 19 | 19 | 21 | 23 | 25 | 27 | 29 |  |  |  |  |  |  |  |  |  |  |  |  |
| GSM 1800 | 16 | 18 | 20.5 | 17 | 19 | 21 | 22 | 24 | 26 |  |  |  |  |  |  |  |  |  |  |  |  |
| GSM 1900 | 16 | 18 | 20.5 | 17 | 19 | 21 | 22 | 24 | 26 |  |  |  |  |  |  |  |  |  |  |  |  |
| UTRA B I | 10 | 12 | 14.5 | 11 | 13 | 15 | 16 | 18 | 20 | 17.5 | 19 | 21.5 |  |  |  |  |  |  |  |  |  |
| UTRA B II | 10 | 12 | 14.5 | 11 | 13 | 15 | 16 | 18 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| UTRA B IV | 10.5 | 12.5 | 15 | 11 | 13 | 16 | 16 | 18 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| UTRA B V | 6.5 | 8.5 | 11 | 10 | 12 | 14 | 14 | 16 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |
| UTRA B VI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UTRA B VIII | 6.5 | 8.5 | 11 | 10 | 12 | 14 | 14 | 16 | 18 | 16.5 | 18.5 | 21 |  |  |  |  |  |  |  |  |  |
| UTRA B IX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 1 | 9 | 11 | 14 | 8 | 11 | 14 | 14 | 16 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 2 | 9 | 11 | 14 | 8 | 11 | 14 | 14 | 16 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 3 | 9 | 11 | 14 | 8 | 11 | 14 | 14 | 16 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |
| Band | Phone, BHH |  |  | Phone, Hand |  |  | Tablet, FS |  |  | LEE/LME, FS |  |  |  |  |  | M2M, FS |  |  | Wearable´s |  |  |
|  | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec |
| E-UTRA B 4 | 6 | 9 | 14 | 8 | 11 | 14 | 14 | 16 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 5 | 3 | 6 | 11 | 6 | 9 | 12 | 12 | 14 | 16 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 7 | 6 | 9 | 14 | 8 | 11 | 14 | 14 | 16 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 8 | 3 | 6 | 11 | 6 | 9 | 12 | 12 | 14 | 16 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 11 | 3 | 6 | 11 | 6 | 9 | 12 | 10 | 12 | 14 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 12 | 1.5 | 4.5 | 9.5 | 4.5 | 7.5 | 10.5 | 10.5 | 12.5 | 14.5 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 13 | 2 | 5 | 10 | 5 | 8 | 11 | 11 | 13 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 18 | 3 | 6 | 11 | 6 | 9 | 12 | 10 | 12 | 14 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 19 | 3 | 6 | 11 | 6 | 9 | 12 | 10 | 12 | 14 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 20 | 3 | 6 | 11 | 6 | 9 | 12 | 12 | 14 | 16 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 21 | 3 | 6 | 11 | 6 | 9 | 12 | 12 | 14 | 16 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 25 | 9 | 11 | 14 | 8 | 11 | 14 | 14 | 16 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 26 | 3 | 6 | 11 | 6 | 9 | 12 | 10 | 12 | 14 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 27 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 28 | 3 | 5 | 7 | 5 | 8 | 11 | 11 | 13 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 38 | 6 | 9 | 14 | 8 | 11 | 14 | 14 | 16 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 39 | 9 | 11 | 14 | 8 | 11 | 14 | 14 | 16 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |
| Band | Phone, BHH |  |  | Phone, Hand |  |  | Tablet, FS |  |  | LEE/LME, FS |  |  |  |  |  | M2M, FS |  |  | Wearable´s |  |  |
|  | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec | Min | Avg | Rec |
| E-UTRA B 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA B 41 | 6 | 9 | 14 | 8 | 11 | 14 | 14 | 16 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |

- TRS, dBm

For UTRA single antenna condition.

For E-UTRA MIMO devices TRS is intentionally left out as another receiver FOM (TBD) will apply.

| Band | Phone, BHH |  |  | Phone, Hand |  |  | Tablet, FS |  |  | LEE/LME, FS |  |  |  |  |  | M2M, FS |  |  | Wearable´s On phantom arm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max | Avg | Rec | Max | Avg | Rec | Max | Avg | Rec | Max | Avg | Rec | Max | Avg | Rec | Max | Avg | Rec | Max | Avg | Rec |
| GSM 850 | -90 | -93 | -95 | -94 | -97 | -99 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GSM 900 | -89 | -92 | -95 | -93 | -96 | -98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GSM 1800 | -93 | -96 | -99 | -94 | -97 | -100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GSM 1900 | -93 | -96 | -99 | -94 | -97 | -100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UTRA B I | -95 | -98 | -100 | -97 | -100 | -102 | -99 | -101 | -104 | -101.5 | -103 | -106 |  |  |  |  |  |  |  |  |  |
| UTRA B II | -93 | -96 | -98 | -95 | -98 | -100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UTRA B IV | -95 | -98 | -100 | -97 | -100 | -102 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Band | Phone, BHH |  |  | Phone, Hand |  |  | Tablet, FS |  |  | LEE/LME, FS |  |  |  |  |  | M2M, FS |  |  | Wearable´s |  |  |
|  | Max | Avg | Rec | Max | Avg | Rec | Max | Avg | Rec | Max | Avg | Rec | Max | Avg | Rec | Max | Avg | Rec | Max | Avg | Rec |
| UTRA B V | -89 | -93 | -96 | -91 | -94 | -98 | -96 | -98 | -101 |  |  |  |  |  |  |  |  |  |  |  |  |
| UTRA B VI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UTRA B VIII | -89 | -93 | -96 | -92 | -95 | -98 | -97 | -99 | -102 | -99 | -101 | -104 |  |  |  |  |  |  |  |  |  |
| UTRA B IX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E-UTRA TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |