3GPP TSG-RAN WG4 Meeting #73	 R4-147009

# San Francisco, CA, USA, 17 – 21 Nov, 2014

Agenda Item:	7.4.2

Source: 	Samsung

Title: 	Simulation results for PUSCH3-2 sub-band CQI test

Document for:	Discussion

1	Introduction

Performance requirements for eDL-MIMO work item in Rel-12 was widely discussed in recently RAN4 meeting. Regarding PUSCH3-2 CQI test set-up, it was agreed that

- PUSCH 3-2 over PUSCH 3-1 for TM6, TAE = [0, 65ns, 0, 65ns]

  - With Full Band scheduling for PUSCH 3-2 and PUSCH 3-1

  - 4x2 EVA 5, ULA Low.

  - Test points will be selected at SNR < 10 dB.

  - Throughput ratio should ensure sufficient implementation margin based on company contributions

- PUSCH 3-2 over PUSCH 1-2 for TM9, TAE = [0, 0, 0, 0]

  - With best sub-band (PUSCH 3-2) over random sub-band scheduling (PUSCH 1-2)

  - 4x2 EVA 5, XP High.

In this contribution, we provide simulation results for PUSCH 3-2 CQI test.

- Analysis

2.1 TM6 Evaluation

Simulation assumption

The simulation assumption is given in table 1 below as agreed in [1].

Table 1 simulation assumption for TM6

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |
| Transmission mode |  |  | 6 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
| Propagation channel |  |  | EVA5Hz |
| Antenna configuration |  |  | Low ULA 4*2 |
| Time offset between TX antenna |  | ns | [0, 65ns, 0, 65ns] |
| CRS reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15, 16,17,18 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 FFFF |
| Reporting interval |  | ms | 5 |
| CQI delay |  | ms | 8 |
| Reporting mode |  |  | PUSCH 3-2 PUSCH 3-1 |
| Scheduled RB |  | RB | 50 (full band ) |
| Max number of HARQ transmissions |  |  | 1 |

Simulation results

Figures 1 below summarized the TP ratios vs. SNR under EVA5 and low correlation channel.

Figure 1: TP ratio vs. SNR for TM6 EVA Low

Based on simulation results, it is observed that throughput ratio for PUSCH 3-2 over PUSCH3-1 with full band scheduling is between 1.15~1.4 and larger throughput gain was observed in low SNR region.

Proposal 1: For TM6 PUSCH 3-2 test, it is proposed to use SNR points {0,1}dB and test requirements 1.05.

2.2 TM9 Evaluation

Simulation assumption

The simulation assumption is given in table 2 below as agreed in [1].

Table 2 simulation assumption for TM9

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |
| Transmission mode |  |  | 9 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
| Propagation channel |  |  | EVA5Hz |
| Antenna configuration |  |  | High XP 4*2 |
| Beamforming Model |  |  | As specified in Section B.4.3 |
| CRS reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15, 16,17,18 |
| CSI-RS periodicity and sub-frame offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| CSI-RS reference signal configuration |  |  | 4 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 FFFF 0000 FFFF |
| Reporting interval |  | ms | 5 |
| CQI delay |  | ms | 8 |
| Reporting mode |  |  | PUSCH 3-2 PUSCH 1-2 |
| Sub-band size |  | RB | 6 (full size) |
| Max number of HARQ transmissions |  |  | 1 |

Simulation results

Figures 3 below summarized the TP ratios vs. SNR for TM9 under EVA5Hz and XP high channel..

Figure 3: TP ratio vs. SNR for TM9 EVA XP High

Based on simulation results, it is observed that throughput ratio for PUSCH 3-2 with best sub-band scheduling over PUSCH 1-2 with random sub-band scheduling is between 1.3~2.0 with -4dB ~10dB SNR region and relative throughput gain is decreased with SNR increased.

Proposal 2: For TM9 PUSCH 3-2 test, it is proposed to use SNR points {0dB, 1dB} and test requirements 1.15.

- Conclusion

In this contribution, the simulation result is provided for PUSCH 3-2 sub-band CQI test and then the test points and test requirements is proposed for TM6 and TM9 test, as summarized below

- Proposal 1: For TM6 PUSCH 3-2 test, it is proposed to use SNR points {0,1}dB and test requirements 1.05.

- Proposal 2: For TM9 PUSCH 3-2 test, it is proposed to use SNR points {0dB, 1dB} and test requirements 1.15.

- Reference

- R4-146633, “Introduction of PUSCH 3-2 requirements into TS36.101”, Huawei