3GPP TSG-RAN WG4 Meeting #73    	R4-147334

# San Francisco, USA, November 17-21, 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 2699 | rev | - | Current version: | 12.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on measurement for MBSFN MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Network, Nokia Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MBMS_LTE_OS-Core |  |  |  |  |  |  | Date: |  | 2014-10-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | The agreements on MBSFN measurement should be captured in specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 4.4 is added for MBSFN measurement in Idle, and all requirments are specified by reference to those for Connected; Section 8.5 is added for the measurement requirements for MBSFN in Connected; Section 9.8 is added for the accuracy requirements and report mapping of the MBSFN measurement in Connected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements on MBSFN measurement are missing in 36.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 4.4, new section 8.5, new section 9.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of change #1>>

## 4.4	MBSFN measurement

UE supporting MBSFN MDT shall be capable of logging MBSFN RSRP, MBSFN RSRQ and MCH BLER measurement and reporting the logged measurements in RRC_IDLE.

The MDT requirements consist of MBSFN RSRP, MBSFN RSRQ and MCH BLER measurement requirements as specified in section 4.4.1 and relative time stamp accuracy requirements as specified in section 4.4.2.

The requirements are applicable when MDT is performed in RRC_IDLE.

Editor’s Note: UE supporting MBSFN MDT shall be capable of logging MBSFN RSRP, MBSFN RSRQ and MCH BLER measurement and reporting the logged measurements also in RRC_CONNECTED, and same MDT requirements are applied for MDT performed in RRC_IDLE and RRC_CONNECED.

### 4.4.1	Measurement

The measurement performed and logged by the UE for MBSFN MDT shall meet the measurement procedure requirements defined in Section 8.5 and the measurement performance requirements defined in Section 9.8.

### 4.4.2	Relative Time Stamp Accuracy

The relative time stamp for a logged measurement is defined as the time from the moment the MDT configuration was received at the UE until the measurement was logged, see TS 36.331 [2].

The accuracy of the relative time stamping is such that the drift of the time stamping shall be TBD.

<< End of change #1>>

## << Start of change #2>>

## 8.5	MBSFN Measurements

### 8.5.1	Introduction

UE shall measure MBSFN RSRP, MBSFN RSRQ and MCH BLER only in subframes and on carriers where UE is decoding PMCH. The requirements are specified for any carrier where PMCH is received by UE, and the carrier can be same as or different from the serving carrier of the UE.

The UE receiving PMCH on any non-serving carrier and performing MBSFN measurements shall not cause interruptions on any serving carrier in subframes where UE is not decoding PMCH.

The requirements apply irrespective of UE DRX state.

The accuracy requirements and the report mapping of the MBSFN RSRP, MBSFN RSRQ and MCH BLER measurements are specified in section 9.8.

### 8.5.2	MBSFN RSRP Measurements

The UE physical layer shall be capable of reporting MBSFN RSRP measurements to higher layers with the measurement period.

The MBSFN RSRP measurement period is defined as the maximum between [640] ms and the period during which the UE decodes 5 subframes containing PMCH transmissions.

### 8.5.3	MBSFN RSRQ Measurements

The UE physical layer shall be capable of reporting MBSFN RSRQ measurements to higher layers with the measurement period.

The MBSFN RSRQ measurement period is defined as the maximum between [640] ms and the period during which the UE decodes 5 subframes containing PMCH transmissions.

### 8.5.2	MCH BLER Measurements

The UE physical layer shall be capable of reporting MCH BLER measurements to higher layers with the measurement period. The report shall be associated with the number of total MCH blocks received by the UE during measurement period.

The MBSFN RSRP measurement period is equal to the configured logging interval.

<< End of change #2>>

## << Start of change #3>>

## 9.8	MBSFN Measurements

The requirements in this clause apply to UE configured with MBSFN measurement.

The reported measurement result after layer 1 filtering shall be an estimate of the average value of the measured quantity over the measurement period. The reference point for the measurement result after layer 1 filtering is referred to as point B in the measurement model described in [25].

### 9.8.1	MBSFN RSRP

#### 9.8.1.1	Absolute MBSFN RSRP Accuracy requirement

The accuracy requirements in Table 9.8.1.1-1 are valid under the following conditions:

MBSFN reference signals are transmitted.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.1 for a corresponding Band

Table 9.8.1.1-1: MBSFN RSRP absolute accuracy

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 6 | 9 | -6 dB | FDD_A, TDD_A | -121 | N/A | -70 |
|  |  |  | FDD_C, TDD_C | -120 | N/A | -70 |
|  |  |  | FDD_D | -119.5 | N/A | -70 |
|  |  |  | FDD_E, TDD_E | -119 | N/A | -70 |
|  |  |  | FDD_F | -118.5 | N/A | -70 |
|  |  |  | FDD_G | -118 | N/A | -70 |
|  |  |  | FDD_H | -117.5 | N/A | -70 |
|  |  |  | FDD_N | -114.5 | N/A | -70 |
| 8 | 11 | -6 dB | FDD_A, TDD_A, FDD_C, TDD_C, FDD_D, FDD_E, TDD_E, FDD_F, FDD_G, FDD_H, FDD_N | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### 9.8.1.2	MBSFN RSRP Measurement Report Mapping

The reporting range of MBSFN RSRP is defined from -140 dBm to -44 dBm with 1 dB resolution.

The mapping of measured quantity is defined in Table 9.8.1.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table 9.8.1.2-1: MBSFN RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| MBSFN_RSRP_00 | MBSFN_RSRP -140 | dBm |
| MBSFN_RSRP_01 | -140 MBSFN_RSRP < -139 | dBm |
| MBSFN_RSRP_02 | -139 MBSFN_RSRP < -138 | dBm |
| … | … | … |
| MBSFN_RSRP_95 | -46 MBSFN_RSRP < -45 | dBm |
| MBSFN_RSRP_96 | -45 MBSFN_RSRP < -44 | dBm |
| MBSFN_RSRP_97 | -44 MBSFN_RSRP | dBm |

### 9.8.2	MBSFN RSRQ

#### 9.8.2.1	Absolute MBSFN RSRQ Accuracy requirement

The accuracy requirements in Table 9.8.2.1-1 are valid under the following conditions:

MBSFN reference signals are transmitted

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.1 for a corresponding Band

Table 9.8.2.1-1: MBSFN RSRQ absolute accuracy

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| 2.5 | 4 | -3 dB | FDD_A, TDD_A | -121 | -50 |
|  |  |  | FDD_C, TDD_C | -120 | -50 |
|  |  |  | FDD_D | -119.5 | -50 |
|  |  |  | FDD_E, TDD_E | -119 | -50 |
|  |  |  | FDD_F | -118.5 | -50 |
|  |  |  | FDD_G | -118 | -50 |
|  |  |  | FDD_H | -117.5 | -50 |
|  |  |  | FDD_N | -114.5 | -50 |
| 3.5 | 4 | -6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### 9.8.2.2	MBSFN RSRQ Measurement Report Mapping

The reporting range of MBSFN RSRQ is defined from -23 dB to -7.5 with 0.5 dB resolution.

The mapping of measured quantity is defined in table 9.8.2.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table 9.8.2.2-1: RSRQ measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| MBSFN_RSRQ_00 | RSRQ -23 | dB |
| MBSFN_RSRQ_01 | -23 RSRQ < -22.5 | dB |
| MBSFN_RSRQ_02 | -22.5 RSRQ < -22 | dB |
| … | … | … |
| RSRQ_30 | -8.5 RSRQ < -8 | dB |
| RSRQ_31 | -8 RSRQ < -7.5 | dB |

### 9.8.3	MCH BLER

MCH BLER shall be measured as defined in [4] and reported together with the number of total MCH blocks received by the UE during measurement period. The reporting range of MCH BLER is defined from [0.1%] to [50%].

The mapping of measured quantity is TBD.

<< End of change #3>>