3GPP TSG-RAN WG4 #74	R4-150675

Athens, Greece, 9th Feb – 13th Feb 2015

Source: 	Ericsson

Title:	Consideration on simulation setup for CRS-IM demodulation (non-COMP)

Agenda Item:	7.5.2

Document for:	Discussion

# Introduction

In RAN#66, LTE_CRSIM-Perf WI is approved [1].  In one companion paper [2], the general overview on the test purposes, target scenarios, and test case list are proposed. In this paper, we provide our detail setup for different test cases.

# Consideration on simulation setup for control channel

## PDCCH/PCFICH

Table 8.4.1.2.4-1: Test Parameters for PDCCH/PCFICH – Non-MBSFN ABS

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 | -3 |
| at antenna port | Noc | dBm/15kHz | -98 | N/A | N/A |
|  |  | dB | Reference Value in Table 8.4.1.2.4-2 | TBD | TBD |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 128 | 1 |
| Number of control OFDM symbols |  |  | 2 | Note 7 | Note 7 |
| Number of PHICH groups (Ng) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 8: The number of the CRS ports in Cell1, Cell2 and Cell 3 is the same. Note 9: SIB-1 will not be transmitted in Cell2 and Cell 3 in the test. |  |  |  |  |  |

Table 8.4.1.2.4-2: Minimum performance PDCCH/PCFICH – Non-MBSFN ABS

| Test Number |  | Aggregation Level |  | Reference Channel |  | OCNG Pattern |  |  |  |  |  | Propagation Conditions (Note 1) |  |  |  |  |  | Correlation Matrix and Antenna Configuration (Note 2) |  | Reference Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 1 |  | Cell 2 |  | Cell 3 |  | Cell 1 |  | Cell 2 |  | Cell3 |  |  |  | Pm-dsg (%) |  | SNR (dB) (Note 3) |  |
| 1 |  | 2 or 1 CCE |  | R.15-2 FDD |  | OP.1 FDD |  | OP.1 FDD |  | OP.1 FDD |  | EVA5 |  | EVA5 |  | EVA5 |  | 2x2 Low |  | 1 |  | TBD |  |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## PHICH

Table 8.5.1.2.4-1: Test Parameters for PHICH

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 | -3 | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 | -3 | -3 |
| at antenna port | Noc | dBm/15kHz | -98 (Note 1) | N/A | N/A |
|  |  | dB | Reference Value in Table 8.5.1.2.4-2 | TBD | TBD |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  | 0 | 128 | 1 |
| PDCCH Content |  |  | UL Grant should be included with the proper information aligned with A.3.6. | N/A | N/A |
| Number of control OFDM symbols |  |  | 2 | Note 7 | Note 7 |
| Number of PHICH groups (Ng) |  |  | 1 | N/A | N/A |
| PHICH duration |  |  | Normal | N/A | N/A |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG |
| Cyclic prefix |  |  | Normal | Normal | Normal |
| Note 8: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 9: SIB-1 will not be transmitted in Cell 2 and Cell 3 in the test. |  |  |  |  |  |

Table 8.5.1.2.4-2: Minimum performance PHICH

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note 1) |  |  | Antenna Configuration and Correlation Matrix (Note 2) | Reference Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Pm-an (%) | SNR (dB) (Note 3) |
| 1 | R.19 | OP.1 FDD | OP.1 FDD | OP.1 FDD | EPA5 | EVA5 | EVA5 | 2x2 Low | 0.1 | TBD |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |  |

# Consideration on simulation setup for PDSCH

## CRS-based transmission scheme (TM2 or TM3)

Table 7.2-1: Link level simulation assumptions for CRS-IM investigations in homogeneous deployments

| Parameter |  |  |  | Value |  |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  |  | 2 GHz |  |
| System bandwidth |  |  |  | 10 MHz |  |
| Duplex mode |  |  |  | FDD |  |
| Transmission mode in serving cell |  |  |  | TM2 |  |
| Transmission mode in interfering cells |  |  |  | Use TM3 for TM2 serving |  |
| MIMO configuration |  |  |  | 2x2, low correlation |  |
| Channel model and Doppler frequency for target and interfering cells |  |  |  | EVA5 |  |
|  |  |  |  | Use different channel seed for between cells |  |
| Number of explicitly modeled interfering cells |  |  |  | 2 interfering cells |  |
| Signal level for serving cell CRS (Es/Noc) |  |  |  | [0:2:20] |  |
| Signal level for interfering cells CRS (interference over Noc) |  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  |  | INR1(Table 7.3-1) | INR2(Table 7.3-1) |
| Other cells interference |  |  |  | AWGN with 1 Noc level |  |
| Network synchronization in time |  |  |  | All cells are synchronous |  |
|  |  |  |  | Time-delay wrt. serving cell |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  |  | [3 us] | [-1 us] |
| Network synchronization in frequency |  |  |  | Frequency shift wrt. serving cell |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  |  | 300 Hz | -100 Hz |
| CRS configuration |  |  |  | 2 CRS ports per cell with planning, non-colliding CRS between explicitly modeled serving and the first two interfering cells |  |
| Downlink power allocation (cf. Chapter 8 of TS36.101) | A |  |  | -3 dB in all modeled cells |  |
|  | B |  |  | -3 dB in all modeled cells (PB=1) |  |
|  |  |  |  | 0 dB in all modeled cells |  |
| CSI reference signals |  |  |  | N/A |  |
| CSI-RS periodicity and subframe offset (TCSI-RS / ICSI-RS) |  |  |  | N/A |  |
| CSI reference signal configuration |  |  |  | N/A |  |
| Subframes for demodulation |  |  |  | All subframes scheduled for demodulation except subframe #5 |  |
| HARQ |  |  |  | 8 HARQ processes and max 4 transmissions |  |
| Feedback mode |  |  |  | None |  |
| Channel and interference estimation at UE |  |  |  | Practical and realizable channel and interference covariance estimates with no a-priori knowledge of the channel state information |  |
| Time/frequency tracking at the UE |  |  |  | Practical algorithms should be used |  |
| Physical channels transmitted in serving cell |  |  |  | PSS/SSS/PBCH |  |
| PCFICH |  |  |  | CFI = 2 in all cells |  |
| PCFICH/PDCCH detection |  |  |  | Not considered |  |
| Physical channels transmitted in interfering cells |  |  |  | PDCCH PDSCH PSS/SSS/PBCH |  |
| Desired PDSCH parameterization |  | Resource allocation |  | 50 PRB |  |
|  |  | Rank |  | Rank-1 |  |
|  |  | PMI |  | TM2: N/A |  |
|  |  | Modulation |  | MCS=19, the information bits are 18336 bits for the subframes other than subframe#5 |  |
|  |  | Code rate |  |  |  |
|  |  | Channel coding, rate matching |  | As specified in TS36.212 |  |
|  |  | CRC |  |  |  |
| Interfering PDSCH parameterization |  | Resource allocation |  | Random full band (50PRB) on/off model, proportional to the average resource utilization in the interfering cells |  |
|  |  | Rank |  | Randomly changing rank per allocated subband from subframe to subframe: 80% rank-1, 20% rank-2 |  |
|  |  | PMI |  | TM3: N/A |  |
|  |  | Modulation |  | Randomly modulated 16QAM symbols over allocated interfering resources |  |
|  |  | Code rate |  | - |  |
|  |  | Channel coding, rate matching |  | - |  |
|  |  | CRC |  |  |  |
| Non-full buffer interference |  | Model |  | Interfering PDSCH transmissions in interfering cells are randomly & independently active over the full band with an activity in time domain equal on average to the targeted resource utilization |  |
|  |  | Average resource utilization |  | { 30%} |  |
| Tx EVM |  |  |  | 6% in both alignment and impairment simulations |  |
| Noc at antenna port |  |  |  | [-98 dBm] |  |
| Cyclic prefix |  |  |  | Normal |  |
| Simulation length |  |  |  | 10000 sub-frames at minimum |  |
| Simulation output |  |  |  | PDSCH throughput vs. serving cell Es/Noc |  |
| UE receiver |  |  | Detector | MMSE-IRC with CRS based interference covariance estimation as defined in TR36.829 |  |
|  |  |  | CRS-IM | Without CRS-IM |  |
|  |  |  |  | With CRS-IM for the 1st and 2nd strongest interfering cells |  |

## CRS-based transmission scheme (Close-loop MIMO TM4 or TM6)

Table 1: Link level simulation assumptions for CRS-IM investigations in homogeneous deployments (TM4 or TM6)

| Parameter |  |  |  | Value |  |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  |  | 2 GHz |  |
| System bandwidth |  |  |  | 10 MHz |  |
| Duplex mode |  |  |  | FDD |  |
| Transmission mode in serving cell |  |  |  | TM6 |  |
| Transmission mode in interfering cells |  |  |  | Use TM3 for TM2 serving |  |
| MIMO configuration |  |  |  | 2x2, low correlation |  |
| Channel model and Doppler frequency for target and interfering cells |  |  |  | EVA5 |  |
|  |  |  |  | Use different channel seed for between cells |  |
| Number of explicitly modeled interfering cells |  |  |  | 2 interfering cells |  |
| Signal level for serving cell CRS (Es/Noc) |  |  |  | [0:2:20] |  |
| Signal level for interfering cells CRS (interference over Noc) |  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  |  | INR1=9.7 dB | INR2 = 3.7 dB |
| Other cells interference |  |  |  | AWGN with 1 Noc level |  |
| Network synchronization in time |  |  |  | All cells are synchronous |  |
|  |  |  |  | Time-delay wrt. serving cell |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  |  | [3 us] | [-1 us] |
| Network synchronization in frequency |  |  |  | Frequency shift wrt. serving cell |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  |  | 300 Hz | -100 Hz |
| CRS configuration |  |  |  | 2 CRS ports per cell with planning, non-colliding CRS between explicitly modeled serving and the first two interfering cells |  |
| Downlink power allocation (cf. Chapter 8 of TS36.101) | A |  |  | -3 dB in all modeled cells |  |
|  | B |  |  | -3 dB in all modeled cells (PB=1) |  |
|  |  |  |  | 0 dB in all modeled cells |  |
| CSI reference signals |  |  |  | N/A |  |
| CSI-RS periodicity and subframe offset (TCSI-RS / ICSI-RS) |  |  |  | N/A |  |
| CSI reference signal configuration |  |  |  | N/A |  |
| Subframes for demodulation |  |  |  | All subframes scheduled for demodulation except subframe #5 |  |
| HARQ |  |  |  | 8 HARQ processes and max 4 transmissions |  |
| Feedback mode |  |  |  | None |  |
| Channel and interference estimation at UE |  |  |  | Practical and realizable channel and interference covariance estimates with no a-priori knowledge of the channel state information |  |
| Time/frequency tracking at the UE |  |  |  | Practical algorithms should be used |  |
| Physical channels transmitted in serving cell |  |  |  | PSS/SSS/PBCH |  |
| PCFICH |  |  |  | CFI = 2 in all cells |  |
| PCFICH/PDCCH detection |  |  |  | Not considered |  |
| Physical channels transmitted in interfering cells |  |  |  | PDCCH PDSCH PSS/SSS/PBCH |  |
| Desired PDSCH parameterization |  | Resource allocation |  | 50 PRB |  |
|  |  | Rank |  | Rank-1 |  |
|  |  | PMI |  | TM2: N/A |  |
|  |  | Modulation |  | MCS=19, the information bits are 18336 bits for the subframes other than subframe#5 |  |
|  |  | Code rate |  |  |  |
|  |  | Channel coding, rate matching |  | As specified in TS36.212 |  |
|  |  | CRC |  |  |  |
| Interfering PDSCH parameterization |  | Resource allocation |  | Random full band (50PRB) on/off model, proportional to the average resource utilization in the interfering cells |  |
|  |  | Rank |  | Randomly changing rank per allocated subband from subframe to subframe: 80% rank-1, 20% rank-2 |  |
|  |  | PMI |  | TM3: N/A |  |
|  |  | Modulation |  | Randomly modulated 16QAM symbols over allocated interfering resources |  |
|  |  | Code rate |  | - |  |
|  |  | Channel coding, rate matching |  | - |  |
|  |  | CRC |  |  |  |
| Non-full buffer interference |  | Model |  | Interfering PDSCH transmissions in interfering cells are randomly & independently active over the full band with an activity in time domain equal on average to the targeted resource utilization |  |
|  |  | Average resource utilization |  | {30%} |  |
| Tx EVM |  |  |  | 6% in both alignment and impairment simulations |  |
| Noc at antenna port |  |  |  | [-98 dBm] |  |
| Cyclic prefix |  |  |  | Normal |  |
| Simulation length |  |  |  | 10000 sub-frames at minimum |  |
| Simulation output |  |  |  | PDSCH throughput vs. serving cell Es/Noc |  |
| UE receiver |  |  | Detector | MMSE-IRC with CRS based interference covariance estimation as defined in TR36.829 |  |
|  |  |  | CRS-IM | Without CRS-IM |  |
|  |  |  |  | With CRS-IM for the 1st and 2nd strongest interfering cells |  |

## DMRS-based transmission scheme (TM9)

Table 7.2-1: Link level simulation assumptions for CRS-IM investigations in homogeneous deployments

| Parameter |  |  |  | Value |  |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  |  | 2 GHz |  |
| System bandwidth |  |  |  | 10 MHz |  |
| Duplex mode |  |  |  | FDD |  |
| Transmission mode in serving cell |  |  |  | TM9 |  |
| Transmission mode in interfering cells |  |  |  | Use TM3 for interference cell |  |
| MIMO configuration |  |  |  | 2x2, low correlation |  |
| Channel model and Doppler frequency for target and interfering cells |  |  |  | EVA5 |  |
|  |  |  |  | Use different channel seed for between cells |  |
| Number of explicitly modeled interfering cells |  |  |  | 2 interfering cells |  |
| Signal level for serving cell CRS (Es/Noc) |  |  |  | [0:2:20] |  |
| Signal level for interfering cells CRS (interference over Noc) |  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  |  | INR1=9.7 dB | INR2 = 3.7 dB |
| Other cells interference |  |  |  | AWGN with 1 Noc level |  |
| Network synchronization in time |  |  |  | All cells are synchronous |  |
|  |  |  |  | Time-delay wrt. serving cell |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  |  | [3 us] | [-1 us] |
| Network synchronization in frequency |  |  |  | Frequency shift wrt. serving cell |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  |  | 300 Hz | -100 Hz |
| CRS configuration |  |  |  | 2 CRS ports per cell with planning, non-colliding CRS between explicitly modeled serving and the first two interfering cells |  |
| Downlink power allocation (cf. Chapter 8 of TS36.101) | A |  |  | -3 dB in all modeled cells |  |
|  | B |  |  | -3 dB in all modeled cells (PB=1) |  |
|  |  |  |  | 0 dB in all modeled cells |  |
| CSI reference signals |  |  |  | N/A |  |
| CSI-RS periodicity and subframe offset (TCSI-RS / ICSI-RS) |  |  |  | N/A |  |
| CSI reference signal configuration |  |  |  | N/A |  |
| Subframes for demodulation |  |  |  | All subframes scheduled for demodulation except subframe #5 |  |
| HARQ |  |  |  | 8 HARQ processes and max 4 transmissions |  |
| Feedback mode |  |  |  | None |  |
| Channel and interference estimation at UE |  |  |  | Practical and realizable channel and interference covariance estimates with no a-priori knowledge of the channel state information |  |
| Time/frequency tracking at the UE |  |  |  | Practical algorithms should be used |  |
| Physical channels transmitted in serving cell |  |  |  | PSS/SSS/PBCH |  |
| PCFICH |  |  |  | CFI = 2 in all cells |  |
| PCFICH/PDCCH detection |  |  |  | Not considered |  |
| Physical channels transmitted in interfering cells |  |  |  | PDCCH PDSCH PSS/SSS/PBCH |  |
| Desired PDSCH parameterization |  | Resource allocation |  | 50 PRB |  |
|  |  | Rank |  | Rank-1 |  |
|  |  | PMI |  | TM2: N/A |  |
|  |  | Modulation |  | MCS=19, the information bits are 18336 bits for the subframes other than subframe#5 |  |
|  |  | Code rate |  |  |  |
|  |  | Channel coding, rate matching |  | As specified in TS36.212 |  |
|  |  | CRC |  |  |  |
| Interfering PDSCH parameterization |  | Resource allocation |  | Random full band (50PRB) on/off model, proportional to the average resource utilization in the interfering cells |  |
|  |  | Rank |  | Randomly changing rank per allocated subband from subframe to subframe: 80% rank-1, 20% rank-2 |  |
|  |  | PMI |  | TM3: N/A |  |
|  |  | Modulation |  | Randomly modulated 16QAM symbols over allocated interfering resources |  |
|  |  | Code rate |  | - |  |
|  |  | Channel coding, rate matching |  | - |  |
|  |  | CRC |  |  |  |
| Non-full buffer interference |  | Model |  | Interfering PDSCH transmissions in interfering cells are randomly & independently active over the full band with an activity in time domain equal on average to the targeted resource utilization |  |
|  |  | Average resource utilization |  | {30%} |  |
| Tx EVM |  |  |  | 6% in both alignment and impairment simulations |  |
| Noc at antenna port |  |  |  | [-98 dBm] |  |
| Cyclic prefix |  |  |  | Normal |  |
| Simulation length |  |  |  | 10000 sub-frames at minimum |  |
| Simulation output |  |  |  | PDSCH throughput vs. serving cell Es/Noc |  |
| UE receiver |  |  | Detector | MMSE-IRC with CRS based interference covariance estimation as defined in TR36.829 |  |
|  |  |  | CRS-IM | Without CRS-IM |  |
|  |  |  |  | With CRS-IM for the 1st and 2nd strongest interfering cells |  |

# Conclusion

In this contribution, the detail setup for different reference channels is provided. We hope the group can consider these setups in the test setup discussion.

# Reference

- RP-142263, New Work Item proposal: Perf. Part: CRS Interference Mitigation for LTE Homogenous Deployments, Ericsson, Dec. 2014.

- R4-14xxxx, Overview for CRS-IM WI, Feb. 2015.