3GPP TSG-RAN WG4 Meeting #75 	R4-153219

Fukuoka, Japan, 25-29 May, 2015

Source:	DISH

Title:	MTC operating bands

Work item	LTE_MTC2_L1 Core

Agenda item:	7.7.3

Document for:	Approval

## Abstract

This document looks how band information is specified in the TS36.101 specifications for UE category 0 or MTC and proposes a new Category O band

## Background

In TS36.101 subclause 4.3A, Suffix E has been allocated to specify the additional requirements need to support UE category 0 where a category 0 UE is designated to support MTC. This is shown below

However the normal procedure when we define a suffix is to also indicate which band would support this additional feature. However, we note there is no operating band defined for UE category O as shown below in Section 5 shown below.

The only band information for a UE category O is provided in the subclause 7.3.1E (Minimum requirements (QPSK) for UE category O) which is copied below

In this case Table 7.3.1E-1A and Table 7.3.1E-1B shows B2, B3, B4, B5, B8, B13, B20, B39 and B41 are the bands defined which will support a Category 0 UE

## Discussion

The focus of the work item as defined [1] is to specify 1.4 MHz operation at the terminal within any LTE system bandwidth, allowing operators to multiplex reduced bandwidth MTC devices and regular devices in their existing LTE deployments. Continuing the normative work started in Release 12, the key focus for Release 13 is to define a new low complexity UE category type that supports reduced bandwidth, reduced transmit power, reduced support for downlink transmission modes, ultra-long battery life via power consumption reduction techniques and extended coverage operation. In this case, sufficient clarity should be provided in the specification to indicate which bands support category 0 devices in TS 36.101 Section 5 (Operating bands and channel arrangements)

At the last R4 meeting there were ongoing discussion relating to UE maximum transmission power level for new Power Class. Furthermore, as the work progresses in R4 many RF parameters will need to be uniquely linked with category 0 devices.  Therefore, it is proposed that a new sub-clause is created in Section 5 called 5.5D (Operating band for category O devices). In the future we can also add future suffix E sub clauses in other areas i.e. for category E power classes. Based on this approach we propose adding the following Table to TS36.101

## 5.5E	Operating bands for Category 0 devices

E-UTRA Category 0 devices is designed to operate in the operating bands defined in Table 5.5E-1.

| E-UTRA Cat. 0 Band | E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  | FDD | HD-FDD | TDD |
| 2 | 2 | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | YES | YES |  |
| 3 | 3 | 1710 MHz | – | 1785 MHz | 1710 MHz | – | 1785 MHz | YES | YES |  |
| 4 | 4 | 1710 MHz | – | 1755 MHz | 1710 MHz | – | 1755 MHz | YES | YES |  |
| 5 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | YES | YES |  |
| 8 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | YES | YES |  |
| 13 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | YES | YES |  |
| 20 | 20 | 832 MHz | – | 862 MHz | 832 MHz | – | 862 MHz |  |  |  |
| 39 | 39 | 1880 MHz | – | 1920 MHz | 1920 MHz | – | 748 MHz |  |  | YES |
| 41 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz |  |  | YES |

In this case there is clear identity of which bands support a category O device and the associated duplex mode of operation.

## New Band supporting Category 0 Devices

From the work item objectives we note the key MTC goal is to improve by 15dB the coverage of delay-tolerant MTC devices, allowing operators to reach MTC devices in poor coverage conditions. Two of the issues associated with MSS application are obtaining sufficient link budget to address coverage and also addressing the increase in latency associated with large link distances which would be of significant value. For bands which support both satellite and terrestrial services. Since B65 will support both MSS and CGC services we propose to add B65 to the list of support bands

| E-UTRA Cat. 0 Band | E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  | FDD | HD-FDD | TDD |
| 65 | 65 | 1920 MHz | – | 2010 MHz | 2100 MHz | – | 2200 MHz | YES | YES |  |

## References

[1]  	RP-141865 : Further LTE Physical Layer Enhancements for MTC