3GPP TSG-RAN WG4 (Radio) Meeting #75	R4-153373

Fukuoka, Japan, 25 – 29 May, 2015

Agenda Item:	8.2.3

Source: 	Alcatel-Lucent

Title: 	BS RF requirements for AWS-Extension Band for LTE

Document for:	Approval

# 1.	Introduction

The Work Item (WI) proposal to specify the AWS-Extension Band for LTE was approved in RAN#67 [1]. One objective of the WI is to specify the band specific Base Station (BS) Radio Frequency (RF) requirements for the new operating band. A Technical Report (TR) has been created to collect appropriate information regarding the WI [2], in which the required changes to the E-UTRA BS TS 36.104 [3] and TS 36.141 [4] are listed.

In this paper, we study the specific E-UTRA BS RF requirements for the new operating band, and provide a text proposal to record the findings in the TR.

# 2.	Text Proposal

<Start of change>

# 8	Study of E-UTRA specific issues

### 8.1	Operating band unwanted emissions

The operating band unwanted emission limits are defined from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band. The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier) and for all transmission modes foreseen by the manufacturer's specification.

As stated in the FCC Order (FCC-14-31) [4], the BS general emission limits mandated by the FCC for the AWS-3 spectrum are the same as those of the AWS-1 (E-UTRA Band 4) and AWS-4 (E-UTRA Band 23), i.e. attenuate out-of-block emissions by a factor of (43 + 10*log(P) dB) below the transmitter power (P) for a resolution bandwidth of 1 MHz or greater. Therefore, the general limits for Category A and the additional limits for E-UTRA bands >1GHz as specified in Clause 6.6.3 of TS 36.104 and TS 36.141 for E-UTRA Bands 2 and 4 shall also be applied for Band 66. Note that the general limits for Category B are not applicable in North America, and thus need not be specified for Band 66.

### 8.2	Additional spurious emissions requirements

These requirements may be applied for the protection of system operating in frequency ranges other than the E-UTRA BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the E-UTRA BS, or they may be set by local or regional regulation as a mandatory requirement for an E-UTRA operating band.

The E-UTRA BS spurious emissions limits currently specified for co-existence with systems operating in other frequency bands are more stringent than the general emissions limits by the FCC. To ensure sufficient protection to the co-existing systems, the same limits as the other E-UTRA bands BS shall be specified for Band 66 in Clause 6.6.4.3 of TS 36.104 and Clause 6.6.4.5.4 of TS 36.141. And to ensure sufficient protection from the co-existing systems, the same limits as the other E-UTRA bands BS shall be specified for coexistence with the BS in Band 66 in Clause 6.6.4.3 of TS 36.104 and Clause 6.6.4.5.4 of TS 36.141. This is shown in Tables 8.2-1 and 8.2-2 below (for TS 36.104).

Table 8.2-1: BS Spurious emissions limits for E-UTRA BS for co-existence with systems operating in other frequency bands

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 66 | 2110 - 2200 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 4, 10 or 66. |
|  | 1710 - 1780 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 66, since it is already covered by the requirement in sub-clause 6.6.4.2. For E-UTRA BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in sub-clause 6.6.4.2. For E-UTRA BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in sub-clause 6.6.4.2. |

Table 8.2-2: Home BS Spurious emissions limits for co-existence with Home BS operating in other frequency bands

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 66 | 1710 - 1780 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 66, since it is already covered by the requirement in sub-clause 6.6.4.2. For Home BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in sub-clause 6.6.4.2. For Home BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in sub-clause 6.6.4.2. |

### 8.3	Co-location with other base stations

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, UTRA FDD, UTRA TDD and/or E-UTRA BS are co-located with an E-UTRA BS. The requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class.

The E-UTRA BS spurious emissions limits currently specified for co-location with another BS are more stringent than the general emissions limits by the FCC. To ensure sufficient protection to the co-located BS, the same limits as the other E-UTRA bands BS shall be specified for Band 66 in Clause 6.6.4.4 of TS 36.104 and Clause 6.6.4.5.5 of TS 36.141. And to ensure sufficient protection from the co-located BS, the same limits as the other E-UTRA bands BS shall be specified for co-location with the BS in Band 66 in Clause 6.6.4.4 of TS 36.104 and Clause 6.6.4.5.5 of TS 36.141. This is shown in Tables 8.3-1 to 8.3-3 below.

Table 8.3-1: BS Spurious emissions limits for Wide Area BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| WA E-UTRA Band 66 | 1710 - 1780 MHz | -96 dBm | 100 kHz |  |

Table 8.3-2: BS Spurious emissions limits for Local Area BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| LA E-UTRA Band 66 | 1710 - 1780 MHz | -88 dBm | 100 kHz |  |

Table 8.3-3: BS Spurious emissions limits for Medium range BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| MR E-UTRA Band 66 | 1710 - 1780 MHz | -91 dBm | 100 KHz |  |

### 8.4	General blocking requirement

The blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer, which are either a 1.4MHz, 3MHz or 5MHz E-UTRA signal for in-band blocking or a CW signal for out-of-band blocking.

The general blocking requirements currently specified for E-UTRA BS apply the in-band blocking signal within +/-20 MHz of the UL frequency range of a band, except when the DL frequency range of the same band falls within such range. Since the duplex distance of Band 66 is larger than 20 MHz and thus no exception is needed, the same general blocking requirements as specified for E-UTRA Bands 4 and 23 BS shall be specified for Band 66 in Clause 7.6.1 of TS 36.104 and Clause 7.6.5.1 of TS 36.141 to ensure sufficient BS receiver blocking ability.

### 8.5	Blocking requirement for co-location with other base stations

This additional blocking requirement may be applied for the protection of E-UTRA BS receivers when GSM, UTRA or E-UTRA BS operating in a different frequency band are co-located with an E-UTRA BS. The requirements assume a 30 dB coupling loss between interfering transmitter and E-UTRA BS receiver and are based on co-location with base stations of the same class.

The blocking requirements currently specified for E-UTRA BS when co-located with BS in other frequency bands do not apply when the interfering signal falls within +/-10 MHz of the UL frequency range of a band, except for Band 13 in order to allow co-location with Band 14 BS. Since no DL frequency range of BS in other frequency bands is within +/-10 MHz of the UL frequency range of Band 66 and thus no exception is needed, the blocking requirements for co-location with other BS shall be specified for Band 66 in Clause 7.6.2 of TS 36.104 and Clause 7.6.5.2 of TS 36.141 using the same interfering signal as the other E-UTRA bands BS. This is shown in Tables 8.5-1 to 8.5-3 below.

Table 8.5-1: Blocking performance requirement for E-UTRA Wide Area BS when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| WA E-UTRA Band 66 | 1710 - 1780 | +16 | PREFSENS + 6dB* | CW carrier |

Table 8.5-2: Blocking performance requirement for Local Area BS when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| LA E-UTRA Band 66 | 1710 - 1780 | -6 | PREFSENS + 6dB* | CW carrier |

Table 8.5-3: Blocking performance requirement for E-UTRA Medium Range BS when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| MR E-UTRA Band 66 | 1710 - 1780 | +8 | PREFSENS + 6dB* | CW carrier |

<End of change>

# References

[1]	RP-150428, “New WI: AWS-Extension Band for LTE”, Ericsson.

[2]	R4-152970, “TR 36.869, "AWS extension for LTE"”, Ericsson.

[3]	3GPP TS 36.104 V12.7.0 (2015-03), “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.

[4]	3GPP TS 36.141 V12.7.0 (2015-03), “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.