3GPP TSG-RAN Working Group 4 Meeting #75	R4-153590

Fukuoka, Japan, 25th May – 29th May 2015

Source:	DISH Network

Title:	TP for TR 36.869: Section 8.1 UE specific

Agenda item:	8.2.2

Document for:	Approval

## 1	Introduction

In this contribution, new filter data for the AWS extension band plan (1710-1780MHz UL/2110-2200MHz DL) is presented.

## 2	Discussion

Newer simulated filter data for the AWS extension band (1710-1780MHz UL/2110-2200MHz DL) is shown in Table 1.  The data provided is all under extreme temperature condition (ETC).

Table 1.  Simulated filter data for AWS extension band

|  | TX IL Max | RX IL Max | TX Isolation Min | RX Isolation Min |
| --- | --- | --- | --- | --- |
| Vendor A | 1.3 dB | 1.8 dB | 60 dB | 60 dB |
| Vendor B | 2.3 dB | 2.9 dB | 55 dB | 50 dB |
| Vendor C | 2.5 dB | 3.2 dB | 45 dB | 50 dB |
| Vendor D | 2.5 dB | 2.8 dB | 52 dB | 48 dB |
| Average | 2.2 dB | 2.7 dB | 53 dB | 52 dB |

In general, an improvement in filter performance is observed over the ones collected during the SI phase [2].

## 3 				Conclusions

It is proposed that the above filter data be included in Section 8.1 of TR 36.869 [1].

## References

R4-152443, “Skeleton TR 36.869 v0.0.2,” Ericsson, 3GPP RAN4 #74bis, Rio, Brazil, April 2015

RP-141971, “ TR 36 v1.0.0 Study on Advanced Wireless Services (AWS) extension band for LTE,” Qualcomm, 3GPP RAN #66, Maui, U. S. A. December 2014

## << Unchanged sections omitted >>

## << Start of changed sections >>

# 8	Study of E-UTRA specific issues

## 8.1			UE specific (stand alone band)

Newer simulated filter data for the AWS extension band (1710-1780MHz UL/2110-2200MHz DL) is shown in Table 8.1-1.  The data provided is all under extreme temperature condition (ETC).

Table 8.1-1.  Simulated filter data for AWS extension band

|  | TX IL Max | RX IL Max | TX Isolation Min | RX Isolation Min |
| --- | --- | --- | --- | --- |
| Vendor A | 1.3 dB | 1.8 dB | 60 dB | 60 dB |
| Vendor B | 2.3 dB | 2.9 dB | 55 dB | 50 dB |
| Vendor C | 2.5 dB | 3.2 dB | 45 dB | 50 dB |
| Vendor D | 2.5 dB | 2.8 dB | 52 dB | 48 dB |
| Average | 2.2 dB | 2.7 dB | 53 dB | 52 dB |

In general, an improvement in filter performance is observed over the ones collected during the SI phase [2].

## << End of changed sections >>