3GPP TSG-RAN WG4 Meeting #75	   					R4-153664

Fukuoka, Japan, 25 - 29 May, 2015

Source:	China Telecom

Title:	TP on simulation results for BS MMSE-IRC interference modelling

Agenda Item:	7.6.2

Document for:	Approval

Introduction

For BS MMSE-IRC WI, the system simulation assumptions and methodologies to derive interference profiles have been agreed in previous meetings [1-4]. Moreover, it has been agreed in RAN4 #74bis meeting that, interested companies are encouraged to provide the following results for both homogeneous and heterogeneous scenarios:

  - (a) Raw data on UL wideband SINR distribution, unconditional DIP1/2 distributions, for calibration purpose.

  - (b) The DIP1/2 values using the methodologies shown in page 3, which will be taken into account for deriving the final DIP values for link evaluation.

This contribution provides a text proposal to capture companies’ simulation results for round robin scheduling, based on the summary spreadsheet in [5].

Reference

[1]	R4-151139, “System-level simulation assumptions for Homogeneous deployment for BS MMSE-IRC,” ZTE, China Telecom, Huawei, Samsung, RAN4 #74, Feb 2015.

[2]	R4-151140, “System-level simulation assumptions for Heterogeneous deployment for BS MMSE-IRC,” ZTE, China Telecom, Huawei, Samsung, RAN4 #74, Feb 2015.

[3]	R4-152404, “WF on system simulation assumptions for BS MMSE-IRC receiver,” ZTE, China Telecom, Huawei, Ericsson, Nokia Networks, Samsung, Alcatel-Lucent, RAN4 #74bis, Apr 2015.R4-151091, “Way forward on MMSE-IRC for LTE BS,” ZTE, China Telecom, Huawei, Samsung, RAN4 #74, Feb 2015.

[4]	R4-152525, “WF on interference modelling for BS MMSE-IRC receiver,” China Telecom, Huawei, Nokia Networks, ZTE, Samsung, Alcatel-Lucent, RAN4 #74bis, Apr 2015.

[5]	R4-153665, “Summary of simulation results for BS MMSE-IRC interference modelling,” China Telecom, RAN4 #75, May 2015.

Text Proposal

<< Start of text proposal >>

- 6.3	System level simulator calibration

To ensure that different system level simulators produce comparable results, UL wideband SINR and unconditional DIP distributions have been evaluated and calibrated. The evaluation results from various interested companies are summarized in this clause. Sub-clause 6.3.1 and 6.3.2 present the results for homogeneous network and heterogeneous network respectively. The simulation assumptions in clause 5 are used.

- 6.3.1	Homogeneous network

6.3.1.1	UL wideband SINR distribution

Company inputs are captured in Figure 6.3.1.1-1 based on raw data in Table 6.3.1.1-1.

Figure 6.3.1.1-1: UL wideband SINR calibration for homogeneous network

Table 6.3.1.1-1: Company inputs on UL wideband SINR calibration for homogeneous network

| Percentile | China Telecom | Huawei | Ericsson | ZTE | Nokia Networks | Alcatel-Lucent |
| --- | --- | --- | --- | --- | --- | --- |
|  | R4-152823 | R4-152596 | R4-152958 | R4-153161 | R4-152236 | R4-153481 |
| 0.00 | -12.936373 | -16.3074 | -9.577345629 | -11.66 | -16.6532 | -12.641 |
| 1.00 | -5.601403 | -10.7635 | -4.304865784 | -5.56 | -8.2105 | -4.814 |
| 2.00 | -4.535233 | -6.2742 | -3.388254755 | -4.49 | -6.878 | -3.776 |
| 3.00 | -3.816456 | -4.5613 | -2.703500241 | -3.77 | -5.5119 | -3.093 |
| 4.00 | -3.250735 | -3.2998 | -2.20848104 | -3.19 | -4.6928 | -2.574 |
| 5.00 | -2.778367 | -2.542 | -1.765964205 | -2.73 | -4.1035 | -2.145 |
| 6.00 | -2.371933 | -2.1406 | -1.397130676 | -2.32 | -3.3622 | -1.772 |
| 7.00 | -2.008988 | -1.8024 | -1.077623294 | -1.98 | -2.9662 | -1.441 |
| 8.00 | -1.672961 | -1.5047 | -0.806813416 | -1.68 | -2.5249 | -1.14 |
| 9.00 | -1.362198 | -1.2061 | -0.554247818 | -1.37 | -2.1144 | -0.862 |
| 10.00 | -1.068674 | -0.9226 | -0.298551652 | -1.08 | -1.7157 | -0.606 |
| 11.00 | -0.796962 | -0.6448 | -0.060722323 | -0.79 | -1.2659 | -0.366 |
| 12.00 | -0.541513 | -0.3965 | 0.143245829 | -0.53 | -0.9214 | -0.139 |
| 13.00 | -0.299013 | -0.1639 | 0.344851752 | -0.29 | -0.7186 | 0.075 |
| 14.00 | -0.062863 | 0.062 | 0.551626545 | -0.06 | -0.3549 | 0.28 |
| 15.00 | 0.168603 | 0.2864 | 0.752821084 | 0.17 | 0.0176 | 0.477 |
| 16.00 | 0.386072 | 0.4979 | 0.931529317 | 0.39 | 0.2368 | 0.665 |
| 17.00 | 0.593768 | 0.7019 | 1.112590852 | 0.60 | 0.5486 | 0.846 |
| 18.00 | 0.794501 | 0.9018 | 1.276844742 | 0.81 | 0.7766 | 1.02 |
| 19.00 | 0.993508 | 1.0996 | 1.466397448 | 1.01 | 1.0355 | 1.19 |
| 20.00 | 1.188856 | 1.2867 | 1.639025721 | 1.20 | 1.2523 | 1.354 |
| 21.00 | 1.377686 | 1.466 | 1.811400431 | 1.38 | 1.4127 | 1.515 |
| 22.00 | 1.558283 | 1.6446 | 1.983846817 | 1.56 | 1.6947 | 1.671 |
| 23.00 | 1.734897 | 1.8275 | 2.145054822 | 1.75 | 1.8532 | 1.824 |
| 24.00 | 1.911289 | 2.0029 | 2.304054346 | 1.93 | 2.003 | 1.973 |
| 25.00 | 2.08257 | 2.1701 | 2.459386245 | 2.10 | 2.211 | 2.118 |
| 26.00 | 2.249617 | 2.3228 | 2.614379505 | 2.25 | 2.3603 | 2.262 |
| 27.00 | 2.414853 | 2.4866 | 2.755811586 | 2.42 | 2.4965 | 2.402 |
| 28.00 | 2.575864 | 2.6421 | 2.892473181 | 2.58 | 2.7318 | 2.54 |
| 29.00 | 2.731665 | 2.7983 | 3.035545416 | 2.74 | 2.9497 | 2.676 |
| 30.00 | 2.888604 | 2.9478 | 3.179672933 | 2.89 | 3.1254 | 2.808 |
| 31.00 | 3.040907 | 3.1026 | 3.317342612 | 3.05 | 3.3307 | 2.939 |
| 32.00 | 3.194734 | 3.2494 | 3.449500273 | 3.20 | 3.4318 | 3.067 |
| 33.00 | 3.34268 | 3.3973 | 3.5902457 | 3.35 | 3.549 | 3.195 |
| 34.00 | 3.486951 | 3.5506 | 3.724553172 | 3.51 | 3.6591 | 3.321 |
| 35.00 | 3.632011 | 3.6913 | 3.865270309 | 3.65 | 3.8193 | 3.446 |
| 36.00 | 3.774955 | 3.8283 | 3.98901791 | 3.79 | 3.9376 | 3.568 |
| 37.00 | 3.913502 | 3.9659 | 4.117980507 | 3.93 | 4.1132 | 3.691 |
| 38.00 | 4.053657 | 4.0999 | 4.262661203 | 4.06 | 4.2855 | 3.811 |
| 39.00 | 4.189767 | 4.2292 | 4.386350557 | 4.19 | 4.5049 | 3.93 |
| 40.00 | 4.323283 | 4.3675 | 4.520784775 | 4.33 | 4.6901 | 4.049 |
| 41.00 | 4.456513 | 4.4993 | 4.658423196 | 4.46 | 4.8362 | 4.167 |
| 42.00 | 4.589614 | 4.6367 | 4.788599708 | 4.60 | 4.9921 | 4.284 |
| 43.00 | 4.719452 | 4.7591 | 4.915572096 | 4.72 | 5.146 | 4.399 |
| 44.00 | 4.851453 | 4.8968 | 5.048158843 | 4.86 | 5.2866 | 4.514 |
| 45.00 | 4.980374 | 5.0247 | 5.168144546 | 4.99 | 5.4547 | 4.629 |
| 46.00 | 5.109397 | 5.1544 | 5.297230249 | 5.12 | 5.5963 | 4.743 |
| 47.00 | 5.238401 | 5.2757 | 5.419280746 | 5.24 | 5.7702 | 4.857 |
| 48.00 | 5.367129 | 5.4037 | 5.540153001 | 5.37 | 5.8515 | 4.97 |
| 49.00 | 5.49401 | 5.5256 | 5.666222442 | 5.49 | 6.0059 | 5.083 |
| 50.00 | 5.621948 | 5.6477 | 5.792391826 | 5.61 | 6.1351 | 5.195 |
| 51.00 | 5.749523 | 5.7679 | 5.909759908 | 5.73 | 6.248 | 5.307 |
| 52.00 | 5.873645 | 5.8964 | 6.035236731 | 5.86 | 6.4412 | 5.419 |
| 53.00 | 5.997783 | 6.0168 | 6.15517245 | 5.98 | 6.5372 | 5.532 |
| 54.00 | 6.123333 | 6.1377 | 6.276829722 | 6.10 | 6.6765 | 5.644 |
| 55.00 | 6.248701 | 6.2651 | 6.396302847 | 6.23 | 6.7813 | 5.757 |
| 56.00 | 6.370418 | 6.3858 | 6.51860499 | 6.35 | 6.9572 | 5.87 |
| 57.00 | 6.492327 | 6.5047 | 6.632443944 | 6.47 | 7.1027 | 5.983 |
| 58.00 | 6.613628 | 6.6325 | 6.7556433 | 6.60 | 7.2851 | 6.096 |
| 59.00 | 6.736956 | 6.7527 | 6.874927011 | 6.72 | 7.4389 | 6.208 |
| 60.00 | 6.858927 | 6.8721 | 6.990844975 | 6.84 | 7.6145 | 6.321 |
| 61.00 | 6.982047 | 6.9909 | 7.103660891 | 6.96 | 7.7341 | 6.434 |
| 62.00 | 7.106308 | 7.1126 | 7.229644919 | 7.08 | 7.8754 | 6.548 |
| 63.00 | 7.228863 | 7.2391 | 7.345839626 | 7.21 | 7.9968 | 6.664 |
| 64.00 | 7.353887 | 7.3598 | 7.46722921 | 7.33 | 8.1275 | 6.78 |
| 65.00 | 7.478927 | 7.4801 | 7.586047506 | 7.45 | 8.2258 | 6.897 |
| 66.00 | 7.602516 | 7.6011 | 7.70920519 | 7.57 | 8.39 | 7.015 |
| 67.00 | 7.729129 | 7.7312 | 7.841305877 | 7.70 | 8.5129 | 7.134 |
| 68.00 | 7.856766 | 7.8516 | 7.959848696 | 7.82 | 8.6499 | 7.255 |
| 69.00 | 7.987248 | 7.9796 | 8.079434977 | 7.95 | 8.7819 | 7.375 |
| 70.00 | 8.119367 | 8.1021 | 8.205689782 | 8.07 | 8.8831 | 7.497 |
| 71.00 | 8.249523 | 8.231 | 8.330244686 | 8.20 | 9.0672 | 7.621 |
| 72.00 | 8.380768 | 8.3689 | 8.46425476 | 8.34 | 9.2358 | 7.748 |
| 73.00 | 8.515463 | 8.4975 | 8.58471955 | 8.47 | 9.3488 | 7.876 |
| 74.00 | 8.653723 | 8.6364 | 8.72014783 | 8.61 | 9.5184 | 8.007 |
| 75.00 | 8.789896 | 8.7748 | 8.854262901 | 8.75 | 9.673 | 8.14 |
| 76.00 | 8.931183 | 8.9132 | 8.985169716 | 8.89 | 9.7995 | 8.275 |
| 77.00 | 9.0736 | 9.0522 | 9.11927584 | 9.03 | 9.932 | 8.415 |
| 78.00 | 9.219728 | 9.2017 | 9.268391209 | 9.18 | 10.1289 | 8.556 |
| 79.00 | 9.372013 | 9.3493 | 9.405289109 | 9.33 | 10.2628 | 8.702 |
| 80.00 | 9.52989 | 9.5072 | 9.556010857 | 9.49 | 10.614 | 8.852 |
| 81.00 | 9.690913 | 9.6734 | 9.711507799 | 9.66 | 10.8619 | 9.007 |
| 82.00 | 9.855916 | 9.8342 | 9.872800931 | 9.82 | 11.0663 | 9.167 |
| 83.00 | 10.028894 | 9.9972 | 10.04133984 | 9.98 | 11.2292 | 9.33 |
| 84.00 | 10.204849 | 10.1657 | 10.2011486 | 10.15 | 11.432 | 9.502 |
| 85.00 | 10.390458 | 10.3502 | 10.36580667 | 10.34 | 11.6646 | 9.682 |
| 86.00 | 10.584242 | 10.5414 | 10.5596779 | 10.53 | 11.8376 | 9.871 |
| 87.00 | 10.787174 | 10.7429 | 10.76599424 | 10.73 | 12.0696 | 10.068 |
| 88.00 | 11.004609 | 10.9517 | 10.96630296 | 10.94 | 12.5387 | 10.274 |
| 89.00 | 11.233517 | 11.1812 | 11.19402427 | 11.17 | 12.8094 | 10.494 |
| 90.00 | 11.472607 | 11.4201 | 11.42926987 | 11.41 | 13.2626 | 10.73 |
| 91.00 | 11.728989 | 11.6739 | 11.66000538 | 11.67 | 13.5295 | 10.983 |
| 92.00 | 12.014296 | 11.9429 | 11.91734331 | 11.94 | 13.8562 | 11.258 |
| 93.00 | 12.317672 | 12.2445 | 12.18843085 | 12.25 | 14.1429 | 11.559 |
| 94.00 | 12.654963 | 12.5914 | 12.49454199 | 12.61 | 14.4829 | 11.895 |
| 95.00 | 13.037834 | 12.9602 | 12.81709771 | 12.99 | 14.8902 | 12.274 |
| 96.00 | 13.480988 | 13.3892 | 13.20028727 | 13.43 | 15.4877 | 12.709 |
| 97.00 | 14.008933 | 13.8927 | 13.63394675 | 13.95 | 16.0533 | 13.233 |
| 98.00 | 14.676667 | 14.5209 | 14.20130382 | 14.59 | 17.1152 | 13.905 |
| 99.00 | 15.632654 | 15.443 | 15.07875133 | 15.52 | 18.3226 | 14.904 |
| 100.00 | 21.477752 | 20.2618 | 19.38030789 | 20.34 | 24.5444 | 21.426 |

6.3.1.2	Unconditional DIP distribution

Unconditional DIP1 distribution:

Company inputs are captured in Figure 6.3.1.2-1 based on raw data in Table 6.3.1.2-1.

Figure 6.3.1.2-1: Unconditional DIP1 calibration for homogeneous network

Table 6.3.1.2-1: Company inputs on unconditional DIP1 calibration for homogeneous network

| Percentile | China Telecom | Huawei | Ericsson | ZTE | Nokia Networks | Alcatel-Lucent |
| --- | --- | --- | --- | --- | --- | --- |
|  | R4-152823 | R4-152596 | R4-152958 | R4-153161 | R4-152236 | R4-153481 |
| 0.00 | -11.607758 | -20.6352 | -11.7426967 | -11.84 | -9.91 | -10.14 |
| 1.00 | -8.32198 | -8.0873 | -8.531353067 | -8.28 | -8.68 | -7.30 |
| 2.00 | -7.792122 | -7.2666 | -7.977248613 | -7.79 | -8.10 | -6.87 |
| 3.00 | -7.433822 | -6.8905 | -7.62035199 | -7.42 | -7.73 | -6.56 |
| 4.00 | -7.151077 | -6.6354 | -7.334383407 | -7.14 | -7.41 | -6.33 |
| 5.00 | -6.919383 | -6.4144 | -7.089211275 | -6.91 | -7.18 | -6.13 |
| 6.00 | -6.723163 | -6.2463 | -6.886238868 | -6.71 | -7.03 | -5.96 |
| 7.00 | -6.544646 | -6.0911 | -6.702625767 | -6.52 | -6.86 | -5.81 |
| 8.00 | -6.384459 | -5.9602 | -6.527534707 | -6.36 | -6.71 | -5.67 |
| 9.00 | -6.241269 | -5.8383 | -6.375950782 | -6.22 | -6.57 | -5.54 |
| 10.00 | -6.109353 | -5.73 | -6.236340718 | -6.08 | -6.43 | -5.43 |
| 11.00 | -5.985336 | -5.626 | -6.104303213 | -5.95 | -6.32 | -5.32 |
| 12.00 | -5.868997 | -5.5246 | -5.981569688 | -5.84 | -6.14 | -5.21 |
| 13.00 | -5.758806 | -5.4293 | -5.871057488 | -5.73 | -6.02 | -5.11 |
| 14.00 | -5.652633 | -5.3401 | -5.759610501 | -5.62 | -5.92 | -5.02 |
| 15.00 | -5.551719 | -5.2527 | -5.654553505 | -5.52 | -5.81 | -4.93 |
| 16.00 | -5.456465 | -5.1716 | -5.548050966 | -5.42 | -5.71 | -4.85 |
| 17.00 | -5.364572 | -5.0949 | -5.457362452 | -5.33 | -5.62 | -4.76 |
| 18.00 | -5.276664 | -5.0172 | -5.36606239 | -5.24 | -5.54 | -4.69 |
| 19.00 | -5.189142 | -4.9489 | -5.274348027 | -5.15 | -5.47 | -4.61 |
| 20.00 | -5.104681 | -4.8769 | -5.183417684 | -5.07 | -5.40 | -4.53 |
| 21.00 | -5.023483 | -4.8098 | -5.09204517 | -4.99 | -5.35 | -4.46 |
| 22.00 | -4.945204 | -4.7408 | -5.010185108 | -4.91 | -5.27 | -4.39 |
| 23.00 | -4.869322 | -4.6737 | -4.931797836 | -4.83 | -5.15 | -4.32 |
| 24.00 | -4.794657 | -4.6095 | -4.85329821 | -4.76 | -5.08 | -4.26 |
| 25.00 | -4.721955 | -4.5447 | -4.771603675 | -4.68 | -5.02 | -4.19 |
| 26.00 | -4.650108 | -4.4804 | -4.698526944 | -4.61 | -4.96 | -4.13 |
| 27.00 | -4.578765 | -4.4183 | -4.618959322 | -4.55 | -4.89 | -4.06 |
| 28.00 | -4.50909 | -4.3586 | -4.545197736 | -4.48 | -4.81 | -4.00 |
| 29.00 | -4.440237 | -4.2975 | -4.470782027 | -4.41 | -4.73 | -3.94 |
| 30.00 | -4.373159 | -4.2413 | -4.3987953 | -4.35 | -4.63 | -3.88 |
| 31.00 | -4.307332 | -4.1814 | -4.334229022 | -4.28 | -4.55 | -3.82 |
| 32.00 | -4.242738 | -4.1212 | -4.263037258 | -4.22 | -4.49 | -3.76 |
| 33.00 | -4.178946 | -4.063 | -4.196172297 | -4.16 | -4.44 | -3.71 |
| 34.00 | -4.115789 | -4.009 | -4.131171937 | -4.09 | -4.37 | -3.65 |
| 35.00 | -4.052369 | -3.9541 | -4.064935407 | -4.03 | -4.27 | -3.60 |
| 36.00 | -3.990135 | -3.8987 | -4.004226168 | -3.97 | -4.21 | -3.54 |
| 37.00 | -3.927733 | -3.8465 | -3.940138275 | -3.90 | -4.15 | -3.49 |
| 38.00 | -3.866867 | -3.7951 | -3.879475515 | -3.84 | -4.06 | -3.43 |
| 39.00 | -3.805513 | -3.7404 | -3.81489538 | -3.78 | -3.99 | -3.38 |
| 40.00 | -3.744533 | -3.689 | -3.752497712 | -3.72 | -3.94 | -3.33 |
| 41.00 | -3.685316 | -3.6356 | -3.69032111 | -3.66 | -3.86 | -3.27 |
| 42.00 | -3.626901 | -3.5829 | -3.629506504 | -3.60 | -3.80 | -3.22 |
| 43.00 | -3.568287 | -3.5308 | -3.568502344 | -3.54 | -3.74 | -3.17 |
| 44.00 | -3.509713 | -3.4793 | -3.511372044 | -3.49 | -3.65 | -3.12 |
| 45.00 | -3.453172 | -3.4266 | -3.450772983 | -3.43 | -3.59 | -3.07 |
| 46.00 | -3.395052 | -3.3751 | -3.395561916 | -3.37 | -3.51 | -3.01 |
| 47.00 | -3.337282 | -3.3244 | -3.33615283 | -3.31 | -3.47 | -2.96 |
| 48.00 | -3.280608 | -3.2741 | -3.282982301 | -3.26 | -3.41 | -2.91 |
| 49.00 | -3.223862 | -3.2208 | -3.230113192 | -3.20 | -3.34 | -2.86 |
| 50.00 | -3.166404 | -3.1692 | -3.176698367 | -3.14 | -3.27 | -2.81 |
| 51.00 | -3.10857 | -3.1163 | -3.12230297 | -3.09 | -3.19 | -2.75 |
| 52.00 | -3.052225 | -3.0648 | -3.068425249 | -3.03 | -3.14 | -2.70 |
| 53.00 | -2.99616 | -3.011 | -3.011242453 | -2.97 | -3.08 | -2.65 |
| 54.00 | -2.938637 | -2.9592 | -2.956639795 | -2.91 | -3.03 | -2.60 |
| 55.00 | -2.881454 | -2.906 | -2.896562904 | -2.86 | -2.96 | -2.55 |
| 56.00 | -2.822551 | -2.8551 | -2.838346998 | -2.80 | -2.91 | -2.49 |
| 57.00 | -2.764862 | -2.8023 | -2.779582371 | -2.75 | -2.86 | -2.44 |
| 58.00 | -2.707033 | -2.7485 | -2.726653699 | -2.69 | -2.80 | -2.39 |
| 59.00 | -2.649841 | -2.6944 | -2.673255085 | -2.63 | -2.73 | -2.34 |
| 60.00 | -2.592483 | -2.6409 | -2.615730239 | -2.57 | -2.64 | -2.28 |
| 61.00 | -2.534468 | -2.5871 | -2.557066651 | -2.52 | -2.57 | -2.23 |
| 62.00 | -2.47643 | -2.5305 | -2.50090151 | -2.46 | -2.50 | -2.18 |
| 63.00 | -2.417989 | -2.4754 | -2.443354881 | -2.40 | -2.44 | -2.12 |
| 64.00 | -2.359218 | -2.4211 | -2.387382659 | -2.34 | -2.39 | -2.07 |
| 65.00 | -2.300181 | -2.3646 | -2.323782464 | -2.29 | -2.32 | -2.02 |
| 66.00 | -2.240501 | -2.3082 | -2.269484822 | -2.23 | -2.25 | -1.97 |
| 67.00 | -2.180649 | -2.2515 | -2.216504769 | -2.17 | -2.17 | -1.91 |
| 68.00 | -2.120916 | -2.1951 | -2.164454885 | -2.11 | -2.11 | -1.86 |
| 69.00 | -2.061183 | -2.1365 | -2.109868589 | -2.05 | -2.06 | -1.81 |
| 70.00 | -1.999736 | -2.0826 | -2.05499015 | -2.00 | -2.01 | -1.75 |
| 71.00 | -1.939221 | -2.025 | -1.997478979 | -1.94 | -1.95 | -1.70 |
| 72.00 | -1.880227 | -1.9652 | -1.942664564 | -1.88 | -1.87 | -1.65 |
| 73.00 | -1.820653 | -1.9066 | -1.884635261 | -1.82 | -1.79 | -1.59 |
| 74.00 | -1.761093 | -1.8532 | -1.828639498 | -1.76 | -1.72 | -1.54 |
| 75.00 | -1.701938 | -1.7959 | -1.774130943 | -1.70 | -1.65 | -1.49 |
| 76.00 | -1.641262 | -1.7378 | -1.715482307 | -1.64 | -1.59 | -1.44 |
| 77.00 | -1.582232 | -1.6774 | -1.655757839 | -1.58 | -1.53 | -1.38 |
| 78.00 | -1.523325 | -1.618 | -1.599904398 | -1.52 | -1.44 | -1.33 |
| 79.00 | -1.464667 | -1.5582 | -1.546063052 | -1.47 | -1.38 | -1.28 |
| 80.00 | -1.404798 | -1.4991 | -1.490707118 | -1.41 | -1.35 | -1.22 |
| 81.00 | -1.345973 | -1.4384 | -1.433726773 | -1.35 | -1.29 | -1.17 |
| 82.00 | -1.285974 | -1.3781 | -1.379109303 | -1.29 | -1.21 | -1.12 |
| 83.00 | -1.228196 | -1.3181 | -1.324931616 | -1.23 | -1.16 | -1.07 |
| 84.00 | -1.168437 | -1.2585 | -1.271965616 | -1.18 | -1.12 | -1.01 |
| 85.00 | -1.107523 | -1.199 | -1.216622332 | -1.12 | -1.06 | -0.96 |
| 86.00 | -1.048222 | -1.138 | -1.159526179 | -1.06 | -0.98 | -0.91 |
| 87.00 | -0.988535 | -1.0745 | -1.104198299 | -1.00 | -0.92 | -0.86 |
| 88.00 | -0.929564 | -1.0118 | -1.046782411 | -0.94 | -0.85 | -0.80 |
| 89.00 | -0.870438 | -0.9461 | -0.990467754 | -0.88 | -0.76 | -0.75 |
| 90.00 | -0.811129 | -0.8829 | -0.928555933 | -0.82 | -0.68 | -0.70 |
| 91.00 | -0.751944 | -0.8171 | -0.872491745 | -0.76 | -0.60 | -0.65 |
| 92.00 | -0.693088 | -0.7533 | -0.815033285 | -0.70 | -0.52 | -0.59 |
| 93.00 | -0.633303 | -0.6814 | -0.755612898 | -0.64 | -0.48 | -0.54 |
| 94.00 | -0.573057 | -0.6132 | -0.692116681 | -0.58 | -0.40 | -0.49 |
| 95.00 | -0.511097 | -0.5415 | -0.632256832 | -0.52 | -0.34 | -0.43 |
| 96.00 | -0.446275 | -0.4655 | -0.567876386 | -0.45 | -0.29 | -0.37 |
| 97.00 | -0.377648 | -0.3894 | -0.490301369 | -0.39 | -0.26 | -0.32 |
| 98.00 | -0.305635 | -0.3097 | -0.403015026 | -0.31 | -0.19 | -0.25 |
| 99.00 | -0.220144 | -0.2162 | -0.297227592 | -0.23 | -0.12 | -0.18 |
| 100.00 | -0.0256 | -0.0244 | -0.098458692 | -0.04 | -0.01 | -0.01 |

Unconditional DIP2 distribution:

Company inputs are captured in Figure 6.3.1.2-2 based on raw data in Table 6.3.1.2-2.

Figure 6.3.1.2-2: Unconditional DIP2 calibration for homogeneous network

Table 6.3.1.2-2: Company inputs on unconditional DIP2 calibration for homogeneous network

| Percentile | China Telecom | Huawei | Ericsson | ZTE | Nokia Networks | Alcatel-Lucent |
| --- | --- | --- | --- | --- | --- | --- |
|  | R4-152823 | R4-152596 | R4-152958 | R4-153161 | R4-152236 | R4-153481 |
| 0.00 | -30.95211 | -29.6831 | -26.37816118 | -29.65 | -28.7559 | -31.551 |
| 1.00 | -19.370581 | -19.7454 | -18.39359609 | -19.25 | -21.6987 | -19.239 |
| 2.00 | -17.650628 | -17.7208 | -16.87033539 | -17.54 | -19.9294 | -17.527 |
| 3.00 | -16.52869 | -16.425 | -15.91149155 | -16.49 | -18.2966 | -16.426 |
| 4.00 | -15.707433 | -15.5001 | -15.20369529 | -15.64 | -17.1797 | -15.6 |
| 5.00 | -15.035429 | -14.8074 | -14.61067492 | -14.97 | -16.4299 | -14.933 |
| 6.00 | -14.485384 | -14.2201 | -14.13848831 | -14.43 | -15.97 | -14.373 |
| 7.00 | -13.998394 | -13.6678 | -13.67432579 | -13.98 | -15.1966 | -13.89 |
| 8.00 | -13.584652 | -13.2182 | -13.30862909 | -13.55 | -14.5581 | -13.463 |
| 9.00 | -13.219005 | -12.8256 | -12.97185103 | -13.21 | -14.0808 | -13.082 |
| 10.00 | -12.873325 | -12.4709 | -12.67752903 | -12.86 | -13.5857 | -12.738 |
| 11.00 | -12.572449 | -12.1501 | -12.40287848 | -12.57 | -13.1235 | -12.424 |
| 12.00 | -12.29328 | -11.859 | -12.16001857 | -12.30 | -12.9558 | -12.137 |
| 13.00 | -12.042537 | -11.5855 | -11.91285241 | -12.03 | -12.7374 | -11.869 |
| 14.00 | -11.813952 | -11.331 | -11.69210463 | -11.78 | -12.4106 | -11.623 |
| 15.00 | -11.598281 | -11.1117 | -11.49940281 | -11.57 | -12.088 | -11.394 |
| 16.00 | -11.40327 | -10.9085 | -11.32958397 | -11.37 | -11.8953 | -11.18 |
| 17.00 | -11.225138 | -10.7269 | -11.16250636 | -11.19 | -11.7025 | -10.978 |
| 18.00 | -11.054184 | -10.5548 | -11.00305099 | -11.02 | -11.5511 | -10.789 |
| 19.00 | -10.891196 | -10.379 | -10.86461781 | -10.85 | -11.3474 | -10.61 |
| 20.00 | -10.73815 | -10.2142 | -10.71699024 | -10.70 | -11.1607 | -10.442 |
| 21.00 | -10.591478 | -10.0665 | -10.58533541 | -10.56 | -10.9921 | -10.282 |
| 22.00 | -10.45609 | -9.9221 | -10.45942093 | -10.42 | -10.8484 | -10.131 |
| 23.00 | -10.329075 | -9.7831 | -10.33893023 | -10.30 | -10.6789 | -9.988 |
| 24.00 | -10.208017 | -9.6613 | -10.22032344 | -10.18 | -10.5333 | -9.851 |
| 25.00 | -10.092986 | -9.5431 | -10.11645588 | -10.07 | -10.4309 | -9.721 |
| 26.00 | -9.984326 | -9.4221 | -10.01661448 | -9.96 | -10.316 | -9.597 |
| 27.00 | -9.881658 | -9.3064 | -9.911300812 | -9.86 | -10.2051 | -9.479 |
| 28.00 | -9.781226 | -9.2016 | -9.812970386 | -9.76 | -10.0688 | -9.366 |
| 29.00 | -9.684551 | -9.1055 | -9.723075982 | -9.66 | -9.998 | -9.258 |
| 30.00 | -9.590871 | -9.008 | -9.633982305 | -9.57 | -9.9099 | -9.155 |
| 31.00 | -9.500057 | -8.9146 | -9.548377178 | -9.49 | -9.8414 | -9.056 |
| 32.00 | -9.411958 | -8.8287 | -9.465468469 | -9.40 | -9.7025 | -8.961 |
| 33.00 | -9.328085 | -8.7385 | -9.383934763 | -9.32 | -9.6058 | -8.869 |
| 34.00 | -9.245238 | -8.655 | -9.300343879 | -9.24 | -9.5151 | -8.782 |
| 35.00 | -9.165154 | -8.5731 | -9.222922095 | -9.16 | -9.4282 | -8.697 |
| 36.00 | -9.085415 | -8.4966 | -9.141649679 | -9.08 | -9.3248 | -8.615 |
| 37.00 | -9.007596 | -8.4236 | -9.060383993 | -9.01 | -9.2598 | -8.535 |
| 38.00 | -8.930607 | -8.3506 | -8.984444561 | -8.93 | -9.1974 | -8.458 |
| 39.00 | -8.857281 | -8.2822 | -8.911549372 | -8.86 | -9.1516 | -8.382 |
| 40.00 | -8.784769 | -8.2154 | -8.847083979 | -8.78 | -9.0836 | -8.309 |
| 41.00 | -8.712837 | -8.1466 | -8.776772376 | -8.71 | -9.0042 | -8.237 |
| 42.00 | -8.641511 | -8.0842 | -8.707921787 | -8.64 | -8.9449 | -8.167 |
| 43.00 | -8.573275 | -8.0182 | -8.641894657 | -8.57 | -8.8709 | -8.099 |
| 44.00 | -8.505493 | -7.9553 | -8.57685659 | -8.50 | -8.8152 | -8.031 |
| 45.00 | -8.43863 | -7.8944 | -8.509641622 | -8.44 | -8.7428 | -7.966 |
| 46.00 | -8.371716 | -7.8342 | -8.44160837 | -8.37 | -8.6793 | -7.901 |
| 47.00 | -8.306195 | -7.7774 | -8.371959977 | -8.31 | -8.5752 | -7.837 |
| 48.00 | -8.24091 | -7.7204 | -8.305611933 | -8.25 | -8.5261 | -7.775 |
| 49.00 | -8.176896 | -7.663 | -8.239291476 | -8.18 | -8.4493 | -7.712 |
| 50.00 | -8.114103 | -7.6061 | -8.176593303 | -8.12 | -8.3931 | -7.651 |
| 51.00 | -8.052583 | -7.5507 | -8.116420571 | -8.05 | -8.3083 | -7.59 |
| 52.00 | -7.9907 | -7.495 | -8.047201939 | -7.99 | -8.2511 | -7.53 |
| 53.00 | -7.926844 | -7.4409 | -7.988048556 | -7.92 | -8.201 | -7.471 |
| 54.00 | -7.864212 | -7.3905 | -7.927954487 | -7.87 | -8.1321 | -7.412 |
| 55.00 | -7.803707 | -7.3383 | -7.864454868 | -7.81 | -8.0756 | -7.354 |
| 56.00 | -7.742534 | -7.2863 | -7.801074355 | -7.74 | -8.0014 | -7.295 |
| 57.00 | -7.681956 | -7.2335 | -7.742161465 | -7.69 | -7.9446 | -7.238 |
| 58.00 | -7.622122 | -7.18 | -7.684424905 | -7.63 | -7.8996 | -7.18 |
| 59.00 | -7.563369 | -7.1258 | -7.621322378 | -7.56 | -7.8128 | -7.123 |
| 60.00 | -7.503029 | -7.0761 | -7.561323441 | -7.50 | -7.7603 | -7.066 |
| 61.00 | -7.442558 | -7.026 | -7.497353584 | -7.44 | -7.6985 | -7.009 |
| 62.00 | -7.381071 | -6.9739 | -7.435759743 | -7.38 | -7.6516 | -6.951 |
| 63.00 | -7.321543 | -6.9212 | -7.376487451 | -7.32 | -7.5744 | -6.894 |
| 64.00 | -7.261459 | -6.8697 | -7.314596482 | -7.26 | -7.526 | -6.838 |
| 65.00 | -7.200622 | -6.8167 | -7.253906468 | -7.20 | -7.4488 | -6.781 |
| 66.00 | -7.139075 | -6.763 | -7.189654534 | -7.14 | -7.3863 | -6.724 |
| 67.00 | -7.077904 | -6.71 | -7.125049819 | -7.08 | -7.3406 | -6.667 |
| 68.00 | -7.017733 | -6.6589 | -7.057109387 | -7.02 | -7.2733 | -6.61 |
| 69.00 | -6.957461 | -6.6069 | -6.994601875 | -6.96 | -7.2263 | -6.552 |
| 70.00 | -6.895641 | -6.5544 | -6.933748938 | -6.90 | -7.1906 | -6.495 |
| 71.00 | -6.833735 | -6.5003 | -6.870060668 | -6.84 | -7.1406 | -6.436 |
| 72.00 | -6.771301 | -6.4459 | -6.803271171 | -6.77 | -7.0848 | -6.378 |
| 73.00 | -6.709284 | -6.393 | -6.734753955 | -6.71 | -7.0216 | -6.319 |
| 74.00 | -6.646511 | -6.3367 | -6.66721743 | -6.64 | -6.9672 | -6.26 |
| 75.00 | -6.582153 | -6.283 | -6.603784415 | -6.58 | -6.887 | -6.2 |
| 76.00 | -6.517093 | -6.2268 | -6.536718426 | -6.51 | -6.827 | -6.139 |
| 77.00 | -6.451233 | -6.169 | -6.468283818 | -6.45 | -6.7866 | -6.077 |
| 78.00 | -6.383491 | -6.1095 | -6.39920204 | -6.38 | -6.7261 | -6.015 |
| 79.00 | -6.31642 | -6.0508 | -6.328714605 | -6.31 | -6.6573 | -5.951 |
| 80.00 | -6.248832 | -5.988 | -6.255760146 | -6.24 | -6.5785 | -5.887 |
| 81.00 | -6.17778 | -5.9268 | -6.179577318 | -6.17 | -6.4776 | -5.821 |
| 82.00 | -6.105317 | -5.8648 | -6.097717555 | -6.10 | -6.3877 | -5.753 |
| 83.00 | -6.031971 | -5.8047 | -6.022893928 | -6.02 | -6.2858 | -5.685 |
| 84.00 | -5.955517 | -5.736 | -5.945735726 | -5.95 | -6.2115 | -5.615 |
| 85.00 | -5.876971 | -5.6703 | -5.857275334 | -5.87 | -6.1528 | -5.543 |
| 86.00 | -5.794784 | -5.5963 | -5.772258564 | -5.79 | -6.0422 | -5.468 |
| 87.00 | -5.711563 | -5.523 | -5.687812659 | -5.70 | -5.9698 | -5.391 |
| 88.00 | -5.623125 | -5.447 | -5.599073251 | -5.62 | -5.8926 | -5.311 |
| 89.00 | -5.533195 | -5.3622 | -5.505310189 | -5.52 | -5.7963 | -5.228 |
| 90.00 | -5.436815 | -5.2781 | -5.405125259 | -5.43 | -5.6512 | -5.141 |
| 91.00 | -5.338206 | -5.1904 | -5.307497667 | -5.33 | -5.5384 | -5.048 |
| 92.00 | -5.23236 | -5.0967 | -5.193590403 | -5.22 | -5.4398 | -4.95 |
| 93.00 | -5.114995 | -4.9944 | -5.079266801 | -5.11 | -5.3233 | -4.845 |
| 94.00 | -4.987157 | -4.8845 | -4.946425167 | -4.99 | -5.1855 | -4.731 |
| 95.00 | -4.847697 | -4.7626 | -4.819820984 | -4.85 | -5.0852 | -4.605 |
| 96.00 | -4.689661 | -4.6289 | -4.667534074 | -4.68 | -4.976 | -4.465 |
| 97.00 | -4.504738 | -4.4619 | -4.484895559 | -4.50 | -4.7212 | -4.303 |
| 98.00 | -4.287422 | -4.2689 | -4.257490708 | -4.28 | -4.473 | -4.106 |
| 99.00 | -3.985876 | -3.9819 | -3.961577279 | -3.99 | -4.0399 | -3.842 |
| 100.00 | -3.072741 | -3.155 | -3.198123349 | -3.15 | -3.3686 | -3.046 |

### 6.3.2	Heterogeneous network

6.3.2.1	UL wideband SINR distribution

Company inputs are captured in Figure 6.3.2.1-1 based on raw data in Table 6.3.2.1-1.

Figure 6.3.2.1-1: UL wideband SINR calibration for heterogeneous network

Table 6.3.2.1-1: Company inputs on UL wideband SINR calibration for heterogeneous network

| Percentile | China Telecom | Huawei | Ericsson | ZTE | Nokia Networks |
| --- | --- | --- | --- | --- | --- |
|  | R4-152823 | R4-152596 | R4-152958 | R4-153162 | R4-152236 |
| 0.00 | -18.956592 | -25.9732 | -24.78146413 | -20.89 | -27.3065 |
| 1.00 | -8.735173 | -10.8639 | -8.354464219 | -8.64 | -9.5327 |
| 2.00 | -6.970898 | -7.1398 | -6.658699074 | -6.94 | -7.7521 |
| 3.00 | -5.841924 | -5.942 | -5.358006957 | -5.81 | -6.6156 |
| 4.00 | -4.989746 | -5.108 | -4.550215874 | -4.96 | -5.6594 |
| 5.00 | -4.300944 | -4.3993 | -3.817360959 | -4.26 | -4.8589 |
| 6.00 | -3.712617 | -3.7903 | -3.183209455 | -3.66 | -4.2495 |
| 7.00 | -3.16765 | -3.2241 | -2.577502347 | -3.13 | -3.699 |
| 8.00 | -2.716178 | -2.7517 | -2.088000324 | -2.65 | -3.1373 |
| 9.00 | -2.299296 | -2.3122 | -1.621311257 | -2.22 | -2.6938 |
| 10.00 | -1.910297 | -1.9024 | -1.185660545 | -1.82 | -2.2309 |
| 11.00 | -1.550903 | -1.5275 | -0.787939777 | -1.46 | -1.8538 |
| 12.00 | -1.21359 | -1.1871 | -0.43921874 | -1.13 | -1.4634 |
| 13.00 | -0.893508 | -0.854 | -0.091584335 | -0.80 | -1.1337 |
| 14.00 | -0.589744 | -0.5436 | 0.23064384 | -0.50 | -0.7836 |
| 15.00 | -0.293808 | -0.2533 | 0.510577466 | -0.20 | -0.4374 |
| 16.00 | -0.005691 | 0.0119 | 0.750394422 | 0.07 | -0.1478 |
| 17.00 | 0.260926 | 0.2694 | 0.992545804 | 0.34 | 0.1333 |
| 18.00 | 0.525046 | 0.5182 | 1.220377672 | 0.60 | 0.4088 |
| 19.00 | 0.783903 | 0.7679 | 1.460925354 | 0.85 | 0.6366 |
| 20.00 | 1.025806 | 0.9932 | 1.657134309 | 1.10 | 0.8905 |
| 21.00 | 1.268284 | 1.2279 | 1.886563427 | 1.33 | 1.1472 |
| 22.00 | 1.503002 | 1.4559 | 2.106698356 | 1.56 | 1.415 |
| 23.00 | 1.731518 | 1.6811 | 2.326147025 | 1.79 | 1.6583 |
| 24.00 | 1.952506 | 1.8968 | 2.534608855 | 2.01 | 1.9003 |
| 25.00 | 2.176908 | 2.1182 | 2.753610064 | 2.23 | 2.1692 |
| 26.00 | 2.384158 | 2.3174 | 2.944382213 | 2.43 | 2.3981 |
| 27.00 | 2.591449 | 2.5333 | 3.171722785 | 2.64 | 2.5921 |
| 28.00 | 2.797618 | 2.7398 | 3.381825513 | 2.84 | 2.7909 |
| 29.00 | 2.997589 | 2.9325 | 3.563692715 | 3.04 | 3.0005 |
| 30.00 | 3.195345 | 3.1248 | 3.746685686 | 3.24 | 3.1983 |
| 31.00 | 3.394276 | 3.3182 | 3.936242642 | 3.43 | 3.398 |
| 32.00 | 3.589059 | 3.5137 | 4.130239452 | 3.63 | 3.5957 |
| 33.00 | 3.781831 | 3.7001 | 4.313324235 | 3.81 | 3.7835 |
| 34.00 | 3.970445 | 3.8886 | 4.501034783 | 4.00 | 3.9745 |
| 35.00 | 4.158521 | 4.067 | 4.66889823 | 4.18 | 4.1667 |
| 36.00 | 4.342236 | 4.2456 | 4.841767456 | 4.36 | 4.3851 |
| 37.00 | 4.521532 | 4.4201 | 5.008635555 | 4.54 | 4.5629 |
| 38.00 | 4.699396 | 4.5997 | 5.189812946 | 4.72 | 4.7343 |
| 39.00 | 4.877215 | 4.7714 | 5.351198375 | 4.90 | 4.9268 |
| 40.00 | 5.0529 | 4.9423 | 5.512818395 | 5.08 | 5.1255 |
| 41.00 | 5.227737 | 5.1185 | 5.688493298 | 5.26 | 5.3008 |
| 42.00 | 5.40674 | 5.2964 | 5.864163425 | 5.44 | 5.4769 |
| 43.00 | 5.580797 | 5.4666 | 6.030650084 | 5.61 | 5.6426 |
| 44.00 | 5.757649 | 5.6439 | 6.207207598 | 5.79 | 5.8272 |
| 45.00 | 5.93034 | 5.8215 | 6.393381098 | 5.96 | 5.9922 |
| 46.00 | 6.09948 | 5.9876 | 6.554509581 | 6.13 | 6.1638 |
| 47.00 | 6.272621 | 6.1639 | 6.732194378 | 6.31 | 6.3363 |
| 48.00 | 6.448094 | 6.336 | 6.901785675 | 6.48 | 6.5152 |
| 49.00 | 6.624554 | 6.5175 | 7.08907497 | 6.66 | 6.667 |
| 50.00 | 6.797393 | 6.6968 | 7.2796601 | 6.83 | 6.8256 |
| 51.00 | 6.97267 | 6.8683 | 7.448156896 | 7.00 | 6.9956 |
| 52.00 | 7.151922 | 7.0517 | 7.642410145 | 7.17 | 7.1753 |
| 53.00 | 7.326125 | 7.2253 | 7.817152259 | 7.34 | 7.3389 |
| 54.00 | 7.500377 | 7.4004 | 7.990639353 | 7.52 | 7.5323 |
| 55.00 | 7.680656 | 7.5692 | 8.147311498 | 7.69 | 7.7441 |
| 56.00 | 7.857192 | 7.7408 | 8.309723384 | 7.87 | 7.9054 |
| 57.00 | 8.035887 | 7.9166 | 8.480708469 | 8.05 | 8.0669 |
| 58.00 | 8.21755 | 8.1016 | 8.671463086 | 8.23 | 8.2412 |
| 59.00 | 8.403202 | 8.2794 | 8.8401771 | 8.41 | 8.4253 |
| 60.00 | 8.592442 | 8.4677 | 9.031779868 | 8.59 | 8.6072 |
| 61.00 | 8.777975 | 8.6592 | 9.229977402 | 8.78 | 8.7947 |
| 62.00 | 8.967586 | 8.8418 | 9.406864485 | 8.96 | 8.9525 |
| 63.00 | 9.155627 | 9.0284 | 9.590309119 | 9.15 | 9.152 |
| 64.00 | 9.351489 | 9.2237 | 9.782740986 | 9.35 | 9.3222 |
| 65.00 | 9.549952 | 9.4165 | 9.971370787 | 9.54 | 9.5023 |
| 66.00 | 9.745222 | 9.6066 | 10.15128048 | 9.74 | 9.7158 |
| 67.00 | 9.946062 | 9.808 | 10.35390393 | 9.94 | 9.9074 |
| 68.00 | 10.148004 | 10.0026 | 10.53851296 | 10.14 | 10.0722 |
| 69.00 | 10.353969 | 10.2108 | 10.74306086 | 10.36 | 10.2535 |
| 70.00 | 10.564923 | 10.4208 | 10.9519911 | 10.57 | 10.4781 |
| 71.00 | 10.77968 | 10.6284 | 11.14629478 | 10.79 | 10.6558 |
| 72.00 | 10.999277 | 10.8445 | 11.35702327 | 11.01 | 10.8686 |
| 73.00 | 11.225293 | 11.0672 | 11.572727 | 11.24 | 11.0814 |
| 74.00 | 11.456225 | 11.295 | 11.8013622 | 11.46 | 11.2642 |
| 75.00 | 11.689228 | 11.5298 | 12.03518789 | 11.70 | 11.4653 |
| 76.00 | 11.931496 | 11.7729 | 12.28076923 | 11.94 | 11.6709 |
| 77.00 | 12.176596 | 12.0219 | 12.53303605 | 12.19 | 11.9253 |
| 78.00 | 12.424979 | 12.2811 | 12.80286537 | 12.45 | 12.1718 |
| 79.00 | 12.687296 | 12.5425 | 13.06387703 | 12.71 | 12.4182 |
| 80.00 | 12.956263 | 12.8029 | 13.30598517 | 12.99 | 12.6361 |
| 81.00 | 13.229052 | 13.0788 | 13.58801154 | 13.26 | 12.9231 |
| 82.00 | 13.511215 | 13.3643 | 13.87942132 | 13.54 | 13.2594 |
| 83.00 | 13.803263 | 13.6645 | 14.18788833 | 13.84 | 13.565 |
| 84.00 | 14.101349 | 13.9675 | 14.49735304 | 14.14 | 13.8295 |
| 85.00 | 14.406624 | 14.2829 | 14.82065121 | 14.46 | 14.1091 |
| 86.00 | 14.730684 | 14.6047 | 15.14608676 | 14.77 | 14.4306 |
| 87.00 | 15.060284 | 14.9337 | 15.47399204 | 15.10 | 14.8384 |
| 88.00 | 15.404918 | 15.2919 | 15.85476338 | 15.44 | 15.2844 |
| 89.00 | 15.772456 | 15.6859 | 16.28736129 | 15.81 | 15.691 |
| 90.00 | 16.155986 | 16.088 | 16.71895665 | 16.19 | 16.1064 |
| 91.00 | 16.552278 | 16.4995 | 17.15146706 | 16.59 | 16.5689 |
| 92.00 | 16.970555 | 16.9394 | 17.62789134 | 17.00 | 17.0518 |
| 93.00 | 17.416284 | 17.4013 | 18.11196842 | 17.45 | 17.5837 |
| 94.00 | 17.911422 | 17.9199 | 18.67830482 | 17.92 | 18.1315 |
| 95.00 | 18.443112 | 18.4974 | 19.3203589 | 18.46 | 18.7087 |
| 96.00 | 19.035085 | 19.1369 | 20.02706684 | 19.06 | 19.3635 |
| 97.00 | 19.747514 | 19.9278 | 20.94210229 | 19.76 | 20.1355 |
| 98.00 | 20.646341 | 20.9246 | 22.09504382 | 20.64 | 21.0762 |
| 99.00 | 21.937776 | 22.3062 | 23.60778422 | 21.94 | 22.1964 |
| 100.00 | 28.763983 | 29.1162 | 30.53653735 | 28.48 | 26.3848 |

6.3.2.2	Unconditional DIP distribution

Unconditional DIP1 distribution:

Company inputs are captured in Figure 6.3.2.2-1 based on raw data in Table 6.3.2.2-1.

Figure 6.3.2.2-1: Unconditional DIP1 calibration for heterogeneous network

Table 6.3.2.2-1: Company inputs on unconditional DIP1 calibration for heterogeneous network

| Percentile | China Telecom | Huawei | Ericsson | ZTE | Nokia Networks |
| --- | --- | --- | --- | --- | --- |
|  | R4-152823 | R4-152596 | R4-152958 | R4-153162 | R4-152236 |
| 0.00 | -16.377394 | -17.62 | -13.25336438 | -15.65 | -14.51 |
| 1.00 | -8.358684 | -9.0828 | -8.083709446 | -8.35 | -7.89 |
| 2.00 | -7.588872 | -8.2563 | -7.36975798 | -7.57 | -7.29 |
| 3.00 | -7.096188 | -7.7461 | -6.950106904 | -7.09 | -6.89 |
| 4.00 | -6.732448 | -7.3695 | -6.64820908 | -6.72 | -6.55 |
| 5.00 | -6.434056 | -7.0642 | -6.402939263 | -6.43 | -6.28 |
| 6.00 | -6.182155 | -6.7964 | -6.178302348 | -6.17 | -6.07 |
| 7.00 | -5.959251 | -6.5654 | -5.99434389 | -5.94 | -5.91 |
| 8.00 | -5.760237 | -6.3549 | -5.812038819 | -5.75 | -5.73 |
| 9.00 | -5.578386 | -6.1548 | -5.634128985 | -5.56 | -5.54 |
| 10.00 | -5.412381 | -5.9806 | -5.486013146 | -5.40 | -5.37 |
| 11.00 | -5.256299 | -5.8166 | -5.351009795 | -5.24 | -5.23 |
| 12.00 | -5.111437 | -5.6662 | -5.226323832 | -5.10 | -5.07 |
| 13.00 | -4.977332 | -5.5257 | -5.112595221 | -4.96 | -4.92 |
| 14.00 | -4.847491 | -5.3809 | -4.977977191 | -4.83 | -4.79 |
| 15.00 | -4.725124 | -5.2563 | -4.879271787 | -4.71 | -4.67 |
| 16.00 | -4.608491 | -5.136 | -4.779448505 | -4.60 | -4.57 |
| 17.00 | -4.496531 | -5.0183 | -4.685163943 | -4.48 | -4.46 |
| 18.00 | -4.38831 | -4.9037 | -4.583962268 | -4.38 | -4.34 |
| 19.00 | -4.286332 | -4.789 | -4.479547035 | -4.27 | -4.25 |
| 20.00 | -4.187716 | -4.685 | -4.387861891 | -4.18 | -4.15 |
| 21.00 | -4.090782 | -4.581 | -4.298972875 | -4.08 | -4.04 |
| 22.00 | -3.997594 | -4.4805 | -4.209241614 | -3.99 | -3.95 |
| 23.00 | -3.907734 | -4.3865 | -4.131483694 | -3.90 | -3.85 |
| 24.00 | -3.820837 | -4.2904 | -4.044882679 | -3.81 | -3.78 |
| 25.00 | -3.737291 | -4.2012 | -3.966472044 | -3.73 | -3.69 |
| 26.00 | -3.65544 | -4.1095 | -3.8793781 | -3.65 | -3.62 |
| 27.00 | -3.576494 | -4.0245 | -3.805629076 | -3.57 | -3.55 |
| 28.00 | -3.499872 | -3.939 | -3.727703057 | -3.49 | -3.46 |
| 29.00 | -3.42478 | -3.8585 | -3.655383874 | -3.42 | -3.38 |
| 30.00 | -3.351595 | -3.7732 | -3.573600353 | -3.34 | -3.31 |
| 31.00 | -3.280315 | -3.6991 | -3.507245303 | -3.28 | -3.25 |
| 32.00 | -3.209977 | -3.6208 | -3.434472772 | -3.21 | -3.18 |
| 33.00 | -3.142468 | -3.5473 | -3.370117363 | -3.14 | -3.12 |
| 34.00 | -3.077279 | -3.4756 | -3.302287723 | -3.08 | -3.06 |
| 35.00 | -3.011773 | -3.398 | -3.225270853 | -3.01 | -3.00 |
| 36.00 | -2.948642 | -3.3276 | -3.158215574 | -2.95 | -2.94 |
| 37.00 | -2.886148 | -3.2592 | -3.095251008 | -2.89 | -2.88 |
| 38.00 | -2.822703 | -3.1928 | -3.038611055 | -2.83 | -2.83 |
| 39.00 | -2.760234 | -3.1266 | -2.981232845 | -2.77 | -2.77 |
| 40.00 | -2.697431 | -3.0607 | -2.925025806 | -2.71 | -2.70 |
| 41.00 | -2.635883 | -2.9916 | -2.864256871 | -2.64 | -2.65 |
| 42.00 | -2.57472 | -2.9243 | -2.802307268 | -2.58 | -2.59 |
| 43.00 | -2.512858 | -2.8584 | -2.744981991 | -2.52 | -2.53 |
| 44.00 | -2.451506 | -2.7918 | -2.685224185 | -2.46 | -2.48 |
| 45.00 | -2.389556 | -2.7244 | -2.623945648 | -2.40 | -2.42 |
| 46.00 | -2.328499 | -2.6576 | -2.563392902 | -2.34 | -2.35 |
| 47.00 | -2.267583 | -2.5922 | -2.506092408 | -2.28 | -2.29 |
| 48.00 | -2.205998 | -2.5271 | -2.450356045 | -2.22 | -2.23 |
| 49.00 | -2.145391 | -2.4609 | -2.390479129 | -2.16 | -2.17 |
| 50.00 | -2.084887 | -2.3935 | -2.32772203 | -2.10 | -2.10 |
| 51.00 | -2.02619 | -2.3294 | -2.2708261 | -2.04 | -2.05 |
| 52.00 | -1.96755 | -2.2638 | -2.210163758 | -1.98 | -1.98 |
| 53.00 | -1.910133 | -2.2003 | -2.152997932 | -1.92 | -1.93 |
| 54.00 | -1.852748 | -2.1364 | -2.089952778 | -1.87 | -1.87 |
| 55.00 | -1.796183 | -2.0757 | -2.03909902 | -1.81 | -1.80 |
| 56.00 | -1.740637 | -2.0117 | -1.978543687 | -1.75 | -1.74 |
| 57.00 | -1.68425 | -1.9489 | -1.921853707 | -1.69 | -1.69 |
| 58.00 | -1.62935 | -1.8878 | -1.862784599 | -1.64 | -1.64 |
| 59.00 | -1.573758 | -1.8254 | -1.806408485 | -1.58 | -1.58 |
| 60.00 | -1.518558 | -1.7653 | -1.750044106 | -1.53 | -1.53 |
| 61.00 | -1.465354 | -1.7037 | -1.692463286 | -1.47 | -1.48 |
| 62.00 | -1.411263 | -1.6438 | -1.635441938 | -1.42 | -1.42 |
| 63.00 | -1.357732 | -1.5863 | -1.583647305 | -1.37 | -1.37 |
| 64.00 | -1.305284 | -1.5252 | -1.526491527 | -1.31 | -1.32 |
| 65.00 | -1.254247 | -1.4686 | -1.473762967 | -1.26 | -1.26 |
| 66.00 | -1.203864 | -1.4136 | -1.42428393 | -1.21 | -1.21 |
| 67.00 | -1.152369 | -1.3552 | -1.367535669 | -1.16 | -1.16 |
| 68.00 | -1.102916 | -1.2996 | -1.315298512 | -1.11 | -1.11 |
| 69.00 | -1.053454 | -1.2419 | -1.25786003 | -1.06 | -1.06 |
| 70.00 | -1.006761 | -1.1895 | -1.205222278 | -1.02 | -1.02 |
| 71.00 | -0.959943 | -1.1341 | -1.150426598 | -0.97 | -0.97 |
| 72.00 | -0.912817 | -1.0788 | -1.095928932 | -0.92 | -0.93 |
| 73.00 | -0.866787 | -1.0251 | -1.044386321 | -0.87 | -0.88 |
| 74.00 | -0.822333 | -0.9747 | -0.993904115 | -0.83 | -0.83 |
| 75.00 | -0.778763 | -0.9264 | -0.947451384 | -0.79 | -0.79 |
| 76.00 | -0.735544 | -0.8729 | -0.892925389 | -0.74 | -0.75 |
| 77.00 | -0.693447 | -0.8213 | -0.836359498 | -0.70 | -0.71 |
| 78.00 | -0.652536 | -0.7738 | -0.788734382 | -0.66 | -0.67 |
| 79.00 | -0.612764 | -0.7272 | -0.742075466 | -0.62 | -0.63 |
| 80.00 | -0.57383 | -0.682 | -0.697705758 | -0.58 | -0.59 |
| 81.00 | -0.536437 | -0.6359 | -0.64923994 | -0.54 | -0.55 |
| 82.00 | -0.499471 | -0.5945 | -0.606971449 | -0.51 | -0.51 |
| 83.00 | -0.463512 | -0.5512 | -0.563496134 | -0.47 | -0.48 |
| 84.00 | -0.428492 | -0.5086 | -0.520979142 | -0.43 | -0.44 |
| 85.00 | -0.394493 | -0.469 | -0.479423152 | -0.40 | -0.40 |
| 86.00 | -0.361494 | -0.4312 | -0.441038598 | -0.37 | -0.37 |
| 87.00 | -0.330284 | -0.3935 | -0.405964488 | -0.33 | -0.34 |
| 88.00 | -0.299514 | -0.3567 | -0.367248293 | -0.30 | -0.31 |
| 89.00 | -0.269615 | -0.3216 | -0.332030247 | -0.27 | -0.28 |
| 90.00 | -0.24121 | -0.2869 | -0.295809586 | -0.24 | -0.25 |
| 91.00 | -0.213638 | -0.2557 | -0.2622913 | -0.22 | -0.22 |
| 92.00 | -0.185996 | -0.2215 | -0.226214728 | -0.19 | -0.19 |
| 93.00 | -0.159922 | -0.1892 | -0.193066441 | -0.16 | -0.17 |
| 94.00 | -0.135553 | -0.1599 | -0.160384187 | -0.14 | -0.14 |
| 95.00 | -0.112679 | -0.1315 | -0.127870591 | -0.12 | -0.11 |
| 96.00 | -0.090307 | -0.1038 | -0.101665857 | -0.09 | -0.09 |
| 97.00 | -0.069067 | -0.0795 | -0.077472302 | -0.07 | -0.06 |
| 98.00 | -0.048278 | -0.0554 | -0.053088935 | -0.05 | -0.05 |
| 99.00 | -0.028428 | -0.0317 | -0.028756155 | -0.03 | -0.02 |
| 100.00 | -0.00126 | -0.0012 | -0.001535668 | 0.00 | 0.00 |

Unconditional DIP2 distribution:

Company inputs are captured in Figure 6.3.2.2-2 based on raw data in Table 6.3.2.2-2.

Figure 6.3.2.2-2: Unconditional DIP2 calibration for heterogeneous network

Table 6.3.2.2-2: Company inputs on unconditional DIP2 calibration for heterogeneous network

| Percentile | China Telecom | Huawei | Ericsson | ZTE | Nokia Networks |
| --- | --- | --- | --- | --- | --- |
|  | R4-152823 | R4-152596 | R4-152958 | R4-153162 | R4-152236 |
| 0.00 | -49.543467 | -48.9462 | -45.96707372 | -47.25 | -41.1755 |
| 1.00 | -27.574739 | -27.2256 | -26.82023133 | -27.52 | -27.9357 |
| 2.00 | -24.758577 | -24.4041 | -23.92057703 | -24.76 | -24.5032 |
| 3.00 | -23.013773 | -22.5632 | -21.99215432 | -22.97 | -22.7165 |
| 4.00 | -21.673548 | -21.2525 | -20.73797502 | -21.62 | -21.2752 |
| 5.00 | -20.607936 | -20.1579 | -19.61771367 | -20.54 | -20.3665 |
| 6.00 | -19.660836 | -19.2143 | -18.62047355 | -19.64 | -19.4736 |
| 7.00 | -18.888721 | -18.4001 | -17.79039272 | -18.83 | -18.5401 |
| 8.00 | -18.169256 | -17.7094 | -17.13114368 | -18.12 | -17.8965 |
| 9.00 | -17.527723 | -17.0681 | -16.4759103 | -17.49 | -17.2164 |
| 10.00 | -16.950668 | -16.5038 | -15.90940485 | -16.93 | -16.6262 |
| 11.00 | -16.448888 | -15.9965 | -15.42896878 | -16.40 | -16.0936 |
| 12.00 | -15.957612 | -15.5185 | -14.95680591 | -15.92 | -15.6238 |
| 13.00 | -15.514013 | -15.0816 | -14.52541963 | -15.48 | -15.1917 |
| 14.00 | -15.115315 | -14.6701 | -14.10899869 | -15.07 | -14.8068 |
| 15.00 | -14.745529 | -14.2954 | -13.75212925 | -14.68 | -14.4543 |
| 16.00 | -14.386742 | -13.9465 | -13.40700237 | -14.33 | -14.1058 |
| 17.00 | -14.054434 | -13.6237 | -13.09567161 | -14.00 | -13.7179 |
| 18.00 | -13.736001 | -13.3164 | -12.80703577 | -13.68 | -13.4154 |
| 19.00 | -13.43708 | -13.007 | -12.48315968 | -13.38 | -13.1344 |
| 20.00 | -13.150785 | -12.7306 | -12.21230002 | -13.10 | -12.8191 |
| 21.00 | -12.884232 | -12.4731 | -11.96172794 | -12.84 | -12.5451 |
| 22.00 | -12.632761 | -12.2275 | -11.7355695 | -12.58 | -12.3077 |
| 23.00 | -12.399915 | -12.0093 | -11.54824622 | -12.34 | -12.0792 |
| 24.00 | -12.167588 | -11.8007 | -11.35988625 | -12.12 | -11.8464 |
| 25.00 | -11.945189 | -11.5937 | -11.17292357 | -11.90 | -11.5989 |
| 26.00 | -11.737436 | -11.4055 | -11.0044922 | -11.70 | -11.4142 |
| 27.00 | -11.54 | -11.2201 | -10.82764279 | -11.51 | -11.2307 |
| 28.00 | -11.348042 | -11.0426 | -10.66955214 | -11.32 | -11.035 |
| 29.00 | -11.165282 | -10.8692 | -10.50663207 | -11.14 | -10.8783 |
| 30.00 | -10.994824 | -10.709 | -10.36134187 | -10.97 | -10.7188 |
| 31.00 | -10.829004 | -10.552 | -10.22580471 | -10.81 | -10.5588 |
| 32.00 | -10.670545 | -10.4109 | -10.09712636 | -10.65 | -10.4053 |
| 33.00 | -10.518286 | -10.2686 | -9.966972635 | -10.50 | -10.2676 |
| 34.00 | -10.374122 | -10.1283 | -9.839610563 | -10.35 | -10.1155 |
| 35.00 | -10.232975 | -9.9958 | -9.718838888 | -10.21 | -9.9726 |
| 36.00 | -10.095104 | -9.868 | -9.608602719 | -10.07 | -9.8556 |
| 37.00 | -9.965729 | -9.7439 | -9.493419487 | -9.94 | -9.7233 |
| 38.00 | -9.838978 | -9.6297 | -9.389586233 | -9.82 | -9.6104 |
| 39.00 | -9.71653 | -9.5142 | -9.28125404 | -9.70 | -9.5022 |
| 40.00 | -9.596249 | -9.4009 | -9.178244102 | -9.58 | -9.3821 |
| 41.00 | -9.480811 | -9.2863 | -9.070557367 | -9.46 | -9.2722 |
| 42.00 | -9.368287 | -9.1852 | -8.983313517 | -9.35 | -9.1673 |
| 43.00 | -9.258204 | -9.0831 | -8.893259997 | -9.24 | -9.0661 |
| 44.00 | -9.151467 | -8.987 | -8.804551179 | -9.14 | -8.9698 |
| 45.00 | -9.048335 | -8.8847 | -8.71214705 | -9.03 | -8.8725 |
| 46.00 | -8.945224 | -8.7873 | -8.619549967 | -8.93 | -8.7659 |
| 47.00 | -8.84642 | -8.692 | -8.530772172 | -8.83 | -8.6665 |
| 48.00 | -8.749766 | -8.597 | -8.442593509 | -8.73 | -8.5829 |
| 49.00 | -8.654895 | -8.5115 | -8.365130684 | -8.64 | -8.4962 |
| 50.00 | -8.560824 | -8.4203 | -8.285538736 | -8.54 | -8.4025 |
| 51.00 | -8.469634 | -8.334 | -8.204996536 | -8.45 | -8.3062 |
| 52.00 | -8.379354 | -8.2495 | -8.128802076 | -8.36 | -8.2107 |
| 53.00 | -8.289745 | -8.1649 | -8.052274151 | -8.27 | -8.1303 |
| 54.00 | -8.201854 | -8.0785 | -7.970749428 | -8.18 | -8.0444 |
| 55.00 | -8.11436 | -7.9965 | -7.900084096 | -8.09 | -7.9722 |
| 56.00 | -8.027905 | -7.9194 | -7.82914192 | -8.01 | -7.888 |
| 57.00 | -7.941035 | -7.8356 | -7.753685593 | -7.92 | -7.8102 |
| 58.00 | -7.856325 | -7.7565 | -7.677067463 | -7.84 | -7.7385 |
| 59.00 | -7.773046 | -7.6797 | -7.605887439 | -7.76 | -7.6666 |
| 60.00 | -7.690194 | -7.6021 | -7.532732773 | -7.68 | -7.5909 |
| 61.00 | -7.608986 | -7.52 | -7.460265821 | -7.59 | -7.5179 |
| 62.00 | -7.526705 | -7.4434 | -7.389216375 | -7.51 | -7.4405 |
| 63.00 | -7.444492 | -7.3669 | -7.318530291 | -7.43 | -7.3537 |
| 64.00 | -7.362233 | -7.2925 | -7.253091606 | -7.35 | -7.2781 |
| 65.00 | -7.281532 | -7.2194 | -7.1905994 | -7.27 | -7.2064 |
| 66.00 | -7.200598 | -7.1428 | -7.11942897 | -7.19 | -7.1209 |
| 67.00 | -7.120811 | -7.0659 | -7.047166129 | -7.11 | -7.0521 |
| 68.00 | -7.03996 | -6.9874 | -6.970923491 | -7.03 | -6.9812 |
| 69.00 | -6.958005 | -6.9091 | -6.895789553 | -6.95 | -6.9113 |
| 70.00 | -6.876916 | -6.8298 | -6.817894955 | -6.87 | -6.8253 |
| 71.00 | -6.794676 | -6.7532 | -6.74692265 | -6.79 | -6.7539 |
| 72.00 | -6.713426 | -6.6767 | -6.675783996 | -6.71 | -6.6788 |
| 73.00 | -6.632305 | -6.5976 | -6.598541557 | -6.63 | -6.596 |
| 74.00 | -6.550674 | -6.5216 | -6.528745671 | -6.55 | -6.5128 |
| 75.00 | -6.469017 | -6.4362 | -6.448295553 | -6.46 | -6.4395 |
| 76.00 | -6.386187 | -6.3542 | -6.363913368 | -6.38 | -6.3582 |
| 77.00 | -6.302869 | -6.2695 | -6.285599749 | -6.29 | -6.2841 |
| 78.00 | -6.217483 | -6.1888 | -6.20509318 | -6.21 | -6.1942 |
| 79.00 | -6.132774 | -6.1067 | -6.133583566 | -6.12 | -6.1033 |
| 80.00 | -6.048661 | -6.0247 | -6.049491873 | -6.04 | -6.0109 |
| 81.00 | -5.960922 | -5.94 | -5.972318339 | -5.95 | -5.9077 |
| 82.00 | -5.871119 | -5.8553 | -5.895489648 | -5.86 | -5.8337 |
| 83.00 | -5.779327 | -5.7696 | -5.816639383 | -5.77 | -5.7419 |
| 84.00 | -5.686294 | -5.6735 | -5.724764013 | -5.67 | -5.6555 |
| 85.00 | -5.592282 | -5.5853 | -5.640196594 | -5.58 | -5.5493 |
| 86.00 | -5.494292 | -5.4953 | -5.552104662 | -5.49 | -5.4388 |
| 87.00 | -5.395035 | -5.3976 | -5.457677702 | -5.39 | -5.3486 |
| 88.00 | -5.292209 | -5.2997 | -5.363767885 | -5.29 | -5.252 |
| 89.00 | -5.18697 | -5.1907 | -5.25498573 | -5.18 | -5.1576 |
| 90.00 | -5.079376 | -5.0803 | -5.143500402 | -5.07 | -5.0547 |
| 91.00 | -4.966996 | -4.9629 | -5.027933178 | -4.95 | -4.9357 |
| 92.00 | -4.846939 | -4.8451 | -4.913479658 | -4.83 | -4.8118 |
| 93.00 | -4.718054 | -4.7243 | -4.793845092 | -4.71 | -4.672 |
| 94.00 | -4.583233 | -4.5899 | -4.666485906 | -4.57 | -4.5375 |
| 95.00 | -4.434075 | -4.4497 | -4.528243312 | -4.43 | -4.3968 |
| 96.00 | -4.274513 | -4.2916 | -4.372749386 | -4.27 | -4.2271 |
| 97.00 | -4.097934 | -4.1078 | -4.195063989 | -4.08 | -4.0537 |
| 98.00 | -3.887045 | -3.8926 | -3.972232774 | -3.87 | -3.8506 |
| 99.00 | -3.626277 | -3.6176 | -3.674925695 | -3.61 | -3.5934 |
| 100.00 | -3.029778 | -3.007 | -3.035074566 | -3.02 | -3.0502 |

## 6.4	Interference profile

This clause presents the system level simulation results for the interference profiles, based on the statistical measure and methodologies defined in clause 6.2. The independent results from different companies as well as the average values are shown in Table 6.4-1 and Table 6.4-2.

Table 6.4-1: Interference profiles for homogeneous network

|  |  | China Telecom (R4-152823) | Ericsson (R4-152958) | Huawei (R4-152597) | ZTE (R4-153161) | Nokia Networks (R4-153468) | Alcatel-Lucent (R4-153481) | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Methodology 1 | DIP1 (dB) | -1.11 | -1.21 | -1.20 | -1.12 | -1.06 | -0.96 | -1.11 |
|  | DIP2 (dB) | -10.94 | -10.77 | -10.30 | -10.82 | -11.65 | -10.95 | -10.91 |
| Methodology 2 | DIP1 (dB) | -1.14 | -1.58 | -1.43 | -1.06 |  |  | -1.30 |
|  | DIP2 (dB) | -9.17 | -7.99 | -8.16 | -9.42 |  |  | -8.68 |

Table 6.4-2: Interference profiles for heterogeneous network

|  |  | China Telecom (R4-152823) | Ericsson (R4-152958) | Huawei (R4-152597) | ZTE (R4-153162) | Average |
| --- | --- | --- | --- | --- | --- | --- |
| Methodology 1 | DIP1 (dB) | -0.39 | -0.48 | -0.47 | -0.40 | -0.43 |
|  | DIP2 (dB) | -14.10 | -13.1 | -13.5 | -14.07 | -13.69 |
| Methodology 2 | DIP1 (dB) | -0.42 | -0.56 | -0.58 | -0.43 | -0.50 |
|  | DIP2 (dB) | -12.20 | -11.45 | -11.63 | -12.28 | -11.89 |

<< End of text proposal >>