# 3GPP TSG-RAN WG4 Meeting #76bis                                                           	R4-155992

# Sophia Antipolis, France, 12 – 16 Oct, 2015

Title: 	TP for TR 36.854-13 on Co-existence analysis for 4DL CA_39C-41C

Source: 	CMCC

Agenda Item:	7.42.2.4

Document for:	Approval

1.	Introduction

The WI proposal to support the LTE Advanced 4 Band Carrier Aggregation (4DL/1UL) of Band 39, Band 39, Band 41 and Band 41 was approved in RAN#69 [1].

In this contribution, on co-existence analysis is proposed for LTE Advanced 4 Band Carrier Aggregation (4DL/1UL) of Band 39, Band 39, Band 41 and Band 41.

2.	Discussion

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting the 4DL CA configurations of CA_39C_41C can be calculated as shown in Table 2-1 below

Table 2-1: Band 39 and Band 41 4DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1880 | 1920 | 2496 | 2690 |
| 2nd harmonics frequency limits (MHz) | 3760 | 3840 | 4992 | 5380 |
| 3rd harmonics frequency limits (MHz) | 5640 | 5760 | 7488 | 8070 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 576 | 810 | 4376 | 4610 |
| 3rd order IMD products | (2*f1-low – f2-high) | (2*f1-high – f2-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 1070 | 1344 | 3072 | 3500 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 6256 | 6530 | 6872 | 7300 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 1840 | 1960 | 2461 | 2725 |

It can be seen from Table 2-1 that 2nd order harmonics may fall into BS receive band of band 43, two-tone 2nd order IMD products may fall into BS receive band of band 12, 13, 14, 17, 27, 28, 44. 3rd order IMD products may fall to UL frequencies of Band 1, 2, 7, 22, 25, 33, 35, 36, 37, 38 or 42.

It is suggested BS transmitters supporting CA of Band 39, Band 39,Bnd 41and Band 41 should not share the same antenna with Band 1, 2, 7, 12, 13, 14, 17, 22, 25, 27, 28, 33, 35, 36, 37, 38, 42, 43 or 44 BS receiver, unless the antenna path meets very stringent third order PIM specification so that the PIM will not cause Band 1, 2, 7, 12, 13, 14, 17, 22, 25, 27, 28, 33, 35, 36, 37, 38, 42, 43 or 44 BS receiver desensitization.

For single UL non-simultaneous UE, the 2nd and 3rd harmonic analysis for B39 and B41 is done in the 2DL/1UL TR 36.851. None of the products falls into the B39 or B41 UE receiving bands.

3        References

[1]	R4-151176,  “New WID: LTE Advanced 4 Band Carrier Aggregation (4DL/1UL) of Band 39, Band 39, Band 41 and Band 41”, CMCC.

2.	Text Proposal

<Start of change>

6.X   LTE Advanced Carrier Aggregation of Band 39 and Band 39 and Band 41 and Band 41 (1 UL)

### 6.X.1	List of specific combination issues

6.X.1.2	Co-existence studies for CA_39C-41C

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting the 4DL CA configurations of CA_39C_41C can be calculated as shown in Table 6.X.1.2-1 below:

Table 6.X.1.2-1: Band 39 and Band 41 4DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1880 | 1920 | 2496 | 2690 |
| 2nd harmonics frequency limits (MHz) | 3760 | 3840 | 4992 | 5380 |
| 3rd harmonics frequency limits (MHz) | 5640 | 5760 | 7488 | 8070 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 576 | 810 | 4376 | 4610 |
| 3rd order IMD products | (2*f1-low – f2-high) | (2*f1-high – f2-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 1070 | 1344 | 3072 | 3500 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 6256 | 6530 | 6872 | 7300 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 1840 | 1960 | 2461 | 2725 |

It can be seen from Table 6.X.1.2-1 that 2nd order harmonics may fall into BS receive band of band 43, two-tone 2nd order IMD products may fall into BS receive band of band 12, 13, 14, 17, 27, 28, 44. 3rd order IMD products may fall to UL frequencies of Band 1, 2, 7, 22, 25, 33, 35, 36, 37, 38 or 42.

It is suggested BS transmitters supporting CA of Band 39, Band 39,Bnd 41and Band 41 should not share the same antenna with Band 1, 2, 7, 12, 13, 14, 17, 22, 25, 27, 28, 33, 35, 36, 37, 38, 42, 43 or 44 BS receiver, unless the antenna path meets very stringent third order PIM specification so that the PIM will not cause Band 1, 2, 7, 12, 13, 14, 17, 22, 25, 27, 28, 33, 35, 36, 37, 38, 42, 43 or 44 BS receiver desensitization.

For single UL non-simultaneous UE, the 2nd and 3rd harmonic analysis for B39 and B41 is done in the 2DL/1UL TR 36.851. None of the products falls into the B39 or B41 UE receiving bands.

<End of change>