3GPP TSG RAN WG4 #76BIS meeting                   	R4-156138

Sofia Antipolis, France, 12th -16th October 2015

- Agenda Item:	9.1

- Source:	LG Electronics

- Title:	A-MPR simulation results for Band 65 UE to protect adjacent channels

Document for:	Discussion

- Introduction

In the last RAN4 #76meeting, RAN4 agreed the A-MPR tables according to coexistence requirements to protect Band 34. So RAN4 just remained to decide the detail A-MPR values for protection of PHS band. And also same approach for B34 can be applied to protect Band 33/B39.

In this contribution, we provide coexistence results to protect B33 and B39 to complete all related coexistence issues for Band 65 WI and SI.

- References

- R4-154936, “TP for TR 36.861: Co-existence with PHS”, Ericsson

- R4-155275, “TP for TR 36.861: Co-existence with Band 34 in Japan”, Ericsson

- R4-155314, “A-MPR analysis of MSS UE for coexistence requirements”, LG Electronics, Ericsson, Nokia Networks

Text proposal for TR 36.861

******************** Start of text proposal to TR 36.861 Chapter 6.2.2 *****************

- 6.2.2.2	UE co-existence analysis to protect Band 34

- 6.2.2.2.1	Summary results

The coexistence studies for Band 34 protection consider A-MPR or UL RB restriction. 
In addition, PUCCH over-provisioning is also studied when A-MPR is specified. A-MPR and UL RB restriction studies are included for different emission protection levels for Band 34 (-50, -40, -30, and -15.5 dBm/MHz). 
In addition, different frequency separations between the E-UTRA carrier edge and the protected range are assumed. The required A-MPR simulation results are summarized in Tables 6.2.2.2.1-1 and 6.2.2.2.1-2. For the Table 6.2.2.2.1-1, the worst A-MPR values are chosen for each simulation cases from Annex A2.

Table 6.2.2.2.1-1: Summary of coexistence studies with Band 34 (A-MPR case)

| Case | E-UTRA Channel Bandwidth (MHz) | Separation between E-UTRA carrier edge and protected range (MHz) | Spurious emissions protection level (dBm/MHz) | A-MPR (dB) | PUCCH over-provisioning required |
| --- | --- | --- | --- | --- | --- |
| 1 | 5/10/15/20 | 0 | -50 | 17/ 17/ 17/ 17 |  |
|  |  |  | -40 | 15/14/14/14 |  |
|  |  |  | -30 | 12/11/11/11 |  |
|  |  |  | -15.5 | 5/4/4/3 |  |
| 2 | 5/10/15/20 | 5 | -50 | 10/15/15/15 |  |
|  |  |  | -40 | 5/10/10/10 |  |
|  |  |  | -30 | 2/5/6/6 |  |
|  |  |  | -15.5 | 1/1/1/1 |  |
| 3 | 5/10/15/20 | 10 | -50 | 3/10/15/15 |  |
|  |  |  | -40 | 1/5/10/10 |  |
|  |  |  | -30 | 0/1/5/5 |  |
|  |  |  | -15.5 | 1/1/1/1 |  |

Observation : From these simulation results, the combination of 5MHz guard band and -40dBm/MHz as a coexistence requirements level is more beneficial.

Table 6.2.2.2.1-2: Summary of coexistence studies with Band 34 (UL RB Restriction case)

| Case | E-UTRA Channel Bandwidth (MHz) | Separation between E-UTRA carrier edge and protected range (MHz) | Spurious emissions protection level (dBm/MHz) | UL RB Restriction required |
| --- | --- | --- | --- | --- |
| 4 | 5/10/15/20 | 0 | -50 |  |
|  |  |  | -40 |  |
|  |  |  | -30 |  |
|  |  |  | -15.5 |  |
| 5 | 5/10/15/20 | 5 | -50 |  |
|  |  |  | -40 |  |
|  |  |  | -30 |  |
|  |  |  | -15.5 |  |
| 6 | 5/10/15/20 | 10 | -50 |  |
|  |  |  | -40 |  |
|  |  |  | -30 |  |
|  |  |  | -15.5 |  |

- 6.2.2.3	UE co-existence analysis to protect PHS band

- 6.2.2.3.1	Summary results

For the PHS protection, RAN WG4 has specified NS_05 for Band 1 UE to protect PHS band. However, lower Band 1 (1920–1940 MHz) regions had some RB allocation restrictions as shown in figure 6.2.2.3.1-1. In order to maximize efficiency of DL side, PUCCH overprovisioning was specified.  However, still lower Band 1 spectrum holder has been forced to discard UL spectra.  Therefore, introduction of new A-MPR scheme to utilize more UL spectra will be more suitable for the new band.

A UE needs both the band number and NS value to be able to decode the maximum A-MPR that it is allowed to apply. Both are signalled by the network in the system information, wherein each NS value is associated with a frequency band indicator (e.g. 1 or 65) in multi-band signalling. This combination of NS+band number message allows for the use of the same NS value for several E-UTRA operating bands with specific A-MPR values per operating band. Thus, NS_05 can be associated to Band 1 as well as Band 65 since the same emissions requirement to protect PHS band is indicated for both these frequency ranges.

Figure 6.2.2.3.1-1 NS_05 for Band 1 (upper figure) and Band 65 (lower figure)

In above figure 6.2.2.3.1-1, utilization of UL spectrum with the same network signaled value (NS_05) is shown for the lower part of Band 1 and Band 65.

Table 6.2.2.3.1-1 is the summary of A-MPR study results to protect PHS Band according to the each channel bandwidths and each guard band [7][8][9][10]. From the simulation results, we can derive the detail A-MPR Tables.

Table 6.2.2.3.1-1: Summary of coexistence studies with PHS Band (A-MPR case)

[Editor Note: Agreed Detail A-MPR Tables will be captured in here]

- 6.2.2.4	UE co-existence analysis to protect Band 33/Band 39

- 6.2.2.4.1	Summary results

For protection of B33/B39, RAN4 need further discussion to define UE-to-UE coexistence with different coexistence requirements compare to Band 1 legacy UE. In the previous analysis of Band 1 UE coexistence requirements, TDD-FDD coexistence approach without A-MPR had applied in TS36.101. However, it also can be defined the UE coexistence requirements with A-MPR using NS_xx or make a guard band as same approach to Band 34 protection.

We propose UE coexistence analysis results [9] in Table 6.2.2.4.1-1 to protect B33/B39 UE for MSS band UE.

Table 6.2.2.4.1-1: Summary of coexistence studies to protect Band33/Band 39 (A-MPR case)

| Case | E-UTRA Channel Bandwidth (MHz) | Separation between E-UTRA carrier edge and protected range (MHz) | Spurious emissions protection level (dBm/MHz) | Maximum A-MPR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 5/10/15/20 | 0 (B33/B39) | -50 | 17.5/17/17/17 |
|  |  |  | -40 | 15/15/14/14 |
|  |  |  | -30 | 12/11/10/9 |
|  |  |  | -15.5 | 5/4/3/2.5 |
| 2 | 5/10/15/20 | 5 (B33/B39) | -50 | TBD/TBD/TBD/TBD |
|  |  |  | -40 | TBD/TBD/TBD/TBD |
|  |  |  | -30 | TBD/TBD/TBD/TBD |
|  |  |  | -15.5 | TBD/TBD/TBD/TBD |
| 3 | 5/10/15/20 | 20 (B39) | -50 | [0]/[5]/[5]/[13] |
|  |  |  | -40 | [0]/[0]/[1]/[4] |
|  |  |  | -30 | [0]/[0]/[[0]/[0] |
|  |  |  | -15.5 | [0]/[0]/[[0]/[0] |

Detail performance results are captured in Annex B section.

***** Unchanged sections are omitted *******

- Annex B (informative): 
A-MPR simulation results to protect Band 33/39

## B.1	simulation results

- B.1-1 simulation results from LG Electronics

To Protect 1920~1900MHz with 0GHz gap from B65, we summary the A-MPR results according to the RB_Start, RB_Length, each channel BW and each coexistence requirements.

(a) 20MHz                                                           (b) 15MHz

(c) 10MHz                                                            (d) 5MHz

Figure B.1.1-1 A-MPR simulation results (-50dBm/MHz as a coexistence level, 0 MHz GB)

(a) 20MHz                                                            (b) 15MHz

(c) 10MHz                                                            (d) 5MHz

Figure B.1.1-2 A-MPR simulation results (-40dBm/MHz as a coexistence level, 0 MHz GB)

(a) 20MHz                                                            (b) 15MHz

(c) 10MHz                                                            (d) 5MHz

Figure B.1.1-3 A-MPR simulation results (-30dBm/MHz as a coexistence level, 0 MHz GB)

(a) 20MHz                                                            (b) 15MHz

(c) 10MHz                                                            (d) 5MHz

Figure B.1.1-4 A-MPR simulation results (-15.5dBm/5MHz as a coexistence level, 0 MHz GB)

To Protect 1880~1900MHz with 20GHz gap from B65, we summary the A-MPR results according to the RB_Start, RB_Length, each channel BW and each coexistence requirements.

(a) 20MHz                                                           (b) 15MHz

(c) 10MHz                                                            (d) 5MHz

Figure B.1.2-1 A-MPR simulation results (-50dBm/MHz as a coexistence level, 20 MHz GB)

(a) 20MHz                                                           (b) 15MHz

(c) 10MHz                                                            (d) 5MHz

Figure B.1.2-2 A-MPR simulation results (-40dBm/MHz as a coexistence level, 20 MHz GB)

(a) 20MHz                                                           (b) 15MHz

(c) 10MHz                                                            (d) 5MHz

Figure B.1.2-3 A-MPR simulation results (-30dBm/MHz as a coexistence level, 20 MHz GB)

(a) 20MHz                                                           (b) 15MHz

(c) 10MHz                                                            (d) 5MHz

Figure B.1.2-4 A-MPR simulation results (-15.5dBm/5MHz as a coexistence level, 20 MHz GB)