3GPP TSG-RAN WG4 (Radio) Meeting #76bis	R4-156395

Sophia Antipolis, France, 12 – 16 October 2015

Agenda Item:	7.26.2

Source: 	Alcatel-Lucent, TeliaSonera, Nokia Networks

Title: 	TP for TR 36.852-13: Updates of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (20 + 28)

Document for:	Approval

# 1.	Introduction

The SI proposal to support LTE Advanced 2 Band Carrier Aggregation (2DL/1UL) of Band 20 and Band 28was approved in RAN#68 [1]. One objective of the WI is to specify the band-combinations specific Radio Frequency (RF) requirements for this new Carrier Aggregation (CA) band combination.

The impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting carrier aggregation of this new bandwidth combination to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to update the coexistence studies in the 2 Band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<Start of change>

##### 6.3.8.1.2	Co-existence studies

Table 6.3.8.1.2-1 shows the second and third order DL harmonics and intermodulation products when two simultaneous DLs are active in Band 20 and Band 28. Band 28 is limited to lower duplexer frequencies.

Table 6.3.8.1.2-1: Band 20 and Band 28 (lower duplex) DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 788 | 791 | 821 |
| 2nd order harmonics frequency range (MHz) | 1516 | 1576 | 1582 | 1642 |
| 3rd order harmonics frequency range (MHz) | 2274 | 2364 | 2373 | 2463 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | (f2_low + f1_low) | (f2_high + f1_high) |
| IMD frequency limits (MHz) | 3 | 63 | 1549 | 1609 |
| 3rd order IMD products | (2*f1_low – f2_high) | (2*f1_high – f2_low) | (2*f2_low – f1_high) | (2*f2_high – f1_low) |
| IMD frequency limits (MHz) | 695 | 785 | 794 | 884 |
| 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 2307 | 2397 | 2340 | 2430 |
| 3rd order IMD products | (f1_low – f2_high + f2_low) | (f1_high + f2_high – f2_low) | (f2_low – f1_high + f1_low) | (f2_high + f1_high – f1_low) |
| IMD frequency limits (MHz) | 728 | 818 | 761 | 851 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1_low – f2_BWmax) | (f1_high + f2_BWmax) | (f2_low – f1_BWmax) | (f2_high + f1_BWmax) |
| IMD frequency limits (MHz) | 738 | 808 | 771 | 841 |

It can be seen from table 6.3.8.1.2-1 that 2nd harmonics caused by BS supporting carrier aggregation of Band 20 and Band 28 may fall into the BS receive band of Band 24, while 3rd harmonic will fall into the BS receive band of Bands 30 and 40.

It can be seen from table 6.3.8.1.2-1 that none 2nd IMD products caused by BS supporting carrier aggregation of Band 20 and Band 28 will fall into the BS receive band of currently defined 3GPP bands, while 3rd IMD products may fall into the BS receive band of Bands 5, 6, 8, 12-14, 17-20, 26-28, 30, 40 and 44. Bands 5, 6, 12-14, 17-19, 26-27, 30, 40 and 44 are not intended for use in the same geographical area as Bands 20 and 28.

The analysis below gives further information on the actual channel bandwidths and carrier positions for that CA combinations which can cause DESENSE to the BS own receive blocks.

Assume that the 2 CC transmitted by the Band (20 + 28) BS are a and b MHz channel bandwidth and c and d MHz from the edges of Bands 20 and 28 DL frequency bands as shown in Figure 6.3.8.1.2-1 below:

f2-high

f2-low

f1-high

f1-low

1st CC

2nd CC

758

821

c MHz

d MHz

a MHz

b MHz

MHz

Figure 6.3.8.1.2-1: CC transmitted by the Band (20 + 28) BS

And the corresponding BS receive blocks are as shown in Figure 6.3.8.1.2-2 below:

f1-low

f1-high

f2-low

f2-high

1st CC

2nd CC

703

862

c MHz

d MHz

a MHz

b MHz

MHz

Figure 6.3.8.1.2-2: CC received by the Band (20 + 28) BS

Then the 3rd order IMD products caused in the BS by transmitting of the 2 CC can be calculated as shown in Table 6.3.8.1.2-2 below:

Table 6.3.8.1.2-2: Band (20 + 28) BS DL 3rd order IMD products

| 3rd order IMD products | IMD frequency limits (MHz) |
| --- | --- |
| (2*f1-low – f2-high) | 695 + 2*c + d |
| (2*f1-high – f2-low) | 695 + 2*c + d + 2*a + b |
| (2*f2-low – f1-high) | 884 – 2*d – c – 2*b – a |
| (2*f2-high – f1-low) | 884 – 2*d – c |
| (f1-low – f2-high + f2-low) | 758 + c – b |
| (f1-high + f2-high – f2-low) | 758 + c + b + a |
| (f2-low – f1-high + f1-low) | 821 – d – a – b |
| (f2-high + f1-high – f1-low) | 821 – d + a |

Comparing the IMD frequency limits in Table 6.3.8.1.2-2 with the BS receive blocks in Figure 6.3.8.1.2-2, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of the 2 CC will only fall into the BS own receive blocks of Bands 20 and 28 under one of the transmit configurations shown in Table 6.3.8.1.2-3 below:

Table 6.3.8.1.2-3: Band (20 + 28) BS transmit configurations with 3rd IMD falling within Band (20 + 28) BS own receive blocks

| 1st CC channel bandwidth (a MHz) | 2nd CC channel bandwidth (b MHz) | 1st and 2nd CC position (c and d MHz) with IMD falling into 1st CC receive block | 1st and 2nd CC position (c and d MHz) with IMD falling into 2nd CC receive block |  |
| --- | --- | --- | --- | --- |
| 10 | 10 | c + d < 18 | c + d < 32 |  |
| 10 | 15 | c + d < 18 | c + d <= 35 |  |
| 15 | 10 | c + d < 23 | c + d < 32 |  |
| 10 | 20 | c + d < 18 | c + d <= 30 |  |
| 15 | 15 | c + d < 23 | c + d <= 30 |  |
| 20 | 10 | c + d < 28 | c + d <= 30 |  |
| 15 | 20 | c + d < 23 | c + d <= 25 |  |
| 20 | 15 | c + d <= 25 | c + d <= 25 |  |
| 20 | 20 | c + d <= 20 | c + d <= 20 |  |

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Bands 8, 20 and 28 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 20 + 28 BS transmitter does not share the same antenna with Bands 8, 20 and 28 BS receiver.

Therefore, it is recommended that Bands 20 + 28 BS transmitter should not share the same antenna with Bands 8, 20 and 28 BS receiver to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 8, 20 and 28 BS receiver desensitization. PIM may be also caused outside the antenna system by e.g. metal objects close to the antennas and cause possible DESENSE into the own receiver, see TR 37.808 Section 6.1.

Table 6.3.8.1.2-4 gives the harmonic products for Bands 20+28 CA with 1UL (Band 28 is limited to lower duplexer frequencies). None of the harmonic products fall into the own UE receive bands.

Table 6.3.8.1.2-4: 1UL B20 + B28 (lower duplex) harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 | 832 | 862 |
| 2nd order harmonics frequency range (MHz) | 1406 to 1466 |  | 1664 to 1724 |  |
| 3rd order harmonics frequency range (MHz) | 2109 to 2199 |  | 2496 to 2586 |  |

<End of change>

# References

[1]	RP-151101, “New SI proposal: Study on LTE Advanced 2 Band Carrier Aggregation (2DL/1UL) of Band 20 and Band 28”, TeliaSonera.

[2]	R4-155076, “	TP to TR 36.852-13: LTE Advanced inter-band Carrier Aggregation of Band 20 and Band 28”, Nokia Networks.

[3]	R4-156121, “TR 36.852-13: LTE-A Rel-13 2DL Inter-band Carrier Aggregation v0.6.0”, Ericsson.