3GPP TSG-RAN WG4 (Radio) Meeting #76bis	R4-156398

Sophia Antipolis, France, 12 – 16 October 2015

Agenda Item:	7.42.2.1

Source: 	Alcatel-Lucent, U.S. Cellular

Title: 	TP for TR 36.852-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced intra-band non-contiguous Carrier Aggregation in Band 12

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced 4-Band Carrier Aggregation of Band 4, Band 5, Band 12 and Band 12 was approved in RAN#69 [1]. One objective of the WI is to specify the specific RF requirements for Band 12 intra-band non-contiguous 2DL Carrier Aggregation (CA).

In this paper, we investigate the impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting intra-band non-contiguous CA in this band to the receiver of own or different BS, and provide a text proposal to record the findings in the Technical Report [2].

# 2.	Discussion

For LTE Advanced BS supporting intra-band non-contiguous CA, the transceivers of the two or more non-contiguous Component Carriers (CC) within the CA band would typically share a common radio front-end and antenna system. When BS for two or more bands share the same antenna, operators should ensure the following:

- One band BS transmitter Out-Of-Band Emissions (OOBE) including harmonics and IMD falling into the other BS receive band do not cause BS receiver desensitization; and one band BS transmit carrier powers do not result in the other band BS receiver blocking.

- Third order Passive Intermodulations (PIM) generated by BS antenna path (including antennas, cables and connectors) and caused by multiple transmit band carriers do not fall into one of multiple BS receive bands and result in BS receiver desensitization.

## 2.1	Harmonics and IMD Products

Harmonics are produced by the BS oscillator or generated in non-linear circuit elements; while IMD products are caused by the mixing of multiple frequencies in non-linear circuit elements. Wireless systems producing transmit signals within multiple frequency blocks must consider harmonics and IMD generation as BS receiver desensitization may occur when harmonics or IMD frequencies fall within the receive band of own or different BS. Usually the non-linear element is an amplifier or mixer, but RF connectors and BS antenna systems can also generate harmonics and IMDs due to imperfect physical contact or dissimilar metals used in their construction. Wireless BS are particularly sensitive to harmonics and IMD interference which fall upon uplink receiver channels that are in use by the BS. To limit the receiver desensitization to 0.4 dB, in-band harmonics and IMD interference must be limited to less than 10 dB below the receiver noise floor.

Harmonics and IMD are measured as the ratio in power levels between the distortion signal and the input signal in units of dBc. The magnitudes of the harmonics and IMD depend on the characteristics of the non-linear element, and the magnitude normally decreases as the order of the harmonics and IMD increases. For this reason, lower order harmonics and IMD products are considered the most damaging to wireless systems, while the higher order harmonics and IMD products are generally less damaging. Therefore, we will focus our investigation on the 2nd and 3rd order harmonics and IMD products caused by transmitting of non-contiguous DL carriers for Band 12 in the BS.

## 2.2	BS Harmonics and IMD for CA

The LTE operating bands currently defined in the 3GPP standards are provided in Table 1 below [3]:

Table 1: LTE frequency bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |  |  |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  |  |  | FDL_low – FDL_high |  |  |  |  |  |  |  |
| 1 | 1920 MHz | – |  | 1980 MHz |  | 2110 MHz |  | – |  |  | 2170 MHz |  | FDD |
| 2 | 1850 MHz | – |  | 1910 MHz |  | 1930 MHz |  | – |  |  | 1990 MHz |  | FDD |
| 3 | 1710 MHz | – |  | 1785 MHz |  | 1805 MHz |  | – |  |  | 1880 MHz |  | FDD |
| 4 | 1710 MHz | – |  | 1755 MHz |  | 2110 MHz |  | – |  |  | 2155 MHz |  | FDD |
| 5 | 824 MHz | – |  | 849 MHz |  | 869 MHz |  | – |  |  | 894MHz |  | FDD |
| 6 (NOTE 1) | 830 MHz | – |  | 840 MHz |  | 875 MHz |  | – |  |  | 885 MHz |  | FDD |
| 7 | 2500 MHz | – |  | 2570 MHz |  | 2620 MHz |  | – |  |  | 2690 MHz |  | FDD |
| 8 | 880 MHz | – |  | 915 MHz |  | 925 MHz |  | – |  |  | 960 MHz |  | FDD |
| 9 | 1749.9 MHz | – |  | 1784.9 MHz |  | 1844.9 MHz |  | – |  |  | 1879.9 MHz |  | FDD |
| 10 | 1710 MHz | – |  | 1770 MHz |  | 2110 MHz |  | – |  |  | 2170 MHz |  | FDD |
| 11 | 1427.9 MHz | – |  | 1447.9 MHz |  | 1475.9 MHz |  | – |  |  | 1495.9 MHz |  | FDD |
| 12 | 699 MHz | – |  | 716 MHz |  | 729 MHz |  | – |  |  | 746 MHz |  | FDD |
| 13 | 777 MHz | – |  | 787 MHz |  | 746 MHz |  | – |  |  | 756 MHz |  | FDD |
| 14 | 788 MHz | – |  | 798 MHz |  | 758 MHz |  | – |  |  | 768 MHz |  | FDD |
| 15 | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  | FDD |
| 16 | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  | FDD |
| 17 | 704 MHz | – |  | 716 MHz |  | 734 MHz |  | – |  |  | 746 MHz |  | FDD |
| 18 | 815 MHz | – |  | 830 MHz |  | 860 MHz |  | – |  |  | 875 MHz |  | FDD |
| 19 | 830 MHz | – |  | 845 MHz |  | 875 MHz |  | – |  |  | 890 MHz |  | FDD |
| 20 | 832 MHz | – |  | 862 MHz |  | 791 MHz |  | – |  |  | 821 MHz |  |  |
| 21 | 1447.9 MHz | – |  | 1462.9 MHz |  | 1495.9 MHz |  | – |  |  | 1510.9 MHz |  | FDD |
| 22 | 3410 MHz | – |  | 3490 MHz |  | 3510 MHz |  | – |  |  | 3590 MHz |  | FDD |
| 23 | 2000 MHz | – |  | 2020 MHz |  | 2180 MHz |  | – |  |  | 2200 MHz |  | FDD |
| 24 | 1626.5 MHz | – |  | 1660.5 MHz |  | 1525 MHz |  | – |  |  | 1559 MHz |  | FDD |
| 25 | 1850 MHz | – |  | 1915 MHz |  | 1930 MHz |  | – |  |  | 1995 MHz |  | FDD |
| 26 | 814 MHz | – |  | 849 MHz |  | 859 MHz |  | – |  |  | 894 MHz |  | FDD |
| 27 | 807 MHz | – |  | 824 MHz |  | 852 MHz |  | – |  |  | 869 MHz |  | FDD |
| 28 | 703 MHz | – |  | 748 MHz |  | 758 MHz |  | – |  |  | 803 MHz |  | FDD |
| 29 | N/A |  |  |  |  | 717 MHz | – |  |  | 728 MHz |  |  | FDD (NOTE 2) |
| 30 | 2305 MHz | – |  | 2315 MHz |  | 2350 MHz |  | – |  |  | 2360 MHz |  | FDD |
| 31 | 452.5 MHz |  | – |  | 457.5 MHz | 462.5 MHz |  |  | – |  |  | 467.5 MHz | FDD |
| 32 | N/A |  |  |  |  | 1452 MHz |  | – |  |  | 1496 MHz |  | FDD (NOTE 2) |
| 33 | 1900 MHz | – |  | 1920 MHz |  | 1900 MHz |  | – |  |  | 1920 MHz |  | TDD |
| 34 | 2010 MHz | – |  | 2025 MHz |  | 2010 MHz |  | – |  |  | 2025 MHz |  | TDD |
| 35 | 1850 MHz | – |  | 1910 MHz |  | 1850 MHz |  | – |  |  | 1910 MHz |  | TDD |
| 36 | 1930 MHz | – |  | 1990 MHz |  | 1930 MHz |  | – |  |  | 1990 MHz |  | TDD |
| 37 | 1910 MHz | – |  | 1930 MHz |  | 1910 MHz |  | – |  |  | 1930 MHz |  | TDD |
| 38 | 2570 MHz | – |  | 2620 MHz |  | 2570 MHz |  | – |  |  | 2620 MHz |  | TDD |
| 39 | 1880 MHz | – |  | 1920 MHz |  | 1880 MHz |  | – |  |  | 1920 MHz |  | TDD |
| 40 | 2300 MHz | – |  | 2400 MHz |  | 2300 MHz |  | – |  |  | 2400 MHz |  | TDD |
| 41 | 2496 MHz | – |  | 2690 MHz |  | 2496 MHz |  | – |  |  | 2690 MHz |  | TDD |
| 42 | 3400 MHz | – |  | 3600 MHz |  | 3400 MHz |  | – |  |  | 3600 MHz |  | TDD |
| 43 | 3600 MHz | – |  | 3800 MHz |  | 3600 MHz |  | – |  |  | 3800 MHz |  | TDD |
| 44 | 703 MHz | – |  | 803 MHz |  | 703 MHz |  | – |  |  | 803 MHz |  | TDD |
| NOTE 1: Band 6 is not applicable. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |  |  |  |  |  |  |

It can be seen from Table 1 that Band 12 is operating at 699 – 716 MHz for UL and 729 – 746 MHz DL. Therefore, the 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of non-contiguous Band 12 DL carriers can be calculated as shown in Table 2 below:

Table 2: Band 12 DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 729 | 746 |
| 2nd harmonics frequency limits (MHz) | 1458 | 1492 |
| 3rd harmonics frequency limits (MHz) | 2187 | 2238 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -17 | 17 |
| 3rd order IMD products | (2*f-low – f-high) | (2*f-high – f-low) |
| IMD frequency limits (MHz) | 712 | 763 |

It can be seen from Table 2 that the 2nd harmonics of BS transmitting in Band 12 may fall into the BS receive band of Band 21, while the 3rd IMD products caused by BS supporting non-contiguous CA in Band 12 may fall into the BS receive band of Bands 12, 17, 28 and 44.

Assume that the 2 non-contiguous CC transmitted by the Band 12 BS are a and b MHz channel bandwidth and c and d MHz from the edges of Band 12 DL frequency band as shown in Figure 1 below:

f2-high

f2-low

f1-high

f1-low

1st CC

2nd CC

729

746

c MHz

d MHz

a MHz

b MHz

MHz

Figure 1: Non-contiguous CC transmitted by the Band 12 BS

And the corresponding BS receive blocks are as shown in Figure 2 below:

f1-low

f1-high

f2-low

f2-high

1st CC

2nd CC

699

716

c MHz

d MHz

a MHz

b MHz

MHz

Figure 2: Non-contiguous CC received by the Band 12 BS

Then the 3rd order IMD products caused in the BS by transmitting of the 2 CC can be calculated as shown in Table 3 below:

Table 3: Band 12 BS DL 3rd order IMD products

| 3rd order IMD products | IMD frequency limits (MHz) |
| --- | --- |
| (2*f1-low – f2-high) | 712 + 2*c + d |
| (2*f1-high – f2-low) | 712 + 2*c + d + 2*a + b |
| (2*f2-low – f1-high) | 763 – 2*d – c – 2*b – a |
| (2*f2-high – f1-low) | 763 – 2*d – c |
| (f1-low – f2-high + f2-low) | 729 + c – b |
| (f1-high + f2-high – f2-low) | 729 + c + b + a |
| (f2-low – f1-high + f1-low) | 746 – d – a – b |
| (f2-high + f1-high – f1-low) | 746 – d + a |

Comparing the IMD frequency limits in Table 3 with the BS receive blocks in Figure 2, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of the 2 CC will only fall into the BS own receive blocks of Band 12 under one of the transmit configurations shown in Table 4 below (note that a and b can only be 5 as stated in the WIDS):

Table 4: Band 12 BS transmit configurations with 3rd IMD within Band 12 BS own receive blocks

| 1st CC channel bandwidth (a MHz) | 2nd CC channel bandwidth (b MHz) | 1st and 2nd CC position (c and d MHz) |
| --- | --- | --- |
| 5 | 5 | c + d < 2 |

## 2.3	Possible Remedies for BS Harmonic and IMD

Note that Bands 21, 28 and 44 are not intended for use in the same geographical area as Band 12. Therefore, the focus here is on the harmonics and IMD falling into the BS receive band of Bands 12 and 17.

The possible remedies for harmonics and IMD caused by BS supporting non-contiguous CA in Band 12 into the BS receive band of Bands 12 and 17 are described below:

- Transmit Path Generated Harmonics and IMD: This type of harmonics and IMD is generated in the oscillator or as signals pass through the amplifier stages of the BS transmit path. The BS transmit path will generate harmonics and IMD which will appear at the BS equipment antenna connector (EAC) as spurious emissions. Spurious emissions appearing at the EAC and falling within the receive band of own or other BS will not be attenuated by the receive filter, and will pass into the receive path and desensitize the receiver. The 3rd order IMD products falling within the receive band of Bands 12 and 17 can be mitigated by transmit filtering, because the Band 12 BS transmit filter should significantly attenuate the spurious emission falling into the Band 12 receive band to avoid own BS receiver desensitization. And if the BS can avoid the transmit configurations shown in Table 4 above, then the 3rd IMD products will not fall into the own BS receive block of Band 12.

- Receiver Path Generated Harmonics and IMD: This type of harmonics and IMD is generated when the BS transmit signals appearing at the EAC pass through the receive filter and are applied to the front end of the receiver chain. This type of interference can be mitigated through a combination of receive filter attenuation within the transmit band and high 2nd and 3rd order Input Intercept Point (IIP2 and IIP3) of the receive path. The Band 12 or 17 BS receive filter should significantly attenuate the Band 12 BS transmission falling into the Band 12 or 17 receive band to protect own BS receiver. And the high IIP2 and IIP3 of the BS receive path would further reduce the potential interference to well below the Band 12 or 17 BS receiver noise floor. Also as discussed above, the 3rd IMD products will fall into the own BS receive block of Band 12 only if the BS use the transmit configurations shown in Table 4.

- Antenna Passive Intermodulation: Even passive systems such as antennas, duplexers or filter combiners can generate harmonics and IMD products via the non-linearities of dissimilar metals used in their construction, discontinuities in connectors or imperfections in the antenna system. This type of interference impacts all BS systems, but can be particularly troublesome when multiple BS operating in different frequency bands share a common antenna system. In the Band 12 case, the non-contiguous DL signals would be combined via a combiner and applied to a common antenna system, with the harmonics and IMD fall within the BS receiver band of Bands 12 and 17, and the 3rd order IMD products fall into the own BS receive block of Band 12 if the BS transmit configurations shown in Table 4 are used, and desensitize the BS receiver; in this case Band 12 BS transmitters should not share the same antenna with Band 12 or 17 BS receiver or the own Band 12 BS receiver, unless the antenna path meets very stringent 2nd and 3rd order PIM specification so that the PIM will not cause Band 12 or 17 BS or the own Band 12 BS receiver desensitization. The following alternatives can be used to avoid antenna sharing of Band 12 BS transmitters with Band 12 or 17 BS receiver or the own Band 12 BS receiver:

- Use separate antenna elements for the transmitter and receiver. This could be done with the higher frequency bands using state-of-the-art Adaptive Active Antenna (AAA). With the performance target of current antenna system and the extra coupling loss between the transmitter and receiver antenna elements, the harmonics and PIM products should be low enough in level that they would not cause BS receiver desensitization.

- Use separate transmitter chains and antenna elements for transmitting the Band 12 non-contiguous carriers. This technique could also be used for load balancing between the multiple transmit antennas of a BS.

# 3.	Conclusions

In this paper, we have investigated the impact of harmonics and IMD products caused by LTE Advanced BS supporting non-contiguous CA in Band 12 to the receiver of own or different BS. We have shown that the harmonics and IMD products caused by BS supporting non-contiguous CA in Band 12 may fall into the BS receive band of Bands 12 and 17, and even the own BS receive block of Band 12 if certain BS transmit configurations are used, hence BS receiver desensitization may be an issue.

However, with the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 12 or 17 BS receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 12 BS transmitters do not share the same antenna with Band 12 or 17 BS receiver, or the own Band 12 BS receiver if the aforementioned BS transmit configurations are used.

Therefore, in order to prevent BS receiver desensitization, it is recommended that Band 12 BS transmitters should not share the same antenna with Band 12 or 17 BS receiver, or the own Band 12 BS receiver if the aforementioned BS transmit configurations are used, unless the antenna path meets very stringent 2nd and 3rd order PIM specification so that the PIM will not cause Band 12 or 17, or the own Band 12 BS, receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

# 4.	Text Proposal

<Start of change>

# 1	Scope

The present document is a technical report for Inter-band Carrier Aggregation for 2DL/1UL under Rel-13 time frame. The purpose is to gather the relevant background information and studies in order to address Inter-band Carrier Aggregation requirements.

This TR covers relevant background information and studies in order to address Inter-band Carrier Aggregation (2DL/1UL) requirements for the Rel-13 band combinations in table 1-1.

Table 1-1: Release 13 inter-band carrier aggregation combinations (2DL/1UL)

| WI code | WI title | Class |
| --- | --- | --- |
| LTE_CA_B2_B28 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 28 | A1 |
| LTE_CA_B3_B31 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 31 | A2 |
| LTE_CA_B4_B28 | LTE Advanced inter-band Carrier Aggregation of Band 4 and Band 28 | A2 |
| LTE_CA_B7_B22 | LTE Advanced inter-band Carrier Aggregation of Band 7 and Band 22 | A5 |
| LTE_CA_B19_B28 | LTE Advanced inter-band Carrier Aggregation of Band 19 and Band 28 | A3 |
| LTE_CA_B20_B31 | LTE Advanced inter-band Carrier Aggregation of Band 20 and Band 31 | A3 |
| LTE_CA_B25_B26 | LTE Advanced inter-band Carrier Aggregation of Band 25 and Band 26 | A1 |
| LTE_CA_B5_B7_BWset | Additional bandwidth combination set for LTE Advanced inter-band Carrier Aggregation of Band 5 and Band 7 | A1 |
| LTE_CA_B4_B12_BWset2 | Additional bandwidth combination set 2 for LTE Advanced inter-band Carrier Aggregation of Band 4 and Band 12 | A2 |
| LTE_CA_B2_B12_BWset | Additional bandwidth combination set for LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 12 | A1 |

The scope of the report also covers the 2 DL fallback modes for the 3 DL Carrier Aggregation combinations with single UL configuration [2]. These 3 DL/1UL combinations (with 2 DL fallback modes) are listed in table 1-2. In addition, the report also covers the LTE TDD-FDD joint operation including Carrier Aggregation combinations [3], captured in Table 1-3. In addition, the report also covers the 2 DL fallback modes for the 4 DL Carrier Aggregation combinations with single UL configuration. These 4 DL/1UL combinations (with 2 DL fallback modes) are listed in table 1-4.

Table 1-2: Release 13 inter-band carrier aggregation combinations (3 DL/1UL)

| WI code | WI title |
| --- | --- |
| LTE_CA_B2_B5_B29 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 5 and Band 29 |
| LTE_CA_B38_B40_B40 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 38, Band 40 and Band 40 |
| LTE_CA_B3_B7_B28 | LTE Advanced 3 Band Carrier Aggregation (3DL) of Band 3, Band 7 and Band 28 |
| LTE_CA_B1_B8_B28 | LTE Advanced 3 band Carrier Aggregation (3DL/1UL) of Band 1, Band 8 and Band 28 |
| LTE_CA_B3_B20_B32 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 3, Band 20 and Band 32 |

Table 1-3: Release 13 inter-band carrier aggregation combinations (LTE TDD-FDD joint operation)

| WI code | WI title | Class |
| --- | --- | --- |
| LTE_CA_B1_B40 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 40 | - |
| LTE_CA_B3_B40 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 40 | A3 |
| LTE_CA_B21_B42 | LTE Advanced inter-band Carrier Aggregation of Band 21 and Band 42 | A5 |
| LTE_CA_B7_B40_B40 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 7, Band 40 and Band 40 | - |
| LTE_CA_B5_B40 | LTE Advanced inter-band Carrier Aggregation of Band 5 and Band 40 | - |
| LTE_CA_B20_B40 | LTE Advanced inter-band Carrier Aggregation of Band 20 and Band 40 | - |
| LTE_CA_B3_B38 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 38 | - |
| LTE_CA_B3_B41 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 41 | - |
| LTE_CA_28A-40C | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 28, Band 40 and Band 40 | - |
| LTE_CA_28A-41C | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 28, Band 41 and Band 41 | - |
| LTE_CA_28A-42C | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 28, Band 42 and Band 42 | - |
| LTE_CA_B7_B42_B42 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 7, Band 42 and Band 42 | - |
| LTE_CA_B20_B42_B42 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 20, Band 42 and Band 42 | - |

Table 1-4: Release 13 inter-band carrier aggregation combinations (4DL/1UL)

| WI code | WI title |
| --- | --- |
| LTE_CA_B4_B5_B12_B12 | LTE Advanced 4 Band Carrier Aggregation (4DL) of Band 4, Band 5, Band 12 and Band 12 |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

<New section>

### 6.3.X	LTE-Advanced non-contiguous Carrier Aggregation in Band 12 (1 UL)

Table 6.3.X-1: Non-contiguous CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_12-12 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |

#### 6.3.X.1		List of specific combination issues

##### 6.3.X.1.1	Channel bandwidths per operating band for CA

Table 6.3.X.1.1-1: Supported E-UTRA bandwidths per CA configuration for non-contiguous CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Band or E-UTRA CA configuration | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_12A-12A | 12 |  |  | Yes |  |  |  | 10 | 0 |
|  | 12 |  |  | Yes |  |  |  |  |  |

##### 6.3.X.1.2	Co-existence studies for 2DL/1UL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of non-contiguous Band 12 DL carriers can be calculated as shown in Table 6.3.X.1.2-1 below:

Table 6.3.X.1.2-1: Band 12 DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 729 | 746 |
| 2nd harmonics frequency limits (MHz) | 1458 | 1492 |
| 3rd harmonics frequency limits (MHz) | 2187 | 2238 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -17 | 17 |
| 3rd order IMD products | (2*f-low – f-high) | (2*f-high – f-low) |
| IMD frequency limits (MHz) | 712 | 763 |

It can be seen from Table 6.3.X.1.2-1 that the 2nd harmonics of BS transmitting in Band 12 may fall into the BS receive band of Band 21, while the 3rd IMD products caused by BS supporting non-contiguous CA in Band 12 may fall into the BS receive band of Bands 12, 17, 28 and 44.

Assume that the 2 non-contiguous CC transmitted by the Band 12 BS are a and b MHz channel bandwidth and c and d MHz from the edges of Band 12 DL frequency band as shown in Figure 6.3.X.1.2-1 below:

f2-high

f2-low

f1-high

f1-low

1st CC

2nd CC

729

746

c MHz

d MHz

a MHz

b MHz

MHz

Figure 6.3.X.1.2-1: Non-contiguous CC transmitted by the Band 12 BS

And the corresponding BS receive blocks are as shown in Figure 6.3.X.1.2-2 below:

f1-low

f1-high

f2-low

f2-high

1st CC

2nd CC

699

716

c MHz

d MHz

a MHz

b MHz

MHz

Figure 6.3.X.1.2-2: Non-contiguous CC received by the Band 12 BS

Then the 3rd order IMD products caused in the BS by transmitting of the 2 CC can be calculated as shown in Table 6.3.X.1.2-2 below:

Table 6.3.X.1.2-2: Band 12 BS DL 3rd order IMD products

| 3rd order IMD products | IMD frequency limits (MHz) |
| --- | --- |
| (2*f1-low – f2-high) | 712 + 2*c + d |
| (2*f1-high – f2-low) | 712 + 2*c + d + 2*a + b |
| (2*f2-low – f1-high) | 763 – 2*d – c – 2*b – a |
| (2*f2-high – f1-low) | 763 – 2*d – c |
| (f1-low – f2-high + f2-low) | 729 + c – b |
| (f1-high + f2-high – f2-low) | 729 + c + b + a |
| (f2-low – f1-high + f1-low) | 746 – d – a – b |
| (f2-high + f1-high – f1-low) | 746 – d + a |

Comparing the IMD frequency limits in Table 6.3.X.1.2-2 with the BS receive blocks in Figure 6.3.X.1.2-2, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of the 2 CC will only fall into the BS own receive blocks of Band 12 under one of the transmit configurations shown in Table 6.3.X.1.2-3 below (note that a and b can only be 5 as stated in the WIDS):

Table 6.3.X.1.2-3: Band 12 BS transmit configurations with 3rd IMD within Band 12 BS own receive blocks

| 1st CC channel bandwidth (a MHz) | 2nd CC channel bandwidth (b MHz) | 1st and 2nd CC position (c and d MHz) |
| --- | --- | --- |
| 5 | 5 | c + d < 2 |

Note that Bands 21, 28 and 44 are not intended for use in the same geographical area as Band 12. Therefore, the focus here is on the harmonics and IMD falling into the BS receive band of Bands 12 and 17.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 12 or 17 BS receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 12 BS transmitters do not share the same antenna with Band 12 or 17 BS receiver, or the own Band 12 BS receiver if the aforementioned BS transmit configurations are used.

Therefore, in order to prevent BS receiver desensitization, it is recommended that Band 12 BS transmitters should not share the same antenna with Band 12 or 17 BS receiver, or the own Band 12 BS receiver if the aforementioned BS transmit configurations are used, unless the antenna path meets very stringent 2nd and 3rd order PIM specification so that the PIM will not cause Band 12 or 17, or the own Band 12 BS, receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of change>

# References

[1]	RP-151521, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 4, Band 5, Band 12 and Band 12”, U.S. Cellular.

[2]	R4-156121, “TR 36.852-13: LTE-A Rel-13 2DL Inter-band Carrier Aggregation v0.6.0”, Ericsson.

[3]	3GPP TS 36.104 v13.1.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.