3GPP TSG-RAN WG4 #76bis	R4-156408

Sophia-Antipolis, France, 12th Oct– 16th Oct 2015

Source: 	Ericsson

Title:	Discussion on non-TM10 setup for CRS-IM WI

Agenda Item:	7.5.1

Document for:	Discussion

# Introduction

In [1], WF on CRS-IM PDSCH demodulation (non-TM10) was agreed. Based on the agreements, the interference level of aggressor cells and the resource utilization for each aggressor cells were agreed. But there are still some open issues for PDSCH demodulation. In this paper, we share our view for these open issues.

# General setup for PDSCH CRS-IC test

## MCS for serving cell

In [5], four MCS options are listed as candidates for the serving cell. The link level simulation results are shown in Appendix C, the target SNR@70% throughput and the gain of 2-cell CRS-IC over 0-cell CRS-IC gain is tabulated in Table 1. From Table 1, we can see that the gain for CRS-IC is very similar for different MCS options. It is about 2 dB.  Based on Error! Reference source not found., we have the following observations:

- For TM2, both MCS=16 and MCS=18 are suitable regarding the target SNR is very close to the interference profile derived from system level evaluation.

- For TM9, MCS = 14 or MCS=16 is more suitable regarding the target SNR is very close to the interference profile derived from system level evaluation.

- For TM4, both MCS=16 and MCS=18 are suitable regarding the target SNR is very close to the interference profile derived from system level evaluation.

Table 1: The target SNR@70% throughput and the 2-cell CRS-IC gain

| TM | MCS | 2-Cell CRS-IC | 1-Cell CRS-IC | No CRS-IC | gain |
| --- | --- | --- | --- | --- | --- |
| TM9 | MCS9 | 5.11 | 5.82 | 8.55 | 3.44 |
|  | MCS14 | 8.57 | 9.22 | 11.31 | 2.74 |
|  | MCS16 | 10.31 | 11.02 | 13.35 | 3.04 |
|  | MCS18 | 11.51 | 11.94 | 13.90 | 2.39 |
| TM2 | MCS9 | 3.41 | 3.92 | 5.83 | 2.41 |
|  | MCS14 | 7.06 | 7.48 | 9.03 | 1.97 |
|  | MCS16 | 8.43 | 9.01 | 10.53 | 2.10 |
|  | MCS18 | 9.79 | 10.30 | 11.75 | 1.96 |
| TM4 | MCS9 | 3.87 | 4.63 | 6.62 | 2.76 |
|  | MCS14 | 7.43 | 7.86 | 9.58 | 2.15 |
|  | MCS16 | 9.04 | 9.53 | 11.29 | 2.25 |
|  | MCS18 | 10.23 | 10.85 | 12.34 | 2.12 |

Considering exploring all the modulation schemes, in Table 2, MCS for different transmission mode is tabulated.

Table 2: MCS for different transmission mode

| TM | MCS |
| --- | --- |
| TM2 | MCS=16 |
| TM9 | MCS=14 |
| TM4 | MCS=18 |

Proposal  1: Taken Table 2 as reference for the MCS decision

## Discussion on test case list

In [1]Table 3, for PDSCH, TM2, TM3, TM4, TM9 and TM10 are agreed to be considered for test cases. Considering the test case number and test case purpose, we can have the following test cases and its corresponding additional test purposes as shown in Table 3.

Table 3: Test case list

| Test case | TM | Test purposes |
| --- | --- | --- |
| 1 | TM2 | - To achieve MMSE-IRC+CRS-IC gain - To cover CRS-based open loop transmission scheme |
| 2 | TM3 | - To achieve robustness test |
| 3 | TM4 | - To achieve MMSE-IRC+CRS-IC gain - To cover close loop MIMO |
| 4 | TM9 | - To achieve MMSE-IRC+CRS-IC gain - To cover DMRS based scheme |
| others | TM10 | TBD |

This is the minimum subset for the test cases. If test cases are further reduced, some test purpose may not be covered.

Proposal  2: Adopt Table 3 as the test case list.

# Further discussion on the performance requirements for the gain test

For the performance requirements, there are several options:

- Option 1: Companies can provide both 1-cell CRS-IC and 2-cell CRS-IC results for alignment and decided the requirements based on the aligned results

- Option 2: Defined performance requirements based on 1-cell CRS-IC

From Rel-11 FeICIC, 2-cell CRS-IC is assumed for the performance requirements. It is mandatory for all Rel-11 UEs. Rel-13 UE shall inherit these implementations and even enhance it. In CRS-IM, it is nature to consider 2-cell CRS-IC.

For option 2, the main concern is UE may need to perform synchronization signal cancelation and/or common channel interference cancelation to acquire the neighbor cell. Synchronization signal/common channel interference cancellation is not a new feature, since synchronization signal and common channel interference handling is mandatory in Rel-11 along with CRS-IC.

In order to save computation power, Rel-11 synchronization and common channel interference handling may be simplified in homogeneous network scenarios. However, since the serving cell’s synchronization signal and common channel is pre-known. The complexity to cancel serving cell’s synchronization signal and common channel is trivial. Assuming the serving cell’s synchronization and common channel is cancelable by nature, the side condition for the weaker aggressor cells are:

EsIot=10log10104.6101010.4510+1=-6.22 dB

This level is reliable to detect the weaker cell even without complex synchronization signal and common channel cancellation.

Another possible way to relax the dependence on the cell acquisition is to revise the interference level for the weaker cell. Although it is a bit artificial, it can exclude bad UE implementation and guarantee all the UEs can achieve good performance.

Proposal  3: The performance requirements for the gain test are based on 2-Cell CRS-IC. To relax the dependence on the cell acquisition, it is possible to revise the interference level for the weaker cell.

# Robustness test

## Discussion on the necessity of robust test

It is well known that any interference cancelation schemes may get performance loss if the interference cancelation is not properly handled. Hence, in FeICIC and NAIC WI, robustness tests are introduced to avoid the performance loss due to improper interference cancelation. In Appendix B, simulation results based on direct CRS interference cancellation are given to illustrate the potential problems for the CRS-IC.

In [5], it is believed the robustness test in the FeICIC can cover the robustness test for CRS-IM. It is not true. The differences lie in the following aspects:

#### Interference characteristics are different

To illustrate the CRS-IC problem, one simple receiving model can be given by

r=H1S1+β2H2S2+β3H3S3+n

where H1,H2 and H3 is the channel model for the serving cell, the first aggressor cell and the second aggressor cell, respectively. β2,β3 are the PDSCH presence indicator for the aggressor cell. If the PDSCH is present, βkk=2 or k=3=1, otherwise, βkk=2 or k=3=0; S1, S2 and S3 are the transmitted signal from the serving cell, the first aggressor cell, and the second aggressor cell. n is the while noise with the variance σn2.

For serving cell channel estimation, the SINR is by

SINR1=H12Es1β2H22Es2+β3H32Es3+σn2

And for the channel estimation of the first aggressor cell and the second aggressor cell, the SINRk(k=2,3) is given by

SINR2=β2H22Es2H12Es1+β3H32Es3+σn2

SINR3=β3H32Es3H12Es1+β2H22Es2+σn2

When serving cell SNR is very high, SINR2 and SINR3 will be very low, the quality of CRS estimation becomes very worst. When the CRS is removed directly, the residual interference may even larger than CRS interference itself. Hence, for high serving cell SNR, it may have performance loss.

In FeICIC scenarios, β2 and β3 are always zero. Hence, one aggressor cell signal transmission will not impact on the CRS estimation of other aggressor cells. However, in homogeneous scenarios, not only the serving cell signal strength impact on the CRS estimation quality, but also one aggressor cell’s signal transmission may impact other aggressor cell’s CRS estimation, since

β2=1,  p=0.20,  &p=0.8

β3=1,  p=0.20,  &p=0.8

Here, the resource utilization is assumed to be 0.2 for all the aggressor cells. In Table 4, we summarize the interference condition for CRS cancelation.

Table 4: Interference condition for CRS cancelation

| TP2 TP3 | CRS only | CRS + PDSCH |
| --- | --- | --- |
| CRS only | Case 1 | Case 2 |
| CRS+PDSCH | Case 3 | Case 4 |

In FeICIC, only Case 1 is verified. Case 2, case 3 and Case 4 are not verified, since only the performance for ABS protected subframes are defined in FeICIC WI. However, in Homogeneous network, it is much more important to have robustness test. In homogenous network, in most cases, the aggressor signal is much weaker than the serving cell signal. As above analysis, it is really challenge to achieve pretty good performance when the interference is weaker. Secondly, the load in aggressor cell is quite dynamic. In some time interval, it may be full load, and in other time interval, it may be no load. It is different from FeICIC, where ABS can be always assumed. For this dynamic interference condition, if the CRS interference is not properly handled, it is easier to lead some performance loss, which is not desirable from network side.

Observation 1:  Robustness test in FeICIC only covers partial interference scenarios of Homogeneous network, it is valuable to introduce robustness test in homogenous scenarios.

#### The availability of Interference information is different

In Rel-11, ABS information is known to UE. UE can utilize this information for performance enhancement. For example, with the ABS information, it can have the following potential enhancement:

- AGC enhancement

  - Since the dynamic range of ABS protected subframes and non-ABS protected subframes is large, with this enhancement, the performance can be improved.

- Channel estimation/noise estimation enhancement

- …

But in Rel-13, all these enhancements depended on the availability of ABS information may be not valid any more. It may lead to new problem for the robustness.

Observation 2: The enhancement in Rel-11 may be not valid any more for Rel-13 if the enhancements implemented in Rel-11 depend on the availability of ABS information.  It may lead to new problem for the robustness.

Based on Observation 1 and observation 2, it is really necessary to have robust test in Rel-13 CRS-IM.

Proposal  4:  Introduce TM3 as robustness test for CRS-IM WI.

## Setup for the robustness test

In [1], for the robustness test, the group had the following agreements:

- Interference profile for the robustness test

  - The 1st set

    - [INR1,INR2] = [0.19, -1.62]

- Resource utilization on the aggressor cells for robustness test: 50%

- Transmission mode: TM3

Other link level simulation assumptions are shown in Appendix A. For different MCS, the simulation results are shown in Figure 1~Figure 4.

Figure 1: MCS=9 for TM3

Figure 2: MCS=14 for TM3 test

Figure 3: MCS=16 for TM3 test

Figure 4: MCS=18 for TM3 test

The required SNR for different MCS are tabulated in Table 5. From Table 5, the required SNR for the serving cell of all the MCSs are much higher than the aggressor cells’ interference level. Under this condition, it increases the challenge to get accurate CRS estimation. Hence, any one of them is proper to set as the MCS.

Table 5: Required target SNR for different MCS in robustness test

| INR1 | INR2 | Mod | 0-cell CRS-IC | 1-cell CRS-IC | 2-Cell CRS-IC |
| --- | --- | --- | --- | --- | --- |
| 0.19 | -1.62 | MCS9 | 8.99 | 8.87 | 8.79 |
| 0.19 | -1.62 | MCS14 | 13.19 | 13.13 | 13.11 |
| 0.19 | -1.62 | MCS16 | 15.16 | 15.14 | 15.14 |
| 0.19 | -1.62 | MCS18 | 16.53 | 16.55 | 16.55 |

Proposal 5:  Any one of MCS9, MCS14, MCS16 and MCS18 can be set as the MCS for the robustness test.

# Conclusion

In this paper, we provide our analysis for the open issues for non-TM10 case in CRS-IM, we have the following proposals:

Proposal  1: Taken Table 2 as reference for the MCS decision

Proposal 2: Adopt Table 3 as the test case list.

Observation 1:  Robustness test in FeICIC only covers partial interference scenarios of Homogeneous network, it is valuable to introduce robustness test in homogenous scenarios.

Observation 2: The enhancement in Rel-11 may be not valid any more for Rel-13 if the enhancements implemented in Rel-11 depend on the availability of ABS information.  It may lead to new problem for the robustness.

Proposal 3: The performance requirements for the gain test are based on 2-Cell CRS-IC. To relax the dependence on the cell acquisition, it is possible to revise the interference level for the weaker cell.

For the robustness test, we have the following observations:

Observation 1:  Robustness test in FeICIC only covers partial interference scenarios of Homogeneous network, it is valuable to introduce robustness test in homogenous scenarios.

Observation 2: The enhancement in Rel-11 may be not valid any more for Rel-13 if the enhancements implemented in Rel-11 depend on the availability of ABS information.  It may lead to new problem for the robustness.

Based on these observations, we propose to have:

Proposal  6:  Introduce TM3 as robustness test for CRS-IM WI.

Proposal 7:  Any one of MCS9, MCS14, MCS16 and MCS18 can be set as the MCS for the robustness test.

# Appendix A: Test setup for TM3 (robustness test)

# Appendix B: Potential performance degradation for CRS-IC

As one example, in Figure 5, it shows the throughput performance of CRS-IC cancelling 0/1/2 aggressor cells. In the simulation, two aggressor cells are considered; both of them are with non-colliding CRS with the victim cells. The ratio of interference to noise (INR) is 9 dB for the 1st strongest interference, and 1 dB for the second strongest interference. Here, the noise includes all the interference and noise exclude the 1st strongest interference and second strongest interference. Two antenna ports and 10 MHz are assumed in the simulation, and the cell ID and number of ports are pre-known. In the CRS-IC receiver, CRS interference is assumed to be cancelled directly after channel estimation.  For simplicity, only CRS is transmitted from the aggressor cell.

From the simulation results, in low SNR, the CRS-IC can provide much benefit for PDSCH. However, in medium or high serving cell SNR (Signal over noise ratio) range, CRS-IC brings large performance loss. For high SNR range, serving cell signal is far strongest than aggressor cell. Since non-colliding case is assumed in this case, aggressor cell’s CRS is colliding with the serving cell’s data. For aggressor cell’s channel estimation, serving cell’s data are interference. Hence, strongest serving cell signal will deteriorate the aggressor cell’s channel estimation. As a result, CRS-IC will bring performance loss in high SNR region if CRS-IC is not properly implemented. To avoid this problem, FeICIC define the robustness test to avoid such problem. This problem would become more challenge for homogeneous scenarios.

Figure 5: Throughput comparison with CRS-IC cancelling 0/1/2 aggressor cells for INR=9 dB and INR=1 dB

Figure 6: Throughput comparison with CRS-IC cancelling 0/1/2 aggressor cells INR=11 dB and INR=-2 dB

# Appendix C

# Performance for TM9

Figure 7: MCS=9

Figure 8: MCS=14

Figure 9: MCS=16

Figure 10: MCS=18

# Performance for TM2

Figure 11: MCS=9

Figure 12: MCS=14

Figure 13: MCS=16

Figure 14: MCS=18

# Performance for TM4

Figure 15: MCS=9

Figure 16: MCS=14

Figure 17: MCS=16

Figure 18: MCS=18

# Reference

- R4-152399, WF on CRS-IM PDSCH demodulation non-TM10, Ericsson, Apr. 2015.

- R4-152352, Discussion on the control channel test for CRS-IM, Ericsson, Apr. 2015.

- R4-151581, CRS-IM demodulation test framework for non-TM10 TMs, Apr. 2015

- R4-155191, WF on CRS-IM demodulation v10 (clean), Ericsson, ZTE, August 2015

- R4-154098, Test configuration for CRS-IM with non-TM10 TMs, Qualcomm Incorporated, Aug. 2015