3GPP TSG-RAN WG4 #76bis	R4-156409

Sophia-Antipolis, France, 12th Oct– 16th Oct 2015

Source: 	Ericsson

Title:	Discussion on TM10 setup for CRS-IM WI

Agenda Item:	7.5.1

Document for:	Discussion

# Introduction

In RAN4#76 meeting, interference model was discussed for TM10 and some open issues were identified for TM10 test. These open issues were captured in [1] [3]. In this contribution, we share our view for these open issues based on some preliminary link level simulation.

# Discussion

## System model

According to the agreement in [1], up to 2-cell generic CRS-IC is adopted as the baseline assumption for introduction of TM10 CRS-IM test case (s). For TM10, we can configure three points in the test. The first point (TP1) is the serving cell; the second point (TP2) is the transmission point which can transmit PDSCH, and the third point (TP3) is the aggressor cell, as shown in Figure 1. The PDSCH transmission can dynamically switch between TP1 and TP2. Hence, the cancelable CRS may come from TP2 & TP3 or TP1&TP3 according to the transmission point of PDSCH.

Interference the UE can cancel

Strongest interferes are canceled only, depending on the conditions)

PDSCH (it may be transmitted from TP1 and TP2)

CRS

PDCCH

Serving cell (TP1)

TP2

TP3

Figure 1: The system setup for CRS-IM for TM10

## Test purpose

Compared with TM9, there are three main new behaviors for TM10 regarding CRS-IC. The first one is the serving cell CRS may serve as interference and it should be cancelled. The second one is the source of aggressor cell dynamically switches between serving cell and cooperating cell. The third one is the CRS assistant information for the aggressor cell within the COMP cooperating set is different from the assistant information for the aggressor cell outside the COMP cooperating set. Both of these three  new behaviors are needed to be verified. In addition to the capability to cancel the CRS interference outside the cooperating set, the test purpose of TM10 test shall at least cover:

- The CRS from the aggressor cell outside the cooperating set shall be cancelled

- The dynamic switching CRS interference within the cooperation set shall be cancelled.

  - When the CRS interference coming from serving cell, the serving cell CRS shall be cancelled.

  - When the CRS interference coming from non-serving cell which is within the cooperation set, the CRS interference shall be cancelled.

- Check the UE behavior for different CRS-assistance information

Proposal  1: The test purpose of TM10 test shall at least cover:

- The CRS from the aggressor cell  outside the cooperating set shall be cancelled

- The dynamic switching CRS interference within the cooperation set shall be cancelled.

  - When the CRS interference coming from serving cell, the serving cell CRS shall be cancelled.

  - When the CRS interference coming from non-serving cell which is within the cooperation set, the CRS interference shall be cancelled.

- Check the UE behavior for different CRS-assistance information

## Key setup parameters for TM10 (one CSI process configuration)

In [3], for one CSI process UE, RAN4 group had the following agreements:

- For one CSI-process capability UE

  - PDSCH scheduling: PDSCH is static scheduled from non-serving TP within CoMP set

- Interference modelling for the TP within the CoMP set

  - Blanking

- Interference modelling for the TP outside the CoMP set

  - Partial load with lower RU 20%

- Transmission mode for the TPs

  - TM10/TM10/TM9

- Interference profiles

  - For the aggressor cell within the CoMP set, the interference level is [10.45] dB

  - For the aggressor cell  outside the CoMP set, the interference level is:

    - Option 1: [8.45] dB

    - Option 2: [6.45] dB

The remaining open issues for one CSI process UE are the interference profile for the aggressor cell outside the COMP set and the MCS selection for the test. Link level simulation results are provided in Appendix A. The target required SNR for 2-cell CRS-IC, 1-cell CRS-IC and 0-Cell CRS-IC are tabulated in Table 1. From Table 1, we can see that both 6.45 and 8.45 dB for the second aggressor cell can be used for differentiate 2-Cell CRS-IC with 0-Cell CRS-IC. In case 6.45 dB is used, for some MCS, for example MCS=18, the difference between 1-Cell and 2-Cell CRS-IC only with 1.09 dB. It may be risk to use this value to differentiate 2-Cell CRS-IC and 1-Cell CRS-IC. Hence, 8.45 dB is slightly preferred. Considering the target serving cell SNR shall be about 9 dB and the different between 2-Cell CRS-IC and 1-Cell CRS-IC shall be large enough, MCS14 or MCS16 is more appropriate. MCS=18 is better not to be used.

Table 1: The target Required SNR for different CRS-IC receiver

| INR1 | INR2 | Mod | 1-Cell CRS-IC | 2-Cell CRS-IC | No-cell CRS-IC | 2-Cell/0-Cell | 2-cell/1-cell |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10.45 | 8.45 | MCS9 | 6.48 | 3.77 | 8.65 | 4.88 | 2.71 |
| 10.45 | 8.45 | MCS14 | 9.37 | 7.41 | 11.21 | 3.79 | 1.95 |
| 10.45 | 8.45 | MCS16 | 11.37 | 9.25 | 13.21 | 3.96 | 2.12 |
| 10.45 | 8.45 | MCS18 | 12.02 | 10.43 | 13.76 | 3.33 | 1.60 |
| 10.45 | 6.45 | MCS9 | 5.43 | 3.65 | 8.12 | 4.48 | 1.78 |
| 10.45 | 6.45 | MCS14 | 8.55 | 7.28 | 10.75 | 3.47 | 1.27 |
| 10.45 | 6.45 | MCS16 | 10.49 | 9.10 | 12.81 | 3.70 | 1.38 |
| 10.45 | 6.45 | MCS18 | 11.36 | 10.27 | 13.29 | 3.02 | 1.09 |

Proposal  2:  8.45 dB is slightly preferred to be taken as the interference level for the aggressor cell outside the CoMP set

Proposal  3: MCS=14 or MCS=16 can be used for the MCS setup for the Tm10 in the one CSI process test.

## Key setup parameters for TM10 (multiple-CSI-process configuration)

### PDSCH model in the test

For multiple CSI-processes UE, there are two options for the test setup:

- Option 1: PDSCH is static scheduled from non-serving TP within CoMP set

- Option 2: DPS, i.e., TP for PDSCH transmission is dynamically changed between two TPs within CoMP set

If option 1 is used, it can only verify the following UE behavior:

- It can verify the UE behavior to cancel the CRS from the aggressor cell outside the cooperating set

- It can verify the UE behavior to cancel the CRS from serving cell

It cannot verify the following UE behavior:

- It CANNOT verify the UE behavior to cancel the CRS interference from non-serving cell within the cooperation set

- It CANNOT verify UE behavior for different CRS-assistance information

This behavior is not similar like the behavior to cancel the CRS from the serving cell. For serving cell CRS, UE has full information. But for the non-serving cell within the cooperation set, UE can only acquire partial CRS information from QCL information.

This behavior is not similar like the behavior to cancel the CRS from the aggressor cell outside the cooperating set either. On one hand, the CRS assistant information is different; one relies on the CRS-AssistanceInfo-r11 signaling, one relies on QCL information. Further, the CRS information from the aggressor cell outside the cooperating set is semi-static, while the QCL information is dynamic. Further, the CRS information in the CRS-AssistanceInfo-r11 is full information and the CRS information in the QCL is partial information.

If option 1 is used, it will lead to the test for TM10 meaningless. Hence, we would like to have option 2 for the multiple CSI-processes setup.

Proposal 4: For multiple-CSI-process UEs, DPS is adopted for the demodulation test to verify CRS-IC performance.

For DPS setup, section 8.3.1.3.2 of 36.101 can be reused with some modification. In this test, PDSCH is transmitted from TP1 with 30% of time and from TP2 with 70% of time. When the PDSCH is transmitted from TP1, the CRS from TP2 and TP3 can be cancelled. When the PDSCH is transmitted from TP2, the CRS from TP1 and TP3 can be cancelled. The details are shown in Table 2.

Table 2: Basic setup for multiple CSI-process capable UE

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | PDSCH | Blanked |
| PQI set 3 | CSI-RS 1 | ZP CSI-RS 1 | Blanked | PDSCH |
| Note: Probability of occurrence of PDSCH transmission is 30% and 70% for TP1 and TP2, respectively. |  |  |  |  |

Proposal  5: For different CSI-process capable UE, Table 2 can be referred.

### Time offset and frequency offset issue

In TM10 test case defined in 36.101, two time offsets are defined for TM10, one is with 2 us and one is with -0.5 us. In FeICIC, the time offsets are -1 us and +3 us. For the transmission points within the COMP cooperating set, tight timing offset is reasonable, since in general, the transmission points within the COMP cooperating set are connected with good backhaul or even co-site. However, for the transmission point outside the COMP cooperating set, tight timing offset is not reasonable. For this case, FeICIC timing offset is more reasonable. Considering these two aspects, we suggest the timing offset of TP2 relative to TP1 is -0.5 us, and the timing offset of TP3 relative to TP1 is +3 us.

For the frequency offset, it has not too much impact on the performance. However, with some frequency offset, it can be used at least verify UE have implement frequency compensation. Hence, we propose to reuse FeICIC frequency offset definition.

Proposal  6: The timing offset between transmission points within COMP cooperation set is -0.5 us, the time offset between the aggressor cells outside the COMP cooperation set and the serving cell is 3 us. The frequency offset of TP2 and TP3 related to TP1 is [-100 300] Hz.

### Transmission power setup

In [1] and [2], the transmission power issue is discussed. The main issues are on how to set the transmission power for TP2 and TP1. For TP3, fixed transmission power can be set.  There are some alternatives for TP2 and TP1 transmission power setup:

Alternative 1: Use different transmission power for TP2 and TP1 (PTP1PTP2≠1)

Alternative 2: Use the same transmission power for TP2 and TP1 (PTP1PTP2=1).

For alternative 1, if DPS is used, different MCS shall be used for TP2 and TP1 PDSCH transmission. It is relatively difficult to define the target SNR, since TP2 and TP1 throughput are dependent. Another consideration for alternative 1 is use fixed transmission point for PDSCH transmission and only one MCS is used. In this case, only serving cell CRS and the CRS from the cell outside the cooperating set are cancelled. The CRS from the transmission point within the cooperating set is not cancelled. Further, the CRS aggressor cell is static and not dynamically changed. Thus, only partial purposes are covered. Hence, it is not preferable.

For alternative 2, the same transmission power for TP2 and TP1 is used. If proper MCS is defined, we can guarantee the target SNR is high enough. With high enough SNR, the aggressor cell will be stronger enough. Thus, the CRS-IC gain can be observed. Therefore, 2-Cell CRS-IC functionality can be verified properly. One example is shown in Figure 2. The parameters are listed in Table 3.

From Figure 2, we can see that the target SNR is about 9.18 dB for 2-cell CRS-IC receiver. If CRS-IC is not enabled, 19.56 dB is required to achieve 70% throughput. If only one-cell CRS-IC is enabled, at least 11.39 dB are required to achieve 70% throughput. Hence, in this case, 2-cell generic CRS-IC must be enabled to meet the requirements.

Table 3: Parameters for the link level simulation

|  | TP1 | TP2 | TP3 |
| --- | --- | --- | --- |
| Probability of occurrence of PDSCH transmission | 30% | 70% | 0 |
| Signal strength | PTP1=PTP2 |  | INR2=8.45 dB |
| Resource Utilization | Blanking if desired PDSCH is not transmitted in this point | Blanking if desired PDSCH is not transmitted in this point | 20% |
| MCS | MCS=16 |  | TM9, 64QAM, |
| Rank | Rank-1 |  | Rank-1 and rank-2 with probability [0.2, 0.8] |
| CRS handling | No CRS-IC: CRSs are not cancelled |  |  |
|  | 2-cell CRS-IC: CRS from TP3 and TP1 (or TP2) are cancelled |  |  |
|  | 1-cell CRS-IC: CRS from TP1 (or TP2) is cancelled |  |  |
| Cell id | 0 | 11 | 106 |

Figure 2: CRS-IC performance for MCS=16 with  PTP1=PTP2 and INR2=8.45 dB

Observation 1: It is feasible for the TM10 test for the multiple-CSI-process UE when the cell within the COMP cooperating set is blanking  and the same transmission power for TP2 and TP1 (PTP1PTP2=1)  is set.

For other MCS and other interference level, the simulation results are shown in Appendix B, The target SNR are tabulated in Table 4.

Table 4: The tabulated results for TM10 (Multiple CSI-Process)

| INR2 (dB) | MCS | No CRS-IC | IC the 3rd cell | IC the 2nd cell | 2-Cell CRS-IC | 2-cell/0-cell | 2-cell/min(1-cell) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8.45 | MCS9 | 7.50 | 4.23 | 6.40 | 3.60 | 3.89 | 0.63 |
| 8.45 | MCS14 | 11.80 | 9.09 | 9.33 | 7.28 | 4.52 | 1.81 |
| 8.45 | MCS16 | 19.56 | 15.44 | 11.39 | 9.18 | 10.38 | 2.22 |
| 8.45 | MCS18 | 19.05 | 15.46 | 12.14 | 10.45 | 8.60 | 1.68 |
| 6.45 | MCS9 | 6.50 | 4.36 | 5.32 | 3.46 | 3.04 | 0.89 |
| 6.45 | MCS14 | 11.05 | 9.30 | 8.45 | 7.13 | 3.92 | 1.32 |
| 6.45 | MCS16 | 18.41 | 15.65 | 10.48 | 9.03 | 9.39 | 1.45 |
| 6.45 | MCS18 | 18.00 | 15.77 | 11.41 | 10.21 | 7.78 | 1.20 |

From Table 4, we can see that when MCS=9, the gain of 2-cell CRS-IC over 0-cell CRS-IC is about 3 dB. However, the gain of 2-cell CRS-IC over 1-Cell CRS-IC is only 0.63 dB. The difference is too small to differentiate different receiver. When the MCS=16, the gain of 2-cell CRS-IC over 0-cell CRS-IC is about 10 dB and the gain of 2-cell CRS-IC over 1-cell CRS-IC is about 2.22 dB. The gap is enough to differentiate 2-Cell CRS-IC and 1-Cell CRS-IC. Hence, to meet the purpose, higher MCS is more preferable.

Observation 2: Higher MCS is much easier to differentiate 2-cell CRS-IC, 1-cell CRS-IC and 0-cell CRS-IC

From Table 4, we also observed that when the aggressor cell outside the cooperating set is 6.45 dB, the gain of 2-cell CRS-IC over 1-cell CRS-IC is about 1 dB for all MCS, while when the aggressor cell outside the cooperating set is 8.45 dB, the gain of 2-cell CRS-IC over 1-cell CRS-IC is more than 1.5 dB except lower MCS=9.  In order to force UE to have 2-cell generic CRS-IC, 8.45 dB or higher interference level is more appropriate.

Observation 3: Higher interference level is preferable for the aggressor cell outside the cooperating set, i.e, 8.45 dB is set the for the interference level of the aggressor cell outside the cooperating set.

Based on the observations 1~3, we propose:

Proposal 7: MCS=16 is selected as the PDSCH MCS and the same transmission power is configured for TP1 and TP2, INR2 is 8.45 dB for TP3.

# Conclusion

Proposal  1: The test purpose of TM10 test shall at least cover:

- The CRS from the aggressor cell  outside the cooperating set shall be cancelled

- The dynamic switching CRS interference within the cooperation set shall be cancelled.

  - When the CRS interference coming from serving cell, the serving cell CRS shall be cancelled.

  - When the CRS interference coming from non-serving cell which is within the cooperation set, the CRS interference shall be cancelled.

- Check the UE behavior for different CRS-assistance information

Proposal  2: 8.45 dB is slightly preferred to be taken as the interference level for the aggressor cell outside the CoMP set

Proposal  3: MCS=14 or MCS=16 can be used for the MCS setup for the Tm10 in the one CSI process test.

Proposal  4: For multiple-CSI-process UEs, DPS is adopted for the demodulation test to verify CRS-IC performance.

Proposal  5: For different CSI-process capable UE, Table 2 can be referred.

Proposal  6: The timing offset between transmission points within COMP cooperation set is -0.5 us, the time offset between the aggressor cells outside the COMP cooperation set and the serving cell is 3 us. The frequency offset of TP2 and TP3 related to TP1 is [-100 300] Hz.

Observation 1: It is feasible for the TM10 test for the multiple-CSI-process UE when the cell within the COMP cooperating set is blanking  and the same transmission power for TP2 and TP1 (PTP1PTP2=1)  is set.

Observation 2: Higher MCS is much easier to differentiate 2-cell CRS-IC, 1-cell CRS-IC and 0-cell CRS-IC

Observation 3: Higher interference level is preferable for the aggressor cell outside the cooperating set, i.e, 8.45 dB is set the for the interference level of the aggressor cell outside the cooperating set.

Proposal  7: MCS=16 is selected as the PDSCH MCS and the same transmission power is configured for TP1 and TP2, INR2 is 8.45 dB for TP3.

# Appendix A: Link level simulation for one CSI process UE

Figure 3: MCS=9 and INR = [10.45 8.45]

Figure 4: MCS=14 and INR = [10.45 8.45]

Figure 5: MCS=16 and INR = [10.45 8.45]

Figure 6: MCS=18 and INR = [10.45 8.45]

Figure 7: MCS=9 and INR = [10.45 6.45]

Figure 8: MCS=14 and INR = [10.45 6.45]

Figure 9: MCS=16 and INR = [10.45 6.45]

Figure 10: MCS=9 and INR = [10.45 6.45]

# Appendix B: Link level simulation for multiple CSI process

Figure 11: MCS=9 and INR2 = 8.45 dB

Figure 12: MCS=14 and INR2 = 8.45 dB

Figure 13: MCS=16 and INR2 = 8.45 dB

Figure 14: MCS=18 and INR2 = 8.45 dB

Figure 15: MCS=9 and INR2 = 6.45 dB

Figure 16: MCS=14 and INR2 = 6.45 dB

Figure 17: MCS=16 and INR2 = 6.45 dB

Figure 18: MCS=18 and INR2 = 6.45 dB

# Reference

- R4-152400, WF on CRS-IM PDSCH demodulation (TM10), Ericsson, Apr. 2015.

- R4-152082, Initial considerations on TM10 test case design, Samsung, Apr. 2015.

- R4-155191, WF on CRS-IM demodulation v10 (clean), Ericsson, ZTE, August 2015