3GPP TSG-RAN WG4 Meeting #76bis 	R4-156774

Sophia Antipolis, France 12 – 16 October 2015

Source:	DISH

Title:	TP for B65 UE RFSENS

Work item	LTE_1980_2170_REG1

Agenda item:	8.1.2

Document for:	Approval

Abstract

This document explores how RFSENS is determined and proposes a value for B65. (pCR)

1		RFSENS background

In [1] we presented the methodology explain how RFSENS was calculated for B1 based on the formula shown below.

REFSENS [dBm] = 	kTB + NF + SNR + IM – 3

Where

- kTB is the thermal noise introduced above in units of dBm in the specified bandwidth B

- NF is the maximum noise figure of the receiver at the antenna port which is assumed to be 9dB.

- SNR is signal to interference plus noise requirements for the chosen modulation and coding scheme which is assumed to be  –1dB for QPSK rate 1/3

- IM is the implementation margin allowed which is assumed to be 2.5dB

- -3 dB represent the Rx diversity gain

These results are aligned with minimum specification defined in TS36.101 Table 7.3.1-1 for B1 copied below

3GPP TS36.101 Table 7.3.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |

In [1] we note that NF of 9 dB which was originally specified in release 99 is quite generous since a state of art of receiver has a much lower NF in the region of 5-6dB (~ 2.0-2.5dB duplexer filter  (UMTS/LTE Band1) insertion loss and a NF for RFIC of about 3dB or less). ]

1.1	B1 insertion loss

Revisiting the data for different B1 filters specifications (worst case) are presented below for a number of different filter vendors.

| E-UTRA BAND | Reference | Insertion loss |  | Isolation |  | Filter vendor |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Tx | Rx | Tx | Rx |  |
|  |  | [dB] | [dB] | [dB] | [dB] |  |
| B1 | Filter 1 | 2.0 | 2.2 | 53 | 45 | A |
|  | Filter 2 | 2.3 | 2.4 | 55 | 55 | A |
|  | Filter 3 | 2.4 | 2.5 | 50 | 55 | A |
|  | Filter 4 | 1.8 | 2.2 | 55 | 44 | B |
|  | Filter 5 | 2.2 | 2.2 | 54 | 55 | B |
|  | Filter 6 | 1.9 | 2.2 | 52 | 43 | C |
|  | Filter 7 | 1.6 | 2.0 | 51 | 48 | D |
|  | Filter 8 | 1.5 | 2.0 | 53 | 43 | D |
|  | Filter 9 | 1.6 | 2.0 | 53 | 43 | D |
|  | Filter 10 | 2.0 | 2.3 | 53 | 50 | D |
|  | Filter 11 | 1.8 | 2.2 | 44 | 44 | D |

We note the average is 2.2dB for all the Rx duplex filter insertion loss, however if we only consider filters which can provide a Tx/Rx isolation values of >50dB (shown highlighted) than the average insertion loss from three different filter vendor for a B1 filter is much higher at 2.4 dB

1.2	B65 insertion loss

B65 is an extension of B1 and therefore we need to consider the delta insertion loss for this band compared with Band 1. In this section we detail the B65 filter simulation results presented in [2]

Table 1.2-1: B65 filter data

| E-UTRA BAND | Reference | Rx Insertion loss | Tx/Rx Isolation |  |
| --- | --- | --- | --- | --- |
|  |  | Rx | Tx | Rx |
|  |  | [dB] | [dB] | [dB] |
| B65 | Filter 12 | 3.9 | 52 | 43 |
|  | Filter 13 | 2.8 | 55 | 52 |
|  | Filter 14 | 2.0 | 55 | 55 |

Worst case filter data 12, 13 and 14 is provided by three different filter vendors.  Taking into account the need to account from multiple filter vendors sourcing, we propose that the RFSENS requirements are based on filter 13 and 14 to support the need for multi-sourcing filters and need to provide a Tx/Rx isolation values >50dB (shown highlighted).  In this case the delta increase in the Rx filter loss would -0.4dB to +0.4dB compared with the average value for B1 of 2.4 dB

1.3	B65 RFSENS

Therefore using the same methodology to determine RFSENS as B1 (and NOT taking in account a more realistic value for the RFIC NF contribution) the RFSENS for B65 would be -0.4dB to +0.4dB relative to B1 depending on implementation either filter 13 or filter 14.  In this case we have two options for defining the RFSENS for B65

Option 1:  B65 RFSENS requirements could be based on the average filter loss as per normal R4 methodology i.e. 0dB degradation with respect to B1

Option 2:  B65 RFSENS requirements could be based on a small relaxation with respect to B1

At the last R4 meeting in Beijing two companies supported a relaxation and four companies supported no relaxation with respect to B1 [2]. Therefore we propose the RFSENS for B64 should be in the range of [0 to 0.4dB] with respect to the B1 RFSENS value and discussions should continue to define a single value

2		Summary

We propose the RFSENS requirements as shown below for B65;

| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |  |
| 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 65 |  |  | - 99.5 | - [96.5 | - 94.7 | - 93.5 | FDD |

A UE that complies with the E-UTRA Band 65 minimum requirements must also comply with the E-UTRA Band 1 minimum requirements. Thus, for UEs supporting inter-band CA including B1 the reference sensitivity would be increased by the amount given in ΔRIB,c in TS36.101 Table 7.3.1-1A and Table 7.3.1-1B when B1 is signalled

3	Reference

[1] 	R4-154819: TP for UE RFSENS; Dish

[2]	R4-155411: B65 ad-hoc meeting notes

### ---------------- Start of TP -----------------------

### 8.1.2	UE REFSENS

The lower duplexer is assumed to be a Band 1 duplexer. This will ensure Band 1 performance within 1920-1980/2110-2170 MHz, including UE REFSENS

The upper duplexer is 2x90MHz. In this design, the duplexer has been for optimized in the non-overlapping frequencies 2170-2200 MHz as shown in Figure 1. The plots are at room temperature and also include a reference to the performance expected including temperature variation (manufacturing process variation is not included). It can be seen that the IL for the 2x90 MHz filter is not worse than for Band 1 for frequencies above 2160 MHz Thus, UE REFSENS for carriers including allocation above 2170MHz can be as Band 1 assuming such filter. Additional relaxations are needed in case of supporting intra-band CA.

Figure 8.1.2-1: B1 (blue), B1+MSS (navy) and B1+MSS RX with optimized performance at the highest frequencies (green)

8.1.2.1		RFSENS background

In [	R4-154819] we presented the methodology explaining how RFSENS was calculated for B1 based on the formula shown below.

REFSENS [dBm] = 	kTB + NF + SNR + IM – 3

Where

- kTB is the thermal noise introduced above in units of dBm in the specified bandwidth B

- NF is the maximum noise figure of the receiver at the antenna port which is assumed to be 9dB.

- SNR is signal to interference plus noise requirements for the chosen modulation and coding scheme which is assumed to be  –1dB for QPSK rate 1/3

- IM is the implementation margin allowed which is assumed to be 2.5dB

- -3 dB represent the Rx diversity gain

These results are aligned with minimum specification defined in TS36.101 Table 7.3.1-1 for B1 copied below

3GPP TS36.101 Table 7.3.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |

In [R4-154819] we note that NF of 9 dB which was originally specified in release 99 is quite generous since a state of art of receiver has a much lower NF in the region of 5-6dB, taking in account; 2.0-2.5dB duplexer filter contribution (UMTS/LTE Band1) insertion loss and a NF for RFIC of about 3dB or less. In this case the total implementation margin would now be 5.5 to 6dB for a B1 non-CA device.

B1 insertion loss

Revisiting the data for different B1 filters specifications (worst case) are presented below for a number of different filter vendors.

| E-UTRA BAND | Reference | Insertion loss |  | Isolation |  | Filter vendor |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Tx | Rx | Tx | Rx |  |
|  |  | [dB] | [dB] | [dB] | [dB] |  |
| B1 | Filter 1 | 2.0 | 2.2 | 53 | 45 | A |
|  | Filter 2 | 2.3 | 2.4 | 55 | 55 | A |
|  | Filter 3 | 2.4 | 2.5 | 50 | 55 | A |
|  | Filter 4 | 1.8 | 2.2 | 55 | 44 | B |
|  | Filter 5 | 2.2 | 2.2 | 54 | 55 | B |
|  | Filter 6 | 1.9 | 2.2 | 52 | 43 | C |
|  | Filter 7 | 1.6 | 2.0 | 51 | 48 | D |
|  | Filter 8 | 1.5 | 2.0 | 53 | 43 | D |
|  | Filter 9 | 1.6 | 2.0 | 53 | 43 | D |
|  | Filter 10 | 2.0 | 2.3 | 53 | 50 | D |
|  | Filter 11 | 1.8 | 2.2 | 44 | 44 | D |

We note the average is 2.2dB for all the Rx duplex filter insertion loss, however if we only consider filters which can provide a Tx/Rx isolation values >50dB than the average insertion loss from three different filter vendor for a B1 filter is much higher at 2.4 dB

B65 insertion loss

B65 is an extension of B1 and therefore we need to review the insertion loss for this band compared with Band 1. In this section we detail the B65 filter simulation results presented in [2]

B65 filter data

| E-UTRA BAND | Reference | Rx Insertion loss | Tx/Rx Isolation |  |
| --- | --- | --- | --- | --- |
|  |  | Rx | Tx | Rx |
|  |  | [dB] | [dB] | [dB] |
| B65 | Filter 12 | 3.9 | 52 | 43 |
|  | Filter 13 | 2.8 | 55 | 52 |
|  | Filter 14 | 2.0 | 55 | 55 |

Worst case filter data 12, 13 and 14 is provided by three different filter vendors.  Taking into account the need to account from multiple filter vendors sourcing, we propose that the RFSENS requirements are based on filter 13 and 14 to support the need for multi-sourcing filters and need to provide a Tx/Rx isolation values >50dB.  In this case the delta increase in Rx filter losses would -0.4dB to +0.4dB compared with the average value for B1 (2.4 dB).

B65 RFSENS

Therefore using the same methodology to determine RFSENS as B1 (and NOT taking in account a more realistic value for the RFIC NF contribution) the RFSENS for B65 would be -0.4dB to +0.4dB relative to B1 depending on implementation either filter 13 or filter 14.  In this case we have two options for defining the RFSENS for B65

Option 1:  B65 RFSENS requirements could be based on the average filter loss as per normal R4 methodology i.e. 0dB degradation with respect to B1

Option 2:  B65 RFSENS requirements could be based on a small relaxation with respect to B1

Summary

We propose the RFSENS requirements as shown below for B65;

| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |  |
| 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 65 |  |  | - 99.5 | - 96.5 | -94.7 | - 93.5 | FDD |

A UE that complies with the E-UTRA Band 65 minimum requirements must also comply with the E-UTRA Band 1 minimum requirements. Thus, for UEs supporting inter-band CA including B1 the reference sensitivity would be increased by the amount given in ΔRIB,c in TS36.101 Table 7.3.1-1A and Table 7.3.1-1B when B1 is signalled.

---------------- End of TP -----------------------