3GPP TSG-RAN WG4 Meeting #77	R4-157000

Anaheim, CA, United States, 16th November – 20th November, 2015

Agenda Item:	9.5.1

Source: 	Intel Corporation

Title: 	Discussion on SFN RRH deployment and UE behavior for high speed train scenario

Document for:	Discussion

- Introduction

The study item “Study on performance enhancements for high speed scenario in LTE” was approved at RAN plenary meeting #66 [1]. One objective of this SI is to identify new conditions that affect the system performance, including BS demodulation, UE demodulation, and UE RRM performance for the new high speed train (HST) scenarios [2]. A common high speed train scenario is single frequency network (SFN) deployment with remote radio heads (RRHs) along high speed train railways [2, 3], and a specified bi-directional HST SFN channel model was agreed as in [3]. Based on this HST channel model, there have been various proposals and discussions on the factors causing performance degradation [4, 5] and on the potential enhancement solutions [5, 6]. The solutions can be made either in the base station side or in the UE side, or both sides. In general, among the various solutions and approaches, it is surely desirable if the solution achieve robust performance without much design changes in both in BSs and UEs. In this contribution, we propose a HST system that may less impact on the legacy system design and behaviours.

- Possible SFN RRH deployment and UE behaviour

In [5], one option to improve UE demodulation performance is to develop a HST enhanced UE (HeUE), which is capable of tracking multiple (for instance, two opposite) Doppler shifts and conducting corresponding channel estimation; another option is to perform pre-compensation to compensate Doppler frequency shift at the base station (BS) side in downlink transmission. The Doppler pre-compensation scheme at BS side was also introduced in [6], and it also claimed that, when large opposite Doppler shifts co-exist in the received signal, the DMRS based interpolation scheme is less resilient than CRS based interpolation and frequency offset tracking scheme. Therefore, the DMRS based transmission mode should be de-prioritized.

In this contribution, we propose a possible SFN RRH deployment to combat with the problem of large opposite Doppler shifts co-existence. Unlike [6], we propose to take advantage of the DMRS based transmission mode to handle the opposite Doppler shifts co-existence effect for interpolation and/or channel estimation. The outline of proposed RRH deployment scenarios is as below:

- SFN RRHs use DMRS-based PDSCH transmission modes (i.e. TM9 or TM10);

- All RRHs in a SFN group transmit same PDSCH payload simultaneously (i.e. non-coherent  joint transmission for data);

- In the RRH deployment along the train rail, the odd-numbered RRHs may use one antenna port (e.g. AP7) and even-numbered RRHs use another antenna port (e.g. AP8);

- The odd- and even-numbered RRHs transmit reference signals using separate different DMRS AP in SFN.

- From the receiver perspective, the UE is able to execute channel parameter estimations per a DMRS AP. PDSCH received signal appears like joint transmission received signal combined between different MIMO layers. Compensation can be applied per a layer.

- A HST receiver can select a layer with a dominant SNR or signal quality. Alternatively, the receiver can combine the multiple layers’ signals for performance improvement.

Specifically, we propose a possible framework by a combination of the SFN data signal (e.g. PDSCH) transmission using orthogonal non-SFN reference signal transmission from different RRHs. With orthogonal reference signal transmitted by different RRHs, UE is able to estimate the propagation channel from each RRH using the reference signal and use this information to improve the demodulation of the SFN data signal. Fig. 1 illustrates the highlights of the proposed SFN RRH deployment.

Fig.1 Conceptual diagram of transmission from RRHs along rail to high speed train

Proposal 1:  We propose a possible framework by a combination of the SFN data signal using orthogonal non-SFN reference signal transmission from different RRHs.

- In the RRH deployment along the train rail, RRH can utilize antenna ports that have orthogonality between the APs. For example, the odd-numbered RRHs may use one antenna port (e.g. AP7) and even-numbered RRHs use another antenna port (e.g. AP8).

- DMRS APs of TM9 or TM10 are feasible to be used for providing orthogonal reference signals between adjacent RRHs.

- A HST UE is able to execute channel parameter estimations per a DMRS AP. PDSCH received signal appears like joint transmission received signal combined between different MIMO layers.

UE receives a superposition of signals from different RRHs. The receiver can demodulate the RS from each AP following the legacy RS demodulation procedure. Only channel parameters for signals from different RRHs can be separately estimated using the demodulated DMRS per each AP. Basically, a new UE behaviours impacts are found in channel parameter estimation. If the UE is aware that it is under a HST communication situation, the UE applies different CE interpolation filter per a layer corresponding to a DMRS AP as Figure 2.

Fig.2 HST UE behaviours for Doppler shift/spread estimation and compensation using DMRS

In the proposed system, the UE can estimate separate channel link per RRHs without significant design changes. In the BS side, we don’t foresee much significant design changes from conventional TM9 or TM10 deployment.

Regarding the detection behaviour, we can write two received signal setups :

Option 1 :

Option 2 :

where Y are received signals in frequency domain from RRHs, H1 and H2 are MIMO channels respectively from RRH1 and RRH2, P is a precoder in each RRH, and   is colored noise due to inter-carrier interference (ICI), and n is AWGN noise. From the received signal model of Option-1, the MMSE-IRC baseline receiver can be used effectively. For the moment when the two RRH signals have equivalent signal quality, i.e. when the train runs in the middle of two RRHs, Option-2 model can be considered. We propose to study the UE behaviors based on Option-1 and Option-2 RX signals under the proposed RRH deployment. An attractive benefit of the proposed system is no need to change much from the legacy behaviors. We propose to study further on performance and system behaviors in the WI.

Proposal-2: An expected benefit from the proposed system is to reuse the legacy UE behaviors without significant design changes.  Based on the network deployment in Proposal-1, we propose to study HST UE behavior and performance in the WI

- Channel parameter estimation behavior per RRH using DMRS AP

- MMSE-IRC baseline receiver performance of suppressing ICI or other RRH interferences.

- Conclusions

In this contribution, we propose a possible SFN RRH deployment and corresponding UE behaviour for HST scenario.

Proposal 1:  We propose a possible framework by a combination of the SFN data signal using orthogonal non-SFN reference signal transmission from different RRHs.

- In the RRH deployment along the train rail, RRH can utilize antenna ports that have orthogonality between the APs. For example, the odd-numbered RRHs may use one antenna port (e.g. AP7) and even-numbered RRHs use another antenna port (e.g. AP8).

- DMRS APs of TM9 or TM10 are feasible to be used for providing orthogonal reference signals between adjacent RRHs.

- A HST UE is able to execute channel parameter estimations per a DMRS AP. PDSCH received signal appears like joint transmission received signal combined between different MIMO layers.

Proposal-2: An expected benefit from the proposed system is to reuse the legacy UE behaviors without significant design changes.  Based on the network deployment in Proposal-1, we propose to study HST UE behavior and performance in the WI

- Channel parameter estimation behavior per RRH using DMRS AP

- MMSE-IRC baseline receiver performance of suppressing ICI or other RRH interferences.

- Reference

[1] RP-142307, “Study on performance enhancements for high speed scenario in LTE”, NTT DOCOMO INC., Huawei, HiSilicon.

[2] R4-150122, “New channel mode for SFN deployment”, Huawei, HiSilicon, Feb. 2015

[3] R4-151587, “Channel model for SFN deployment with RRH for HST scenario”, Qualcomm, April 2015.

[4] R4-155618, “Discussion on UE performances in High Speed Train Scenario”, Intel, Oct. 2015.

[5] R4-154243, “UE performance evaluation and enhancement under the new scenarios”, Huawei, HiSilicon, August 2015.

[6] R4-154630, “Feasibility of DMRS based transmission mode for high speed train in SFN deployment”, MediaTek, August 2015.