3GPP TSG-RAN WG4 Meeting #77	R4-157201

Anaheim, USA, 16 – 20 Nov 2015

Agenda Item:	7.7.5

Source: 	Alcatel-Lucent

Title: 	Initial simulation results for eMTC cell identification under enhanced coverage

Document for:	Discussion

# Introduction

This contribution presents simulation assumptions for cell identification procedure of Rel-13 MTC UE under enhanced coverage were agreed in RAN4 #76bis [1]. In this paper, we provide the initial simulation results based on the agreement.

# Simulation Assumptions

For convenience, the simulation assumptions for cell identification procedure of Rel-13 MTC UE under enhanced coverage agreed in RAN4 #76bis are copied in the tables [1].

Table 1: Cell Identification Link Simulation Parameters

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 | Channel 1 |
| Data and Control PSD relative to RS PSD | dB | 0 | 0 | 0 |
| PSS and SSS PSD relative to RS PSD | dB | 0 | 0 | 0 |
| Carrier frequency | GHz | 2 | 2 | 2 |
| Number of RB’s |  | 6 | 6 | 6 |
| RB Utilization | % | 100 | 100 | 100 |
| Data Modulation | - | QPSK | QPSK | QPSK |
| Frame Structure Type | - | 1(FDD) | 1(FDD) | 1(FDD) |
| CP Length | - | Normal | Normal | Normal |
| DRX | - | Off | Off | Off |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 | 0 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | 0 | CP/2 |
| Relative Delay of 1st Path (asynchronous) | μs | 0 | 1.5 | 3.0 |
| Es/Noc | dB | -4.82 | -9.71 | Test 1: -6.45 Test 2: -8.45 Test 3: -10.45 Test 4: -13.45 Test 5: -16.45 |
| Number of Tx antennas | - | 1 | 1 | 1 |
| PSS Sequence ID | - | See Table 3, 4 | See Table 3, 4 | See Table 3, 4 |
| SSS Sequence ID | - | See Table 3, 4 | See Table 3, 4 | See Table 3, 4 |
| Propagation Condition | - | AWGN, EPA5, ETU30, EPA1, ETU1 |  |  |
| Noc Model | - | AWGN |  |  |
| Sampling rate | ms | 40ms |  |  |
| NOTE : Noc value doesn’t include the three simulated eNB signals’ power |  |  |  |  |

Table 2: Other simulation assumption parameters for cell identification

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 and Cell 2 by the UE | Yes |
| Cell 1, 2, 3 carrier frequency | Same |
| False detect threshold | Required as in a real UE implementation |
| UE having apriori knowledge of system being synchronous or synchronous (by signaling) | No |
| Duty cycle | 100% (to represent non-DRX case) |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence id. |
| CP length detection | Both short or long CP may be present, hence UE needs to detect CP length |
| Receive antennas | 1 |

Table 3: Cell Id Combinations to be simulated

| case # | Cell 3 (Desired Cell) |  | Cell 1 (Interferer 1) |  | Cell 2 (Interferer 2) |  | Scenario |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | psc3 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 2 | psc1 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 3 | psc1 | ssc1a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 4 | psc3 | ssc1a, ssc1b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 5 | psc3 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |
| 6 | psc1 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |
| 7 | psc1 | ssc1a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |
| 8 | psc3 | ssc1a, ssc1b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |

Table 4: PSC, SSC indices for simulations

| Label | Code index |
| --- | --- |
| psc1 | 29 |
| psc2 | 25 |
| psc3 | 34 |

| Label | Code index | Cell group index |
| --- | --- | --- |
| (ssc1a, ssc1b) | (6, 8) | 36 |
| (ssc2a, ssc2b) | (10, 12) | 40 |
| (ssc3a, ssc3b) | (7, 9) | 37 |
| (ssc1a, ssc3b) | (6, 9) | 65 |

# Simulation Results

Cell search and identification are based on the PSS and the SSS. Normally, the detection of PSS is based on non-coherent detection, while SSS can be based on non-coherent detection or coherent detection by using PSS as a phase reference for SSSS detection.

For coverage enhancement simulation in this paper, only non-coherent detection is used in this simulation for both PSS and SSS with 40ms sampling rate.

The simulation results show the time for the UE to identify the Cell3 based on PSS/SSS detection. As the agreement, the performance criterion is the 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence id. For this simulation, the detection threshold was chosen conservatively with no false detection.

Figure 1: Cell Detection time for AWGN channel (Synchronous)

Figure 2: Cell Detection time for EPA5 channel  (Synchronous)

Figure 3: Cell Detection time for ETU30 channel  (Synchronous)

Figure 4: Cell Detection time for EPA1 channel (Synchronous)

Figure 5: Cell Detection time for ETU1 channel  (Synchronous)

# Summary

In this paper we provided the initial simulation results on small cell identification based on the agreed simulation assumptions from synchronized scenario. We suggest these results be taken into consideration when defining the requirements.

# References

- R4-156656, Simulation assumptions for cell identification for Rel-13 MTC under enhanced coverage, Nokia Networks, Ericsson